3GPP TSG-RAN WG4 Meeting #110                                          R4-2400956

Athens, Greece, February 26 – March 1, 2024

Source: 	Huawei, HiSilicon

Title: 	Remaining issues for Rel-18 MPR reduction

Agenda Item:	8.19.1.2

Document for:	Approval

Introduction

With the approved feature CR [1] and LS [2] on capability/signalling design, the main work on Rel-18 MPR reduction was declared as accomplished in the last meeting. However, some leftover issues on particular specification and UE capability are identified after review. In this contribution, we would like to share our views and check whether clarifications are needed.

Discussion

Clarification on specification

A-MPR

It was agreed to further check A-MPR part in last meeting. By reviewing the feature CR [1], we understand the the following part was introduced for Rel-18 power boosting with the intention that the UE would still be allowed to meet all related requirements with maximum output power reduction and/or additional maximum output power reduction considering transmitted power boosted by ΔPPowerBoost.

But the highlighted sentence may cause ambiguity since no new A-MPR was defined throughout the whole discussion. Consequently we propose the following modification to erase the ambiguity.

Proposal 1: Adopt the CR in R4-2400957 as a way to clearly define the applicable power reduction considering power boosting.

Clarification on UE capability

As captured in the LS [2], two UE capabilities were introduced to distinguish between two sets of requirements (i.e. the one with EVM flatness relaxation and the other one w/o relaxation), while the network can configure power boosting for different modulation order via two separate RRC IEs.

From UE perspective, it could be possible to support both two aforementioned schemes for power boosting. But with current signalling design, it is unclear about which scheme that such UE should utilize.

Observation: For Rel-18 MPR reduction, two RRC IEs were introduced to configure power boosting for different modulation order, but corresponding UE capabilities were defined for two different power boosting schemes.

One straightforward solution could be adding RRC signalling to explicitly indicate which scheme that the UE should apply. Another solution, which could be less complicated, is to add note for the existing UE capabilities in order to explicitly rule out simultaneous indication and keep RRC IEs intact. Thus we have the following proposal.

Proposal 2: Introduce the following note for Rel-18 MPR reduction UE capabilities [powerBoostRel18] and [powerBoostTSRel18]:

- Note: UE cannot indicate the support of [powerBoostRel18] and [powerBoostTSRel18] simultaneously.

Conclusion

In this contribution we discussed on the maintenance aspects on Rel-18 further coverage enhancement by PAR&MPR reduction, we have the following observation and proposals:

Observation: For Rel-18 MPR reduction, two RRC IEs were introduced to configure power boosting for different modulation order, but corresponding UE capabilities were defined for two different power boosting schemes.

Proposal 1: Adopt the CR in R4-2400957 as a way to clearly define the applicable power reduction considering power boosting.

Proposal 2: Introduce the following note for Rel-18 MPR reduction UE capabilities [powerBoostRel18] and [powerBoostTSRel18]:

- Note: UE cannot indicate the support of [powerBoostRel18] and [powerBoostTSRel18] simultaneously.

- References

- R4-2321826, “CR to 38.101 for introduction of MPR reduction”, Nokia, Nokia Shanghai Bell, Qualcomm, Huawei, Ericsson, Apple, Vivo, OPPO, MediaTek, 3GPP TSG-RAN4 #109, Chicago, USA, November 13 – 17, 2023.

- R4-2321960, “LS on UE capabilities for MPR reduction”, Nokia, 3GPP TSG-RAN4 #109, Chicago, USA, November 13 – 17, 2023.