3GPP TSG-RAN WG4 Meeting # 110	R4-2400998

# Athens, GR, 26 Feb – 01 Mar, 2024

Agenda Item:	8.5.1.1

Source: 	OPPO

Title: 	Discussion on case 1 requirements for Pre-MG and conMG

Document for:	Approval

1	Introduction

RAN4 #109 meeting discussed the RRM core requirements for case 1 (pre-configured MG and concurrent MG), and the majority issues have been discussed and captured in [1]. This contribution will give our view on UE capability for dynamic collisions.

2	Discussion

Issue 2-1-5: [Case 1] Whether to define a new UE capability for dynamic collisions? - Way Forward - Option 1: - Add a UE capability to indicate whether the UE supports Case 1 gap combinations that cause dynamic collisions. - Option 2: - No additional capability is needed to handle the dynamic collision.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [32-1] | Concurrent gap with Pre-MG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Detail in Clause [9.1.x.2] of TS 38.133. | 19-3-x and 19-2 x = 1 or 2 | Yes | No | Network should not configure concurrent gap with Pre-MG | Per UE | No | No | N.A |  | Optional with capability signalling |
| [32-2] | 2 Pre-MG configuration with simultaneous activation/deactivation | Support configurations of 2 Pre-MG with simultaneous activation/deactivation in the same FR. | 32-1 | Yes | No | Unknown activation time for simultaneous Pre-MG is expected | Per UE | No | No | N.A |  | Optional with capability signalling |
| [32-3] | Dynamic collision | Support the RRM requirements when the activation/deactivation delay of Pre-MG overlaps the other measurement gap with lower priority or Pre-MG | 32-1 | Yes | No | UE is not expected to meet the requirements | Per UE | No | No | N.A |  | Optional with capability signalling |

As shown above, whether to introduce a new UE capability for dynamic collision is not settled. In our view, dynamic collision is not only about the static gap configuration such as the number of Pre-MGs, but also related to when and how to trigger (de)activation for the Pre-MG. It will happen even if there is only one Pre-MG in the concurrent gaps. From this point of view, we slightly prefer option 2, which means UE should have the capability to handle dynamic collision by default if supporting [pre-MG + concurrent gaps] capability.

Proposal 1: No additional capability is needed to handle the dynamic collision.

3	Conclusion

This contribution discussed the requirements for case 1: Pre-MG and conMG, and gave the following proposals.

Proposal 1: No additional capability is needed to handle the dynamic collision.

4	Reference

- R4-2321604, WF for NR_MGE-2_part1, MediaTek Inc