3GPP TSG-RAN WG4 Meeting #110	R4-2401114

Athens, GR, 26 Feb – 01 Mar, 2024

Source:	Samsung

Title:	Simulation results on Rel-18 MIMO UE demodulation performance and CSI requirements

Agenda item:	8.21.4.1

Document for:	Discussion

- Introduction

In the RAN plenary RAN#98 meeting, the WID [1] for Rel-18 NR MIMO evolution for downlink and uplink has been approved. RAN4 work plan for this topic including RF, RRM and performance part was endorsed in [2]. In RAN4#108bis meeting, initial discussion on UE demodulation performance and CSI reporting parts started and overall work scope endorsed [3]. In last RAN4#109 meeting, there are some agreements on general scope, initial simulation assumptions for the feasibility study of Type II Doppler, Type II CJT codebooks and Rel-18 DMRS [4]. In this contribution, we provide our initial simulation results and observations on Type II Doppler, Type II CJT codebooks and Rel-18 DMRS separately.

- TypeII-Doppler-r18

For the TypeII-Doppler-r18 codebook introduced in Rel-18, which is UE-side prediction, UE predicts the channel at furture time slots based on past CSI-RS measurements and reports the future PMIs calculated from the predicted channel. Simulation cases in this section are based on Figure 2-1 in which CSI request periodicity is 8ms.

Figure 2-1 CSI configuration for TypeII-Doppler-r18 simulation

  - Simulation assumptions

For the initial simulation assumptions of feasibility study, we get some agreements in RAN4#109 meeting, these agreements are summarized into below Table 2-1.

Table 2-1. Simulation assumptions of TypeII-Doppler-r18

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-30 TDLA30-50 TDLA30-100 |
| Antenna configuration |  |  | XP Medium 16 x 2 XP Medium 16 x 4 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
|  | number of NZP CSI-RS resources (K) |  | 4 |
|  | separation between two consecutive CSI-RS resources |  | 2 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Not configured |
| Sub-band Size |  | RB | 4 |
| csi-ReportingBand |  |  | 1111111111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 4 |
| CSI request |  |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration for Rel-16 cases | Codebook Type |  | typeII-r16 |
|  | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  | 0010 |
| Codebook configuration for Rel-18 cases | Codebook Type |  | typeII-doppler-r18 |
|  | paramCombination-Doppler-r18 |  | 7 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMISubbandsPerCQISubband-Doppler-r18) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-Doppler-RI-Restriction-r18) |  | 0010 |
|  | N4 |  | 1 or 4 |
|  | DD unit duration | slot | 2 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| MCS |  |  | MCS13 MCS20 |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.2.1.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in AnnexB.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

In last RAN4#109 meeting, we get the agreement to define PMI reporting requirements with ‘typeII-Doppler-r18’ if both option 1 and option 2 could be fulfilled, in which option 1 means UE throughput with ‘typeII-Doppler-r18’ codebook could outperform Rel-16 Type II codebook with the same CSI-RS configurations and medium/high UE speed, option 2 means UE throughput with ‘typeII-Doppler-r18’ codebook could outperform random precoding based on Single Panel Type I codebook with the same CSI-RS configurations and medium/high UE speed. Therefore, in this document, we focus on comparing the throughput performance of Rel-16 Type II codebook, ‘typeII-Doppler-r18’ codebook and random precoding based on Single Panel Type I codebook.

  - Simulation results

PMI reporting performance comparison with 2Rx between cases with Rel-16 Type II codebook and ‘typeII-Doppler-r18’ codebook are as Figure 2-2 – Figure 2-5.

Figure 2-2. 2Rx performance comparison Rel-16 Type II vs ‘typeII-Doppler-r18’ with Doppler 200Hz

Figure 2-3. 2Rx performance comparison Rel-16 Type II vs ‘typeII-Doppler-r18’ with Doppler 100Hz

Figure 2-4. 2Rx performance comparison Rel-16 Type II vs ‘typeII-Doppler-r18’ with Doppler 50Hz

Figure 2-5. 2Rx performance comparison Rel-16 Type II vs ‘typeII-Doppler-r18’ with Doppler 30Hz

From the performance comparsion results on Figure 2-2 – Figure 2-4, compared with Rel-16 Type II codebook, there are no gain of ‘typeII-Doppler-r18’ codebook cases with Doppler 200Hz, 100Hz and 50Hz, no matter with MCS13 or MCS20 configurations. From the performance comparsion results on Figure 2-5, there are gains of ‘typeII-Doppler-r18’ codebook cases with Doppler 30Hz for both MCS13 and MCS20.

Observation 1: For the 2Rx PMI reporting performance simulation results on ‘typeII-Doppler-r18’ codebook vs Rel-16 Type II codebook, no gain is observed for TDLA30 cases with Doppler 200Hz, 100Hz, 50Hz, neither with MCS13 nor MCS20.

Observation 2: For the 2Rx PMI reporting performance simulation results on ‘typeII-Doppler-r18’ codebook vs Rel-16 Type II codebook, some gain could be observed for TDLA30 cases with Doppler 30Hz,  both with MCS13 and MCS20.

For Doppler 30Hz case, we run more simulation cases to check the performance of ‘typeII-Doppler-r18’ codebook with N4 equals 1 vs N4 equals 4, even the performance comparison with random precoding based on Single Panel Type I codebook. The simulation results are on Figure 2-6 and Figure 2-7. From these results, we know that both cases using ‘typeII-Doppler-r18’ codebook with N4=1 and N4=4 could outperform the case using Rel-16 Type II codebook, and the performance of ‘typeII-Doppler-r18’ codebook cases using N4=4 is better than N4=1.

Observation 3: For Doppler 30Hz case, both cases using ‘typeII-Doppler-r18’ codebook with N4=1 and N4=4 could outperform cases using Rel-16 Type II codebook, and the performance of ‘typeII-Doppler-r18’ codebook cases using N4=4 is better than N4=1.

Figure 2-6. 2Rx performance comparison Rel-16 Type II vs ‘typeII-Doppler-r18’ with Doppler 30Hz

Figure 2-7. 4Rx performance comparison Rel-16 Type II vs ‘typeII-Doppler-r18’ with Doppler 30Hz

In order to display the performance gain more clearly, we summarize the performance gain on ‘typeII-Doppler-r18’ codebook vs Rel-16 Type II codebook in below Table 2-2 and Table 2-3. In these tables, Test metric defined as γ1=ttypeII-dopplertrnd as a starting point, where ttypeII-doppler is X % (e.g. X=90, 70, 60) of the maximum throughput obtained at SNRtypeII-doppler using the typeII-Doppler-r18 precoder configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-doppler with random precoding based on Type I Single Panel codebook.

Table 2-2. X % SNR point for TDLA30-30 cases

| case | X | SNRtypeII-r16 | SNRtypeII-doppler-r18 N4=1 | SNRtypeII-doppler-r18 N4=4 |
| --- | --- | --- | --- | --- |
| 2Rx MCS13 | 60 | 6.0 | 5.8 | 5.5 |
|  | 70 | 7.4 | 7.2 | 7.0 |
|  | 90 | 10.2 | 9.9 | 9.8 |
| 2Rx MCS20 | 60 | 12.6 | 12.3 | 12.1 |
|  | 70 | 14.2 | 14.0 | 13.8 |
|  | 90 | 17.4 | 17.1 | 17.1 |
| 4Rx MCS13 | 60 | 1.5 | 1.5 | 1.2 |
|  | 70 | 2.6 | 2.6 | 2.3 |
|  | 90 | 4.1 | 4.0 | 3.9 |

Table 2-3. Test metric γ1 for TDLA30-30 cases

| case | X | Test metric γ1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Rel-16 Type II | Rel-18 Doppler N4=1 | Rel-18 Doppler N4=4 |
| 2Rx MCS13 | 60 | 1.72 | 1.79 | 1.91 |
|  | 70 | 1.71 | 1.75 | 1.79 |
|  | 90 | 1.68 | 1.73 | 1.74 |
| 2Rx MCS20 | 60 | 1.48 | 1.52 | 1.54 |
|  | 70 | 1.53 | 1.55 | 1.57 |
|  | 90 | 1.57 | 1.62 | 1.62 |
| 4Rx MCS13 | 60 | 2.0 | 2.0 | 2.14 |
|  | 70 | 1.92 | 1.93 | 2.0 |
|  | 90 | 1.88 | 1.91 | 1.95 |

Observation 4: For Doppler 30Hz case, the performance gain at the 60% of the maximum throughput point is higher than 70% and 90% of the maximum throughput point.

Observation 5: For Doppler 30Hz case, considering the 60% SNR working point without impairment, MCS13 is more suitable than MCS20.

Observation 6: For Doppler 30Hz with MCS13 case, the 60% SNR working point of Rel-18 Doppler codebook could get about 0.5dB gain over Rel-16 codebook for 2Rx cases, and about 0.3dB gain for 4Rx cases.

Observation 7: For Doppler 30Hz with MCS13 case, the test metric γ1 on 60% SNR working point of Rel-18 Doppler codebook is about 1.91 for 2Rx case, and 2.14 for 4Rx case.

- TypeII-CJT-r18

For CJT schemes which is introduced in Rel-18, all the PDSCH/DMRS ports are jointly transmitted from multiple TRPs and signals from multiple TRPs are combined coherently, hence multiple geographically separated TRPs or RRHs for CJT are assumed to be well synchronized in time and frequency as well as the phase and amplitude of their antenna arrays are calibrated. In last RAN4#109 meeting [4], we agreed to study the feasibility of introducing PMI reporting requirements with ‘typeII-CJT-r18’ (FR1 FDD only) based on below test metric.

Issue 1-1-4: clarify if PMI reporting requirements are needed for ‘typeII-CJT-r18’ codebook Agreement: - Focus on co-located scenario (zero time offset and zero frequency offset), introduce PMI reporting requirements with ‘typeII-CJT-r18’ (FR1 FDD only) if performance gain could be observed, with Test metric defined as γ3=ttypeII-CJTtrnd, where ttypeII-CJT is Z % (e.g., Z=90) of the maximum throughput obtained at SNRtypeII-CJT using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-CJT with random precoding based on type I Single Panel codebook.

  - Simulation assumptions

According to the agreements of initial simulation assumptions for Type II CJT codebook in sub-topic 2-2 [4], the simulation assumptions could be summarized as below Table 3-1.

Table 3-1. Simulation assumptions of TypeII-CJT-r18

| Parameter |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) |  | TRxP #2(Note 1) |
| Duplex mode |  |  |  |  | FDD |  |  |
| Bandwidth |  |  |  | MHz | 10 |  |  |
| Subcarrier spacing |  |  |  | kHz | 15 |  |  |
| Active DL BWP index |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  | TDLA30-10 |  |  |
| Antenna configuration per TRxP |  |  |  |  | High XP 8 x 2 (N1,N2) = (4,1) High XP 8 x 4 (N1,N2) = (4,1) |  |  |
| Beamforming Model |  |  |  |  | As specified in Annex B.4.1 (Note 4) |  |  |
| CSI-RS resources K |  |  |  |  | 2 |  |  |
| Number of TRPs NTRP |  |  |  |  | 2 |  |  |
| restrictedCMR-Selection |  |  |  |  | ON |  |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |  |
|  | k0 |  |  |  | 0 |  |  |
|  | Starting symbol (S) |  |  |  | 2 |  |  |
|  | Length (L) |  |  |  | 12 |  |  |
|  | PRB bundling type |  |  |  | Static |  |  |
|  | PRB bundling size |  |  |  | 2 |  |  |
|  | Resource allocation type |  |  |  | Type 1 |  |  |
|  | RBG size |  |  |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | 1000 |  | 1002 |
|  | TCI state |  |  |  | TCI State #1 |  | TCI State #2 |
|  | DMRS Type |  |  |  | Type 1 |  |  |
|  | Number of additional DMRS |  |  |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  | QCL Type |  | Type A |  | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  | QCL Type |  | N/A |  | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A |  | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  | QCL Type |  | N/A |  | N/A |
| Resource allocation |  |  |  |  | Full-overlapping |  |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  | us | 0 |  |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  | Hz | 0 |  |  |
| Number of HARQ Processes |  |  |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | 2 |  |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  | Periodic |  |  |
|  |  | Number of CSI-RS ports (X) |  |  | 4 |  |  |
|  |  | CDM Type |  |  | FD-CDM2 |  |  |
|  |  | Density (ρ) |  |  | 1 |  |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) |  |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | (9) |  |  |
|  |  | CSI-RS periodicity and offset |  | slot | 5/1 |  |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  | Resource #9 | Resource #10 |  |
|  |  | CSI-RS resource Type |  |  | Aperiodic | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  |  | 8 | 8 |  |
|  |  | CDM Type |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |  |
|  |  | Density (ρ) |  |  | 1 | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 8, (4,6) | Row 8, (4,6) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | (5) | (9) |  |
|  |  | CSI-RS periodicity and offset |  | slot | Not configured | Not configured |  |
|  |  | aperiodicTriggeringOffset |  |  | 0 | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  |  | Aperiodic |  |  |
|  |  | CSI-IM RE pattern |  |  | Pattern 0 |  |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4,9) |  |  |
|  |  | CSI-IM timeConfig periodicity and offset |  | slot | Not configured |  |  |
| ReportConfigType |  |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  | Not configured |  |  |
| Sub-band Size |  |  |  | RB | 4 |  |  |
| csi-ReportingBand |  |  |  |  | 1111111111111 |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  |  | 5 |  |  |
| CSI request |  |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | CodebookType |  |  | typeII-CJT-r18 |  |  |
|  |  | CodebookMode |  |  | Mode2 |  |  |
|  |  | paramCombination-CJT-L-r18 |  |  | 7 {4, 4} 4 {2, 2} |  |  |
|  |  | paramCombination-CJT-r18 |  |  | 4 (pν=14and18, β=12) 7 (pν=12and12, β=12) for paramCombination-CJT-L-r18 = 7 1 (pν=18, β=14) for paramCombination-CJT-L-r18 = 4 |  |  |
|  |  | R(numberOfPMI-SubbandsPerCQI-Subband-CJT-r18) |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (4,1) |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,1) |  |  |
|  |  | CodebookSubsetRestriction |  |  | 0x 7FF FFFF FFFF FFFF FFFF |  |  |
|  |  | RI Restriction |  |  | 00000001 (1 MIMO layer per TRxP) |  |  |
| Physical channel for CSI report |  |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 8 |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  |  | R.PDSCH.1-6.4 |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with subband granularity |  |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 4: Randomization of the principle beam direction per TRxP shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |  |  |  |

  - Simulation results

paramCombination-CJT-L-r18 = 4 and paramCombination-CJT-r18 = 1

For paramCombination-CJT-L-r18 = 4 {2, 2} and paramCombination-CJT-r18 = 1 (pν=18, β=14), the absolute TP of following TypeII-CJT-r18 codebook vs random single panel Type I codebook are as Figure 3-1, and the relative TP of following TypeII-CJT-r18 codebook vs random Type I codebook are as Figure 3-2.

Figure 3-1. Absolute TP of following TypeII-CJT-r18 codebook vs random Type I codebook

Figure 3-2. Relative TP of following TypeII-CJT-r18 codebook vs random Type I codebook

Accordingly, the SNRtypeII-CJT  at Z% of the maximum TP by using TypeII-CJT-r18 and related test metric  γ3=ttypeII-CJTtrnd are summarized in below Table 3-2, where ttypeII-CJT is Z% (e.g., Z=90) of the maximum throughput obtained at SNRtypeII-CJT using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-CJT with random precoding based on type I Single Panel codebook.

Table 3-2 SNRtypeII-CJT and test metric  γ3 of paramCombination-CJT-L-r18 = 4 and paramCombination-CJT-r18 = 1

| case | Z | SNRtypeII-CJT | γ3 |
| --- | --- | --- | --- |
| 2 Rx | 60 | 2.0 | 2.99 |
|  | 70 | 3.3 | 2.55 |
|  | 90 | 7.9 | 1.47 |
| 4 Rx | 60 | 0.7 | 3.49 |
|  | 70 | 1.6 | 2.96 |
|  | 90 | 5.6 | 1.67 |

paramCombination-CJT-L-r18 = 7 and paramCombination-CJT-r18 = 4

For paramCombination-CJT-L-r18 = 7 {4, 4} and paramCombination-CJT-r18 =4 (pν=14and18, β=12), the absolute TP of following TypeII-CJT-r18 codebook vs random Type I codebook are as Figure 3-3, and the relative TP of following TypeII-CJT-r18 codebook vs random Type I codebook are as Figure 3-4.

Figure 3-3. Absolute TP of following TypeII-CJT-r18 codebook vs random Type I codebook

Figure 3-4. Relative TP of following TypeII-CJT-r18 codebook vs random Type I codebook

Accordingly, the SNRtypeII-CJT  at Z% of the maximum TP by using TypeII-CJT-r18 and related test metric  γ3=ttypeII-CJTtrnd are summarized in below Table 3-3, where ttypeII-CJT is Z% (e.g., Z=90) of the maximum throughput obtained at SNRtypeII-CJT using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-CJT with random precoding based on type I Single Panel codebook.

Table 3-3 SNRtypeII-CJT and test metric  γ3 of paramCombination-CJT-L-r18 = 7 and paramCombination-CJT-r18 = 4

| case | Z | SNRtypeII-CJT | γ3 |
| --- | --- | --- | --- |
| 2 Rx | 60 | 2.5 | 2.67 |
|  | 70 | 3.5 | 2.44 |
|  | 90 | 5.9 | 1.93 |
| 4 Rx | 60 | 1.1 | 3.03 |
|  | 70 | 1.8 | 2.76 |
|  | 90 | 3.9 | 2.21 |

paramCombination-CJT-L-r18 = 7 and paramCombination-CJT-r18 = 7

For paramCombination-CJT-L-r18 = 7 {4, 4} and paramCombination-CJT-r18 =7 (pν=12and12, β=12), the absolute TP of following TypeII-CJT-r18 codebook vs random Type I codebook are as Figure 3-5, and the relative TP of following TypeII-CJT-r18 codebook vs random Type I codebook are as Figure 3-6.

Figure 3-5. Absolute TP of following TypeII-CJT-r18 codebook vs random Type I codebook

Figure 3-6. Relative TP of following TypeII-CJT-r18 codebook vs random Type I codebook

Accordingly, the SNRtypeII-CJT  at Z% of the maximum TP by using TypeII-CJT-r18 and related test metric  γ3=ttypeII-CJTtrnd are summarized in below Table 3-3, where ttypeII-CJT is Z% (e.g., Z=90) of the maximum throughput obtained at SNRtypeII-CJT using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-CJT with random precoding based on type I Single Panel codebook.

Table 3-3 SNRtypeII-CJT and test metric  γ3 of paramCombination-CJT-L-r18 = 7 and paramCombination-CJT-r18 = 7

| case | Z | SNRtypeII-CJT | γ3 |
| --- | --- | --- | --- |
| 2 Rx | 60 | 2.4 | 2.73 |
|  | 70 | 3.4 | 2.49 |
|  | 90 | 5.9 | 1.94 |
| 4 Rx | 60 | 1.0 | 3.09 |
|  | 70 | 1.8 | 2.78 |
|  | 90 | 3.9 | 2.22 |

Observation 8: For the test metric at 90% of the maximum TP, the test metric value increase along with the number of beams (indicated by paramCombination-CJT-L-r18) increases.

Observation 9: For the test metric at 90% of the maximum TP, the test metric value increase along with the paramCombination-CJT-r18 increases, but the increase values are limited.

Observation 10: When paramCombination-CJT-L-r18 = 4, the test metric at 90% of the maximum TP is about 1.4 for 2 Rx, is about 1.6 for 4Rx.

Observation 11: When paramCombination-CJT-L-r18 = 7, the test metric at 90% of the maximum TP is about 1.9 for 2 Rx, is about 2.2 for 4Rx.

- Rel-18 DMRS

In RAN4#108bis meeting, it is agreed to introduce PDSCH demodulation requirements for Rel-18 enhanced DMRS for SU-MIMO scenario [3], and in RAN4#109 meeting, some agreements about simulation assumptions are achieved [4]. In this simulation, we’d like to know the PDSCH demodulation performance difference between cases by using Rel-15 DMRS and Rel-18 enhanced DMRS.

  - Simulation assumptions

Table 4-1. Simulation assumptions of R18 enhanced DMRS

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | FDD: 12 TDD: Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping inter-leaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
|  | Antenna ports indexes |  | {1008} for Rank 1 tests {1008-1009} for Rank 2 tests {1008-1010} for Rank 3 tests {1008-1011} for Rank 4 tests |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| Number of HARQ Processes |  |  | FDD: 4 for 2Rx FDD: 8 for 4Rx TDD: 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | FDD: 2 TDD: Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

  - Simulation results

For FDD and TDD with 2Rx and 4Rx, the PDSCH demodulation performance results are as Figure 4-1 – Figure 4-3.

Figure 4-1. FDD and TDD PDSCH demodulation performance with 2Rx Rank 2

Figure 4-2. FDD PDSCH demodulation performance with 4Rx Rank 2 and Rank 4

Figure 4-3. TDD PDSCH demodulation performance with 4Rx Rank 2 and Rank 4

The SNR point without impairment at 70% fraction of maximum throughput could be summarized as below Table 4-2 and Table 4-3.

Table 4-2 SNR point at 70% fraction of maximum throughput for FDD cases

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | SNR(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | R15 DMRS | R18 DMRS |
| 1-1 | R.PDSCH.1-3.1 FDD | 10 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x2, ULA Low | 15.4 | 15.4 |
| 1-2 | R.PDSCH.1-2.4 FDD | 10 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x4, ULA Low | 9.4 | 9.4 |
| 1-3 | R.PDSCH.1-2.4 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 11.6 | 11.6 |

Table 4-3: SNR point at 70% fraction of maximum throughput for TDD cases

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | SNR(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | R15 DMRS | R18 DMRS |
| 1-1 | R.PDSCH.2-3.1 TDD | 40 / 30 | 64QAM, 0.50 | FR1.30-1 | TDLA30-10 | 2x2, ULA Low | 15.2 | 15.2 |
| 1-2 | R.PDSCH.2-3.1 TDD | 40 / 30 | 64QAM, 0.50 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 9.4 | 9.4 |
| 1-3 | R.PDSCH.2-2.4 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 11.9 | 11.9 |

Obviously, there is no performance difference between Rel-15 DMRS cases and Rel-18 enhanced DMRS cases.

Observation 12: For the PDSCH demodulation performance, cases with Rel-18 enhanced DMRS almost have the same performance with Rel-15 DMRS cases.

- Conclusion

In this contribution, we provide our simulation results and observations on UE demodulation performance and CSI requirements for Rel-18 NR MIMO enhancements for downlink.

The observations could be summarized as:

Observation 1: For the 2Rx PMI reporting performance simulation results on ‘typeII-Doppler-r18’ codebook vs Rel-16 Type II codebook, no gain is observed for TDLA30 cases with Doppler 200Hz, 100Hz, 50Hz, neither with MCS13 nor MCS20.

Observation 2: For the 2Rx PMI reporting performance simulation results on ‘typeII-Doppler-r18’ codebook vs Rel-16 Type II codebook, some gain could be observed for TDLA30 cases with Doppler 30Hz,  both with MCS13 and MCS20.

Observation 3: For Doppler 30Hz case, both cases using ‘typeII-Doppler-r18’ codebook with N4=1 and N4=4 could outperform cases using Rel-16 Type II codebook, and the performance of ‘typeII-Doppler-r18’ codebook cases using N4=4 is better than N4=1.

Observation 4: For Doppler 30Hz case, the performance gain at the 60% of the maximum throughput point is higher than 70% and 90% of the maximum throughput point.

Observation 5: For Doppler 30Hz case, considering the 60% SNR working point without impairment, MCS13 is more suitable than MCS20.

Observation 6: For Doppler 30Hz with MCS13 case, the 60% SNR working point of Rel-18 Doppler codebook could get about 0.5dB gain over Rel-16 codebook for 2Rx cases, and about 0.3dB gain for 4Rx cases.

Observation 7: For Doppler 30Hz with MCS13 case, the test metric γ1 on 60% SNR working point of Rel-18 Doppler codebook is about 1.91 for 2Rx case, and 2.14 for 4Rx case.

Observation 8: For the test metric at 90% of the maximum TP, the test metric value increase along with the number of beams (indicated by paramCombination-CJT-L-r18) increases.

Observation 9: For the test metric at 90% of the maximum TP, the test metric value increase along with the paramCombination-CJT-r18 increases, but the increase values are limited.

Observation 10: When paramCombination-CJT-L-r18 = 4, the test metric at 90% of the maximum TP is about 1.4for 2 Rx, is about 1.6 for 4Rx.

Observation 11: When paramCombination-CJT-L-r18 = 7, the test metric at 90% of the maximum TP is about 1.9 for 2 Rx, is about 2.2 for 4Rx.

Observation 12: For the PDSCH demodulation performance, cases with Rel-18 enhanced DMRS almost have the same performance with Rel-15 DMRS cases.

- Reference

[1] RP-223276, WID Update: MIMO Evolution for Downlink and Uplink, Samsung

[2] R4-2301929, Work plan for Rel-18 WI on MIMO evolution, Samsung

[3] R4-2316920, WF on NR_MIMO_evo_DL_UL_demod, Samsung

[4] R4-2321141, WF on [109][327] NR_MIMO_evo_DL_UL_demod, Samsung