3GPP TSG-RAN WG4 Meeting # 110	R4-2401172

# Athens, Greece, Feb. 26 – Mar. 1, 2024

Source: 	Xiaomi

Title: 	Discussion on testability and interoperability issues for CSI

Agenda Item:	11.1.4

Document for:	Discussion

# 1 Introduction

Since whether CSI prediction and CSI compression will be specified in WI is still FFS. In this contribution, we will not discuss test goal, test metric and generation related issue. Here, we only provide views on test feasibility of two side mode for AI/ML.

- Discussion

      - 2.1 Two side mode test feasibility

In last meeting, the main issue related to CSI compression test is whether and how to define reference decoder. There are 4 options are two-side mode:

- Option 1: DUT provides the decoder - Option 2: Infra vendor provides the decoder - Option 3: Full decoder specification in standard - Option 4: TE vendor provides the decoder

In the next table, we fill in our views regarding to the 4 options.

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | DUT vendor | Decoder vendor (infra vendor in case of testing UEs) | RAN4 specifications | TE vendor, decoder developed based on RAN4 specifications |
| Source of decoder training data | Up to DUT vendor (no need to be specified) | Coordination with encoder vendor is required | Not needed, decoder fully specified (used as part of the RAN4 procedure to specify the decoder) | Alignment of training data between UE and TE may be required. |
| DUT vendor knowledge of the test decoder | Full knowledge | No or partial or enough or full knowledge based on alignment with infra vendors or specifications | Full knowledge based on the specifications | Partial knowledge – based on the RAN4 specification |
| Supported training collaboration type (source of training data should be consistent with the collaboration type) | FFS | FFS | FFS | FFS |
| Test decoder verification procedure at TE and/or DUT | Need to ensure that decoder performance is not degraded (as intended by the decoder provider) on the TE | - Need to ensure that decoder performance is not degraded (as intended by the decoder provider) on the TE - Need to ensure that decoder performance is good enough to enable a DUT that meets the minimum requirements to pass the test | Not needed as long as the standardized model implementation can be similar enough between TE vendors | Not needed as long as the model implementation can be similar enough between TE vendors |
| Feasibility of test decoder verification procedure | Yes. | FFS. If the performance of decoder will be verified, what’s the reference encoder? If different encoders from different UE vendor are provided, how to verify the decoder? The decoder verification seems to be encoder specific. | Yes. | FFS. Similar as option 2. |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment (knowledge of model, training type, etc.) | No, there may be mismatch between decoder from UE and NW vendor | Yes. | No, there may be mismatch between decoder from specification and NW vendor | Depends on what’s partially specified for the decoder. There may be mismatch between decoder from specification and TE |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) | Higher than Option 3/4 in terms of that maybe more than one decoder are implemented by TE. Lower than Option 3/4 in terms of that no training at TE is required | Higher than Option 3/4 in terms of that maybe more than one decoder are implemented by TE. Lower than Option 3/4 in terms of that no training at TE is required | Lower complexity than Option 1/2 in terms of that only one decoder is implemented by TE Lower than Option 4 in terms of that no training at TE is required | Lower complexity than Option 1/2 in terms of that only one decoder is implemented by TE Higher than Option 3 in terms of that training at TE is required Note: How to ensure compatibility/interoperability between TE and DUT needs further study. |
| Specification Effort (e.g. test decoder) | Low | Low | Highest - RAN4 needs to standardize the entire decoder | High RAN4 needs study and decide on what to standardize |
| Confidentiality/IP issues | Need to be considered | Need to be considered | No. | No. |
| Applicability to different scenarios/conditions/ configurations | Depends on how to design the test to guarantee the generalization | Depends on how to design the test to guarantee the generalization | Depends on how to design the test to guarantee the generalization | Depends on how to design the test to guarantee the generalization |
| Complexity of actual testing procedure for the ecosystem | Testing the encoder at DUT Higher than Option 3/4 Need for interaction between TE vendor | Testing the encoder at DUT Higher than Option 3/4 Testing complexity higher also than option 1. | Testing the encoder at DUT Low – no need for interaction between TE vendors and other parties | Testing the encoder at DUT Low – no need for interaction between TE vendors and other parties |
| Complexity of verifying/testing the test decoder | Higher than option 3/4 FFS compared to option 2 | Higher than Option 3/4 FFS compared to Option 1 | Low | Low |
| Complexity of deploying for the ecosystem |  |  |  |  |
| Friendly to STOA(state of the art) model test / Forward compatibility when new AI models are invented | Yes. | No. depends on the implementation of decoder in NW vendor. Offline alignment may still be needed. | Depends. - If new AI decoder is invented which requires simpler encoder. With simpler new encoder, UE may not pass the test with old decoder. - If new AI encoder is invented which requires more simple decoder. With more advanced encoder, UE can pass the test with old decoder. | No. depends on the implementation of decoder in TE vendor. Offline alignment may still be needed. |
| Relationship with reference decoder/encoder for defining requirement | If reference decoder is defined, a reference encoder maybe needed either. Otherwise, how to verify the performance of reference decoder when there are many encoders is FFS. | If reference decoder is defined, a reference encoder maybe needed either. Otherwise, how to verify the performance of reference decoder when there are many encoders is FFS. | Reference encoder is not needed. | Depends on what’s specified for the decoder. |
| Whether model transfer/delivery is needed during the test procedure | Not specific to two side model. | Not specific to two side model. | Not specific to two side model. | Not specific to two side model. |

- Conclusion

In this contribution, we provide our views regarding to test feasibility of two side model.

- Reference

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface