3GPP TSG-RAN WG4 Meeting #110	R4-2401216

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2078 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for Rel-18 38.101-1 is to modify the requirements for eRedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh-Core |  |  |  |  |  | Date: |  |  | 2024-01-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CR for Rel-18 38.101-1 is to modify the requirements for eRedCap |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not, specified requirements cause some confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of changes >

5.3I	Channel bandwidth for RedCap

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 with maximum channel bandwidth of 20 MHz. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths up to 20 MHz. When UE supports IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18 the requirements in this specification apply with maximum transmission bandwidth of 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths.

< Omit unchanged parts >

7.3I.3	Reference sensitivity power level for eRedCap

For UE supporting IE supportOfERedCap-r18 and IE eRedCapNotReducedBB-BW-r18, the REFSENS requirements for RedCap UE in clause 7.3I.2 apply.

For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18, the reference sensitivity requirements for 5 MHz channel bandwidth with 15 kHz SCS defined in clause 7.3I.2 apply. These reference sensitivity requirements for 5 MHz channel bandwidth apply also for 10, 15 and 20 MHz channel bandwidths with 15 kHz SCS. In case the reference sensitivity requirements for 5 MHz channel bandwidth are not defined, the reference sensitivity requirements for 10 MHz channel bandwidth apply with the reference sensitivity level reduced by 3.2 dB, and the UL configuration set to minimum between RB number as specified in UL configuration Table 7.3.2-3 and Table 7.3I.2-3 and 25 RBs. These reference sensitivity requirements for 10 MHz channel bandwidth apply also for 15 and 20 MHz channel bandwidths. Both Tx RBs in UL configuration and Rx RBs in FRC, when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 25 RBs for 15 kHz SCS.

For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18, for 30 kHz SCS, the reference sensitivity requirements defined for 10 MHz channel bandwidth in clause 7.3I.2 apply with reference sensitivity level reduced by 3.0 dB and the UL configuration set to minimum between RB number specified in UL configuration Table 7.3.2-3 and Table 7.3I.2-3 and 12 RBs. These reference sensitivity requirements for 10 MHz channel bandwidth apply also for 15 and 20 MHz channel bandwidths with 30 kHz SCS. Both Tx RBs in UL configuration and Rx RBs in FRC, when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 12 RBs for 30 kHz SCS.

NOTE:	It is not necessary to repeat verification when same requirement applies for multiple channel bandwidths.

< Omit unchanged parts >

A.3.2.2	FRC for receiver requirements for QPSK

Table A.3.2.2-1 Fixed reference channel for receiver requirements (SCS 15 kHz, FDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 3 | 5, 10, 15, 20 (Note 5) | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 984 | 1672 | 3368 | 5120 | 6912 | 8712 | 10504 | 14088 | 17424 |
| Transport block CRC | Bits | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 3240 | 5400 | 11232 | 17064 | 22896 | 28728 | 34560 | 46656 | 58320 |
| Max. Throughput averaged over 1 frame | Mbps | 0.787 | 1.338 | 2.694 | 4.096 | 5.530 | 6.970 | 8.403 | 11.270 | 13.9392 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: Channel bandwidths 10, 15, and 20 MHz in this column only apply to UEs supporting IE supportOfERedCap-r18 but not eRedCapNotReducedBB-BW-r18. |  |  |  |  |  |  |  |  |  |  |

Table A.3.2.2-2 Fixed reference channel for receiver requirements (SCS 30 kHz, FDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 10, 15, 20 (Note 5) | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 24 | 12 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 217 | 245 | 273 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | Bits | 1608 | 808 | 2472 | 3368 | 4224 | 4992 | 6912 | 8712 | 10504 | 14088 | 15880 | 17928 |
| Transport block CRC | Bits | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | Bits | 5184 | 2592 | 8208 | 11016 | 14040 | 16848 | 22896 | 28728 | 34992 | 46872 | 52920 | 58968 |
| Max. Throughput averaged over 1 frame | Mbps | 2.734 | 1.374 | 4.202 | 5.726 | 7.181 | 8.486 | 11.750 | 14.810 | 17.857 | 23.950 | 26.996 | 30.478 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: Channel bandwidths 10, 15, and 20 MHz in this column only apply to UEs supporting IE supportOfERedCap-r18 but not eRedCapNotReducedBB-BW-r18. |  |  |  |  |  |  |  |  |  |  |  |  |  |

< Omit unchanged parts >

A.3.2.4	FRC for maximum input level for 256 QAM

Table A.3.2.4-1 Fixed reference channel for maximum input level receiver requirements (SCS 15 kHz, FDD, 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 3, 5, 10,15, 20 (NOTE 5) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination |  | 256QAM |  |  |  |  |  |  |  |  |
| Modulation |  | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 9992 | 16896 | 34816 | 53288 | 71688 | 90176 | 108552 | 143400 | 180376 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slot 0,1 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 2 | 3 | 5 | 7 | 9 | 11 | 13 | 18 | 22 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 12960 | 21600 | 44928 | 68256 | 91584 | 114912 | 138240 | 186624 | 233280 |
| Max. Throughput averaged over 1 frame | Mbps | 7.994 | 13.517 | 27.853 | 42.630 | 57.350 | 72.141 | 86.842 | 114.720 | 144.301 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: Channel bandwidths 5, 10, 15, and 20 MHz in this column only apply to UEs supporting IE supportOfERedCap-r18. |  |  |  |  |  |  |  |  |  |  |

< Omit unchanged parts >

A.3.3.2	FRC for receiver requirements for QPSK

Table A.3.3.2-1 Fixed reference channel for receiver requirements (SCS 15 kHz, TDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5, 10, 15, 20 (Note 5) | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | Bits | 1672 | 3368 | 5120 | 6912 | 8712 | 10504 | 14088 | 17424 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | CBs | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | Bits | 5400 | 11232 | 17064 | 22896 | 28728 | 34560 | 46656 | 58320 |
| Max. Throughput averaged over 1 frame | Mbps | 0.669 | 1.347 | 2.048 | 2.765 | 3.485 | 4.202 | 5.635 | 6.970 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: Channel bandwidths 10, 15, and 20 MHz in this column only apply to UEs supporting IE supportOfERedCap-r18 but not eRedCapNotReducedBB-BW-r18. |  |  |  |  |  |  |  |  |  |

< End of changes >