3GPP TSG-RAN WG4 Meeting # 110                                   R4-2401241

# Athens, Greece, February 26 – March 01, 2024

Agenda item:	8.26.3

Source:	China Telecom

Title:	Discussion on RRM performance requirements for NES

Document for:	Discussion

- Introduction

In RAN4 #109 meeting, RRM performance requirements for NES was discussed and the WF [1] was approved. In this paper, we present our views about RRM performance requirements for NR network energy saving.

- Discussion

Performance part related to SSB-less

Status in the WF [1]:

Issue 3-1-1: Test case for FR1 inter-band SSB-less activation/deactivation Agreement: - Define test case for FR1 inter-band SSB-less activation. - FFS detailed test configurations. Issue 1-2-1: SCell activation requirements Online session (Tuesday Nov 14, 2023) Agreement: - Define both TRS and A-TRS based SCell activation requirements - For TRS based Scell activation, Tactivation_time = Tfirst_TRS + TTRS + [3 or 5] ms; - TTRS is periodicity of the TRS. - For A-TRS based Scell activation, Tactivation_time = Tfirst_ATRS+Tgap + TATRS +[3 or 5] ms; - The applicable conditions are discussed in sub-topic 1-1..

For performance part related to SSB-less, FR1 inter-band SSB-less activation is mainly discussed in core part, TRS/A-TRS is needed for SCell activation when the side conditions are fulfilled. Therefore, test cases for FR1 inter-band SSB-less activation based on TRS/A-TRS in non-DRX need to be defined.

Proposal 1: Test cases for FR1 inter-band SSB-less activation based on TRS/A-TRS in non-DRX need to be defined.

Since both TRS and A-TRS based SCell activation requirements are defined in core part, separate test cases for TRS based and A-TRS based FR1 inter-band SSB-less activation need to be defined.

Proposal 2: Since both TRS and A-TRS based SCell activation requirements are defined in core part, separate test cases for TRS based and A-TRS based FR1 inter-band SSB-less activation need to be defined.

- Conclusion

The contribution presents our views on RRM performance requirements for NES, with the following proposals:

Proposal 1: Test cases for FR1 inter-band SSB-less activation based on TRS/A-TRS in non-DRX need to be defined.

Proposal 2: Since both TRS and A-TRS based SCell activation requirements are defined in core part, separate test cases for TRS based and A-TRS based FR1 inter-band SSB-less activation need to be defined.

- References

- R4-2321562, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon, RAN WG4 Meeting #109, November 2023.