# 3GPP TSG-RAN WG4 Meeting # 110                                                             R4-2401253

# Athens, Greece, Feb. 26th – Mar. 1st , 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2080 | rev |  | Current version: | 16.18.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Core) Correct the equation in the NOTE for harmonic mixing MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, NR_CADC_R16_2BDL_xBUL-Core, NR_6GHz_unlic_full |  |  |  |  |  |  | Date: |  | 2024-01-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Similar with UL/DL frequency condition of harmonic MSD, for harmonic mixing MSD test point, except the nth aggressor UL frequency hit the mth victim victim DL frequency, the valid aggressor UL carrier frequency must be restricted in the range of BWchannel/2 higher than the lower edge and lower than the upper edge of aggressor UL band, respectively. Out of this range, it is invalid MSD test point. However, only NOTE4 includes the above condition, but NOTE2/5 don’t. Thus it should be keep consistence. Moreover, the term in the equation in NOTE 4 is incorrect, it should be fULHB, rather than fULLB. Similar problem is found in Table 7.3F.5.2-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add the condition/equation of FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 to restrict the aggressor UL carrier frequency in the range of BWchannel/2 higher than the lower edge and lower than the upper edge of aggressor UL band, respectively. 2. Reword some texts to void duplication. 3. Correct the incorrect term in some Notes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - There may exist invalid test points for harmonic mixing MSD requirements, also incorrect descriptions for the NOTEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.4, 7.3F.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-1 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

# << Start of change >>

7.3A.4	Reference sensitivity exceptions due to UL harmonic interference for CA

<unchanged texts are omitted.>

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same CA configuration. Reference sensitivity exceptions are specified in Table 7.3A.4-4 with uplink configuration specified in Table 7.3A.4-4a. Sensitivity degradation is not required for receiver even order harmonic mixing with aggressor 3rd order and above harmonic interference.

Table 7.3A.4-4: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz(dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz(dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n25 | n713,4 | 26.5 | 23.3 | 20.9 | 15.3 |  |  |  |  |  |  |  |  |  |
| n40 | n284 | 37.8 | 34.8 | 33 | 30.3 |  |  |  |  |  |  |  |  |  |
| n77 | n2 | 6.7 | 5.0 | 4.0 | 3.7 |  |  |  |  |  |  |  |  |  |
| n77 | n5 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |  |  |
| n78 | n285 | 31 | 28 | 26.2 | 25 |  |  |  |  |  |  |  |  |  |
| n78 | n402 | 10.4 | 10.4 | 10.4 | 10.4 |  |  | 7.2 | 6.2 | 5.5 |  | 4.5 |  |  |
| n78 | n412 |  | 10.4 | 10.4 | 10.4 |  |  | 8.2 | 7.6 | 7.3 |  | 6.6 | 6.4 | 6.3 |
| n79 | n85 | 25 | 21.8 | 19.4 | 13.8 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 with fULHB the UL carrier frequency and BWCℎannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 w fULHB the UL carrier frequency and BWCℎannelHB the channel bandwidth configured in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n25 | n71 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n40 | n28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n77 | n2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n77 | n5 | 25 | 25 | 20 | 20 |  |  |  |  |  |  |  |  |  |  |
| n78 | n28 | 15 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| n78 | n40 | 30 | 50 | 50 | 50 | 50 |  |  | 50 | 50 | 50 |  | 50 |  |  |
| n78 | n41 | 30 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n79 | n8 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# << Next change >>

7.3F.5.2	Reference sensitivity exceptions due to receiver harmonic mixing

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same CA configuration. Reference sensitivity exceptions are specified in Table 7.3F.5.2-1 with uplink configuration specified in Table 7.3F.5.2-2

Table 7.3F.5.2-1: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz(dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz(dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n46 | n481 | 22.6 | 19.5 | 17.8 | 16.6 |  |  | 14 | 13.1 | 12.6 | 12 | 12 | 12 | 12 |
| NOTE 1: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such thatfULHB=15∗fDLLB0.1 in MHz and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# << End change >>