3GPP TSG-WG4 Meeting #110 	R4-2401437

# Athens, Greece, February 26 - March 1

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on multi-Rx TCI state switching requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRx_DL-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Proposals to update and correct some of the TCI state switching requirements for multi-Rx. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Known TCI state conditions: - Separating the conditions between s-DCI and m-DCI and correcting some of the terminology. MAC-CE based TCI state switch: - Adding a reference to legacy requirements for s-DCI for improved readability of the spec. DCI-based TCI state switch: - Splitting the section to s-DCI and m-DCI sub-sections. - Clarifying that after completing both switches in m-DCI, the UE is expected to be able to receive with target TCI states simultaneously. - For s-DCI, removing the condition of UE being configured only with GBBR and not with non-GBBR from the dual to single TCI state switch without delay. Active TCI state list update: - Splitting the section to s-DCI and m-DCI sub-sections. - Removing simultaneous PDCCH reception from the requirement. Additionally, some general correction of typos and language. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification not according to the latest agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Change 1 >

## 8.10E	Active TCI state switching delay for UE operating in FR2-1 and configured with groupBasedBeamReporting-r17

### 8.10E.1	Introduction

The requirements in this clause apply for a UE configured with groupBasedBeamReporting-r17 and dual TCI state configurations on PCell in standalone NR. The requirements apply when the dual target TCI states are known and the UE can receive the dual target TCI states simultaneously, where the known conditions are defined in 8.10E.2. UE shall complete the switch of active TCI state within the delay defined in this clause.

### 8.10E.2	Known conditions for TCI state

In s-DCI, when the UE receives a TCI state switching command indicating a TCI state switch to dual target TCI states, the dual TCI states are known if the following conditions are met:

-	Dual TCI states are QCL-ed with typeD to reference signals in a resource group in the latest Rel-17 group based beam report

-	The dual TCI states and all the RSs in the two QCL chains remain detectable during the TCI state switching period

-	SNR of the TCI state ≥ -3dB

-	RS resource group configured for dual TCI states is reported in the last [1280]ms

In m-DCI, when the UE receives a TCI state switching command indicating a TCI state switch to a target TCI state in one CORESETPoolIndex, the target TCI state is known if the following conditions are met:

-	If the UE receives a TCI state switching command also for the other CORESETPoolIndex, the target TCI states in the two TCI state switching commands are QCL-ed with typeD to reference signals in a resource group in the the latest Rel-17 group based beam report, or

-	If the TCI state of the other CORESETPoolIndex is not switched, the target TCI state and the TCI state currently indicated for the other CORESETPoolIndex are QCL-ed with typeD to reference signals in a resource group in the the latest Rel-17 group based beam report

-	The dual TCI states and all the RSs in the two QCL chains remain detectable during the TCI state switching period

-	SNR of the TCI state ≥ -3dB

-	RS resource group configured for dual TCI states is reported in the last [1280]ms

Editor’s note: FFS whether additional conditions are needed for tests.

### 8.10E.3	MAC-CE based dual DL TCI state switch delay

#### 8.10E.3.1	MAC-CE based dual DL TCI state switching delay for sDCI

For a UE capable of singleDCI-SDM-scheme-r16 and configured with sDCI, the requirements for MAC-CE based TCI state switch in clause 8.10.3 apply.

The requirement in this clause applies when UE is provided with twoQCLTypeDforPDCCHRepetition.

When MAC CE commands indicating dual TCI state switch for two CORESETs with different CORSET index p and q (where the TCI states of CORESET p and q are QCLed with different QCL type-D RS), are received in slot n, and if the TCI states are known, UE shall be able to receive on the target TCI states at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*( max (Tfirst-SSBp, Tfirst-SSBq)  + TSSB-proc) / NR slot length. The UE shall be able to receive both PDCCH with the old TCI states until slot n+ THARQ + 3Nslotsubframe,µ ,

Where:

-	THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	Tfirst-SSBp is time to first SSB transmission associated with TCI state of CORESET p after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state.

-	Tfirst-SSBq is time to first SSB transmission associated with TCI state of CORESET q after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	TSSB-proc = 2 ms.

-	TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise.

#### 8.10E.3.2	MAC-CE based dual DL TCI state switching delay for mDCI

The requirement in this clause applies when UE is capable of multiDCI-MultiTRP-r16 and configured with two CORESETPoolIndexes.

When a MAC CE command indicating a TCI state switch for CORESETPoolIndex p is received at slot n, and if the TCI state is known, UE shall be able to receive on the target TCI state at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*( Tfirst-SSBp, + TSSB-proc) / NR slot length. The UE shall be able to receive PDCCH with the old TCI states until slot n+ THARQ + 3Nslotsubframe,µ.

Where:

-	THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	Tfirst-SSBp is time to first SSB transmission (i.e., SSB associated to CORESETPoolIndex p) after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state.

-	TSSB-proc = 2 ms.

-	TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise.

When UE receives TCI state switch command for CORESETPoolIndex p while UE performing TCI state switch of CORESETPoolIndex q, UE is expected to receive on new TCI states of CORESETPoolIndex p and q after completing both the TCI state switch of CORESETPoolIndex p and q.

### 8.10E.4	DCI based dual DL TCI state switch delay for sDCI and mDCI

### 8.10E.4.1	DCI based dual DL TCI state switching delay for sDCI

For sDCI, if the dual target TCI states are known, when a UE is configured with the higher layer parameter tci-PresentInDCI from one TRP which is set as 'enabled' for the CORESET scheduling two PDSCHs at slot n, UE shall be able to receive PDSCHs with target TCI states of the serving cell on which TCI state switch occurs at the first slot that is after slot n+timeDurationForQCL, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and apply spatial QCL information received in DCI for PDSCHs processing as described in TS 38.214 [26], the value of timeDurationForQCL is defined in TS 38.331 [2].

For sDCI, If TCI state switching is from dual TCI states to single TCI state and the target TCI state is one of the source TCI states, there is no TCI state switching delay.

The known condition for dual TCI states defined in clause 8.10E.2 is applied.

#### 8.10E.4.2	DCI based dual DL TCI state switching delay for mDCI

For mDCI, if the dual target TCI states are known, when a UE is configured with the higher layer parameter tci-PresentInDCI from two TRPs which is set as 'enabled' for the DCIs scheduling two PDSCHs at slot n1 and n2 respectively, UE shall be able to receive PDSCHs with target TCI states of the serving cell on which TCI state switch occurs at the first slot that is after slot n1 + timeDurationForQCL and slot n2 + timeDurationForQCL respectively, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and apply spatial QCL information received in DCI for PDSCHs processing as described in TS 38.214 [26], the value of timeDurationForQCL is defined in TS 38.331 [2]. UE shall be able to simultaneously receive PDSCHs with target TCI states after slot max(n1, n2) + timeDurationForQCL.

The known condition for dual TCI states defined in clause 8.10E.2 is applied.

### 8.10E.5	RRC based dual DL TCI state switch delay

When there is single TCI configured by RRC for a CORESET other than a CORESET with index 0, for each TCI state switch, requirements in clause 8.10.5 apply.

### 8.10E.6	Active DL TCI state list update delay

#### 8.10E.6.1	Active DL TCI state list update delay for sDCI

For sDCI, if the dual target TCI states are known, upon receiving PDSCH carrying MAC-CE active TCI state list update from a TRP at slot n, UE shall be able to receive PDCCH to schedule PDSCHs from both TRPs with the new target TCI states at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(max (Tfirst-SSB1, Tfirst-SSB2)+ TSSB-proc) / NR slot length. Where THARQ, Tfirst-SSB1, Tfirst-SSB2, TSSB-proc and TOk are defined in clause 8.10E.3.

#### 8.10E.6.2	Active DL TCI state list update delay for mDCI

For mDCI case, if the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update from a TRP at slot n, UE shall be able to receive PDCCH to schedule PDSCH from the TRP with the new target TCI state at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3. Dual target TCI states can be used in the same slot for PDSCH only after both TCI states on TCI state list(s) are activated.

< End of change 1 >