3GPP TSG-RAN WG4 Meeting #110	R4-2401482

Athens, Greece, 26th February – 1st March 2024

Source:	Ericsson, T-Mobile US

Title:	TP for 38718-02-01 adding UL CA_n25A-n41C to DL CA_n25(2A)-n41C

Agenda item:			7.10.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for 38718-02-01 adding UL CA_n25A-n41C to DL CA_n25(2A)-n41C

# 2. Text Proposal

# ---Start of changes---

#### 5.70.1.2	Channel bandwidths per operating band for CA

Table 5.70.1.2-1: Supported bandwidths per CA band combination of band n25+n41

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n25A-n41C | CA_n25A-n41C | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n25(2A)-n41C | CA_n25A-n41C | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |

#### 5.70.1.3	Co-existence studies

The co-existence study has been completed for this CA except the triple-beat analysis, that this addition brings. The analysis of the IMD is found in Table 5.70.1.3-1

Table 5.70.1.3-1: Triple-beat IMD analysis of band n25+n41

| CC location | fU1L | fU2L | fU3L | fU1H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2496 | 2516 | 2666 | 2690 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min channel separation |
| Frequency | 1850 | 1915 | 2670 | 2520 |  | 10 |
| 1st order triple beat | I fUL3 -fUL1 - fSCCL ¦ | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ | I fUL3 -fUL1 + fSCCH ¦ |  | Max channel separation |
| Ranges | 1680 | 1870 | 1895 | 2085 |  | 170 |
| 1st order triple beat | I fUL2 +fUL1 - fSCCH ¦ | I fUH1 +fUH2 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ | I fUH1 +fUH2 + fSCCH ¦ |  |  |
| Ranges | 3097 | 3510 | 6862 | 7275 |  |  |

There is an IMD issue for this combination.

#### 5.70.1.4	∆TIB and ∆RIB values

These values are already part of the specification. There’s nothing new for the proposed bandwidth combination set.

#### 5.70.1.5	REFSENs requirements

Table 5.70.1.5-1 lists the MSD required due to the triple-beat IMD product of CA_n25A-n41C.

Table 5.70.1.5-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n41 | n25 | 1860 | 5 | 25 | 1940 | 15.3 | FDD | IMD3 |
|  | n41 | 2501 | 10 | 1 RBstart = 25 | 2501 | N/A | TDD | N/A |
|  |  | 2556 | 100 | 1 RBstart = 208 | 2556 |  |  |  |

#### 5.70.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

---End of changes---

# Reference

[1] 	RP-232920, Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2), RAN#102