3GPP TSG RAN WG4 Meeting #110	R4-2401505

Athens, Greece, February 26th – Marth 1th, 2024

Agenda item:	8.16.2.2

Source: 	vivo

Title: 	Discussion on test cases for R18 NR Mobility Enhancements

Document for:	Discussion

- Introduction

In Rel-18, the core part on NR Mobility Enhancements have been finalized, which include the subsequent CPAC, improvement on FR2 SCell/SCG setup delay and enhanced CHO configurations. The latest progress was captured in WF [1]. As per the working plan, the discussion on performance requirements would be triggered in RAN4#108bis. In the last meeting, companies provided initial thinking on the coverage of test cases and no details were discussed. The summary on views was captured in [2]. Based on it, we would like to further provide our views on the test cases for NR Mobility Enhancements in this paper.

- Discussion

2.1 Discussion on test case list for NR-DC with selective activation of cell groups via L3 enhancements

As listed in Table 1 below, for the test cases of subsequent CPC, both FR1+FR2 and FR1+FR1 NR-DC scenarios need to be considered. Besides, compared with the current requirement on conditional PSCell addition, conditional PSCell change does not only focus inter-frequency measurement but also intra-frequency measurement. Given this difference, we propose that the test cases for subsequent CPC need to consider both intra-frequency and inter-frequency measurement. In order to reduce the number of cases, we can assign these two cases respectively to FR1-FR1 NR-DC and FR1-FR2 NR-DC scenarios.

Specifically, there are some testability issue for the existing test cases in FR1+FR2 scenario. According to the Reply LS [3] from RAN4 in RAN4#108bis, RAN4 agreed to update the lists of test cases UE does not have to pass in Table A.3.13A. The update can be found in appendix of LS. Similarly, for the test cases related to FR1+FR2 (as shown in TC2) for subsequent CPC, our preference is to firstly define TC2 and add it to list of tests in clause A.3.13A.

For the test configuration, the test parameters in current CPA test cases can be the baseline since considering no CPC test case was defined in R16 and R17.

Table 1: Test case list for NR-DC with selective activation of cell groups via L3 enhancements in 38.133

| NR-DC with selective activation of cell groups via L3 enhancements |  |  |  |
| --- | --- | --- | --- |
| TC NO. | TC | Description | Whether to test |
| 1 | Subsequent conditional PSCell change delay (FR1-FR1 NR-DC) | Focus on intra-frequency measurement on PSCell: There are two cells in the test, Cell 1 and a FR1 neighbour cell (Cell 2) on the same frequency as the Cell 1 | Y |
| 2 | Subsequent conditional PSCell change delay (FR1-FR2 NR-DC) | Focus on inter-frequency measurement on PSCell: There are two cells in the test, Cell 1 and a FR2 neighbour cell (Cell 2) on the different frequency as the Cell 1 | Y To define the test case firstly and add it to the list of tests in TS 38.133 clause A.3.13A |

Proposal 1: RAN4 to define the test cases for NR-DC with selective activation of cell groups via L3 enhancements as below. Specifically, add the TC2 to list of tests in TS 38.133 clause A.3.13A.

- TC1: Subsequent conditional PSCell change delay (intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC)

- TC2: Subsequent conditional PSCell change delay (inter-frequency CPC from FR1-FR1 NR-DC to FR1-FR2 NR-DC) (with testability issues)

Proposal 2: Test parameters in existing conditional PSCell addition can be used as baseline for defining subsequent Conditional PSCell Change test cases.

2.2 Discussion on test case for improvement on SCell/SCG setup delay

For the part of improvement on SCell/SCG setup delay, there are two parts including existing measurements and additional measurements. Based on the latest output from RAN plenary in RAN#102, the newly introduced solution on additional measurement has been dropped. Therefore, for the performance part, we can only focus on the test design on existing measurements. Regarding this solution, it was introduced the validity check functionality for measurement results obtained before Msg1 transmission. For this new functionality, we prefer to define the corresponding test procedure to verify whether UE can correctly perform the validity check and the reported results are within the last [X] sec before msg1 transmission for RRC resume/setup request.

Proposal 3: RAN4 to define test case on existing measurement for verifying the procedure on validity check

2.3 Discussion on test case list for Enhanced CHO including target MCG and target SCG (obj.3)

For CHO with PSCell change, the current HO with PSCell performance requirements can be the reference on test case design, in which both parallel processing and sequential processing cases are specified. For NR-DC deployment, we list FR1-FR1 NR-DC and FR1-FR2 NR-DC as below.

For obj.3, since the basics procedure of CHO with PSCell change is quite similar as the combination of HO with PSCell and conditional handover, it is fine with us to reduce the number of test cases and select two of following cases to test.

Specifically, As mentioned in subsequent CPC, there are also some testability issues for CHO with PSCell change(obj.3) and CHO with CPAC (obj.4) in FR1+FR2 scenario. Similar as HO with PSCell, our preference is to firstly define the test cases related to FR1+FR2 (as shown in Table 2 and Table 3) and add them to the list of tests in TS 38.133 clause A.3.13A.

Table 2: Test case list for Enhanced CHO including target MCG and target SCG for CPC in 38.133

| Enhanced CHO including target MCG and candidate SCGs for CPC |  |  |  |  |
| --- | --- | --- | --- | --- |
| TC NO. | TC | PCell conditional handover | Conditional PSCell change | Whether to test in R18 |
| 1 | Conditional handover with PSCell change from NR-DC to NR-DC with parallel processing | Inter-frequency conditional handover from FR1 to FR1 | SMTC of target known PSCell is present in reconfigurationWithSync Target PSCell in FR1 | Y |
| 2 | Conditional handover with PSCell change from NR-DC to NR-DC with sequential processing | Inter-frequency conditional handover from FR1 to FR1 | SMTC of target unknown PSCell is present in targetcellSMTC-SCG-r16 but not in reconfigurationWithSync Target PSCell in FR1 | N |
| 3 | Conditional handover with PSCell change from NR-DC to NR-DC with parallel processing | Inter-frequency conditional handover from FR1 to FR1 | SMTC of target known PSCell is present in reconfigurationWithSync Target PSCell in FR2 | N |
| 4 | Conditional handover with PSCell change from NR-DC to NR-DC with sequential processing | Inter-frequency conditional handover from FR1 to FR1 | SMTC of target unknown PSCell is present in targetcellSMTC-SCG-r16 but not in reconfigurationWithSync Target PSCell in FR2 | Y To define the test case firstly and add it to the list of tests in TS 38.133 clause A.3.13A |

Proposal 4: RAN4 to define the test cases for Enhanced CHO including target MCG and target SCG (obj.3) as below:

- TC1: Conditional handover with PSCell change from NR-DC to NR-DC with parallel processing (both PCell and PSCell are in FR1)

  - The SMTC of target known PSCell is present in reconfigurationWithSync

- TC2: Conditional handover with PSCell change from NR-DC to NR-DC with sequential processing (PCell is in FR1 and PSCell is in FR2)

  - The SMTC of target unknown PSCell is present in targetcellSMTC-SCG-r16 but not in reconfigurationWithSync

Proposal 5: RAN4 to define test cases related to FR1-FR2 joint testing for Enhanced CHO configurations (both Obj.3 and Obj.4) and add them to list of tests in TS 38.133 v17.10.0 clause A.3.13A if there are still testability issues after assessment.

2.4 Discussion on test case list for Enhanced CHO including target MCG and candidate SCGs for CPC (obj.4)

As mentioned in Clause 2.3, there is also testability issue on FR1+FR2 joint testing in CHO with CPC from OTA perspective. The details are shown in the following table.

Specifically, for the case that CHO-only configuration (or Rel-17 CHO with SCG configuration) is provided, when CHO condition is fulfilled and no CPC condition is met at the same time, UE would directly proceed legacy CHO procedure without CPC. Although RAN4 already has defined test case on legacy CHO procedure, it is still necessary to verify whether UE can behave correctly (i.e., UE would directly perform legacy CHO procedure instead of continuing the evaluation of CPC) when both CHO-only configuration and CHO with CPC configuration are provided and CHO execution condition is met firstly. The part of UE behavior has never been verified in previous test cases. Therefore, the test for following scenario 2 also needs to be considered in Rel-18.

Table 3: Test case list for Enhanced CHO including target MCG and candidate SCGs for CPC in 38.133

| Enhanced CHO including target MCG and candidate SCGs for CPC |  |  |  |  |
| --- | --- | --- | --- | --- |
| TC NO. | TC | PCell conditional handover | Conditional PSCell change | Whether to test |
| Scenario 1: complementary CHO-only configuration is not provided |  |  |  |  |
| 1 | Conditional handover with conditional PSCell change from NR-DC to NR-DC | Inter-frequency conditional handover from FR1 to FR1 | Intra-frequency conditional PSCell change from FR1 to FR1 | Y |
| 2 | Conditional handover with conditional PSCell change from NR-DC to NR-DC | Inter-frequency conditional handover from FR1 to FR1 | Intra-frequency conditional PSCell change from FR1 to FR2 | Y Although there are still testability issues in FR1+FR2, we prefer to define the test case firstly and add it to the list of tests in TS 38.133 v17.10.0 clause A.3.13A after assessment |
| Scenario 2: complementary CHO-only configuration is provided when CHO execution condition is met, no CPC execution condition is met simultaneously |  |  |  |  |
| 3 | Conditional handover from NR-DC to NR-DC | Intra-frequency conditional handover from FR1 to FR1 |  | Y Aim to verify whether UE would directly perform legacy CHO procedure instead of continuing the evaluation of CPC |

Proposal 6: RAN4 to define the test cases for Enhanced CHO including target MCG and candidate SCGs for CPC (obj.4) for the following scenarios:

- Scenario 1: complementary CHO-only configuration is not provided

  - TC1: Conditional handover with conditional PSCell change from NR-DC to NR-DC (from FR1-FR1 to FR1-FR1)

  - TC2: Conditional handover with conditional PSCell change from NR-DC to NR-DC (from FR1-FR1 to FR1-FR2) (with testability issues)

- Scenario 2: complementary CHO-only configuration is provided and when CHO execution condition is met, no CPC execution condition is met simultaneously

  - TC1: Conditional handover from NR-DC to NR-DC (from FR1-FR1 to FR1-FR1)

- Conclusions

Proposal 1: RAN4 to define the test cases for NR-DC with selective activation of cell groups via L3 enhancements as below. Specifically, add the TC2 to list of tests in TS 38.133 clause A.3.13A.

- TC1: Subsequent conditional PSCell change delay (intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC)

- TC2: Subsequent conditional PSCell change delay (inter-frequency CPC from FR1-FR1 NR-DC to FR1-FR2 NR-DC) (with testability issues)

Proposal 2: Test parameters in existing conditional PSCell addition can be used as baseline for defining subsequent Conditional PSCell Change test cases.

Proposal 3: RAN4 to define test case on existing measurement for verifying the procedure on validity check

Proposal 4: RAN4 to define the test cases for Enhanced CHO including target MCG and target SCG (obj.3) as below:

- TC1: Conditional handover with PSCell change from NR-DC to NR-DC with parallel processing (both PCell and PSCell are in FR1)

  - The SMTC of target known PSCell is present in reconfigurationWithSync

- TC2: Conditional handover with PSCell change from NR-DC to NR-DC with sequential processing (PCell is in FR1 and PSCell is in FR2)

  - The SMTC of target unknown PSCell is present in targetcellSMTC-SCG-r16 but not in reconfigurationWithSync

Proposal 5: RAN4 to define test cases related to FR1-FR2 joint testing for Enhanced CHO configurations (both Obj.3 and Obj.4) and add them to list of tests in TS 38.133 v17.10.0 clause A.3.13A if there are still testability

issues after assessment.

Proposal 6: RAN4 to define the test cases for Enhanced CHO including target MCG and candidate SCGs for CPC (obj.4) for the following scenarios:

- Scenario 1: complementary CHO-only configuration is not provided

  - TC1: Conditional handover with conditional PSCell change from NR-DC to NR-DC (from FR1-FR1 to FR1-FR1)

  - TC2: Conditional handover with conditional PSCell change from NR-DC to NR-DC (from FR1-FR1 to FR1-FR2) (with testability issues)

- Scenario 2: complementary CHO-only configuration is provided and when CHO execution condition is met, no CPC execution condition is met simultaneously

  - TC1: Conditional handover from NR-DC to NR-DC (from FR1-FR1 to FR1-FR1)

- Reference

[1] R4-2321398 WF on R18 Further NR mobility enhancement – part 2, Apple.

[2] R4-2321399 WF on RRM performance requirements of R18 Further NR mobility enhancement, Apple.

[3] R4-2317352 Reply LS on RRM test cases with testability issues, Apple.