3GPP TSG-RAN WG4 Meeting #110	R4-2401510

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-2 on initial UE orientation of FR2-1 multiRx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-Core |  |  |  |  |  | Date: |  |  | 2024-1-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The UE alignment for default test condition in multi-Rx is missing, and the reference coordinate system is also not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add 12 DUT orientatio for UE alignment in multi-Rx - Add figure to explain coordinate system |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test condition for multiRx is not clear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex J.2, Annex L.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************************** START OF CHANGES ************************************

# J.2	Test conditions and angle definitions

Tables J.2-1 through J.2-3 below provides the test conditions and angle definitions for three permitted device alignment for the default test condition, DUT orientation 1, and two different options for each permitted device alignment to re-position the device for DUT Orientation 2 as outlined by figures in Tables J.2-1 through J.2-3.

Table J.2-1: Test conditions and angle definitions for Alignment Option 1

| Test condition | DUT orientation | Link angle | Measurement angle | Diagram |
| --- | --- | --- | --- | --- |
| Free space DUT Orientation 1 (default) | α = 0º; β = 0º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| Free space DUT Orientation 2 – Option 1 (based on re-positioning approach) | α = 180º; β = 0º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| Free space DUT Orientation 2 – Option 2 (based on re-positioning approach) | α = 0º; β = 180º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| NOTE 1: A polarization reference, as defined in relation to the reference coordinate system in J.1-1, is maintained for each signal angle, link or interferer angle, and measurement angle. NOTE 2: The combination of rotations is captured by matrix M=Rz(g)•Ry(b)•Rx(a) |  |  |  |  |

Table J.2-2: Test conditions and angle definitions for Alignment Option 2

| Test condition | DUT orientation | Link angle | Measurement angle | Diagram |
| --- | --- | --- | --- | --- |
| Free space DUT Orientation 1 (default) | α = 0º; β = -90º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| Free space DUT Orientation 2 – Option 1 (based on re-positioning approach) | α = 180º; β = 90º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| Free space DUT Orientation 2 – Option 2 (based on re-positioning approach) | α = 0º; β = 90º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| NOTE 1: A polarization reference, as defined in relation to the reference coordinate system in J.1-1, is maintained for each signal angle, link or interferer angle, and measurement angle. NOTE 2: The combination of rotations is captured by matrix M=Rz(g)•Ry(b)•Rx(a) |  |  |  |  |

Table J.2-3: Test conditions and angle definitions for Alignment Option 3

| Test condition | DUT orientation | Link angle | Measurement angle | Diagram |
| --- | --- | --- | --- | --- |
| Free space DUT Orientation 1 (default) | α = 90º; β = 0º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| Free space DUT Orientation 2 – Option 1 (based on re-positioning approach) | α = -90º; β = 0º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| Free space DUT Orientation 2 – Option 2 (based on re-positioning approach) | α = 90º; β = 180º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |
| NOTE 1: A polarization reference, as defined in relation to the reference coordinate system in J.1-1, is maintained for each signal angle, link or interferer angle, and measurement angle. NOTE 2: The combination of rotations is captured by matrix M=Rz(g)•Ry(b)•Rx(a) |  |  |  |  |

For each UE requirement and test case, each of the parameters in Table J.2-1 through J.2-3 need to be recorded, such that DUT positioning, DUT beam direction, and angles of the signal, link/interferer, and measurement are specified in terms of the fixed coordinate system.

Due to the non-commutative nature of rotations, the order of rotations is important and needs to be defined when multiple DUT orientations are tested.

The rotations around the x, y, and z axes can be defined with the following rotation matrices

and

.

with the respective angles of rotation, a, b, g, and

Additionally, any translation of the DUT can be defined with the translation matrix

with offsets tx, ty, tz in x, y, and z, respectively and with

The combination of rotations and translation is captured by the multiplication of rotation and translation matrices.

For instance, the matrix M

M=Ttx,ty,tz·Rzγ·Ryβ·Rxα

describes an initial rotation of the DUT around the x axis with angle , a subsequent rotation around the y axis with angle , and a final rotation around the z axis with angle . After those rotations, the DUT is translated by tx, ty, tz in x, y, and z, respectively.

For the requirement of simultaneous reception from multiple directions, there are 12 permitted device alignment for the default test condition,  as shown in Table J.2-4.

Table J.2-4: Initial DUT orientation for simultaneous reception from multiple directions

| DUT orientation | α | β | γ |
| --- | --- | --- | --- |
| 1 | 0° | 0° | 0° |
| 2 | 180° | 0° | 0° |
| 3 | 0° | 180° | 0° |
| 4 | 0° | 0° | 180° |
| 5 | 0° | -90° | 0° |
| 6 | 180° | 90° | 0° |
| 7 | 0° | 90° | 0° |
| 8 | 0° | -90° | 180° |
| 9 | 90° | 0° | 0° |
| 10 | -90° | 0° | 0° |
| 11 | 90° | 180° | 0° |
| 12 | 90° | 0° | 180° |

***************************** UNCHANGED CLAUSES OMITTED **********************************

Annex L (normative): Requirement metric for simultaneous reception from multiple directions

# L.1	Spherical coverage grid for simultaneous reception in multiple directions

The reference coordinate system grid is defined as follows: The grid is the set of locations described by a source/probe from the UE’s perspective when the TE completes a 3D spherical scan with a constant step size for each rotational axis of its positioner. The grid comprises intersection points of circles of fixed polar angle (geographic analog: ‘latitude’) and meridians of fixed azimuthal angle (geographic analog: ‘longitude’) and completely covers the sphere around the UE. A valid set of locations for the source/probes are those that describes the same constant step size grid from the UE’s perspective over a complete spherical scan. This condition is satisfied when the active probes/sources are always contained in a plane of constant azimuthal angle ‘’ in this reference system. The reference coordinate system with example UE orientation is shown in figures L.1-1.

Figure L.1-1 Reference coordinate system with example UE orientation

********************************* END OF CHANGE **************************************