3GPP TSG-RAN WG4 Meeting #110	R4-2401539
Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.11.2

Source:	vivo

Title:	Analysis of 3GPP TRP TRS AC lab alignment and RC harmonization measurement results-phase 2

Document for:	Approval

1	Introduction

Based on the agreed working procedure [1] and pass fail limit [2], phase 1 AC lab alignment has been concluded in RAN4#109 meeting. 6 test labs are well aligned at band n78 [2].

This contribution presents the full picture of Rel-18 AC lab alignment and RC harmonization outcome with updated analysis of phase 2 LAD1/2/3/4 measurement data, and proposes the conclusions for the TRP TRS AC lab alignment activity for band n78 and n28 BHH.

2	Discussion

The reference value is derived based on linear average of all the labs results. The detailed analysis file is in the attached Excel worksheet of this contribution.

BHH: Beside Head and Hand, i.e., talk mode

BH: Beside Hand only, i.e., browsing mode

Figure 1 share the measured results of LAD1 and LAD2 for band n78, LAD3 and LAD4 for band n28, from each test lab. Figure 2 analyse the deviation between each test lab and reference value.

Figure 1: NR FR1 TRP and TRS AC lab alignment measurement results from each test lab (no n28 BHH results from lab1)

Figure 2: NR FR1 TRP and TRS AC lab alignment analysis, deviation between each test lab and reference value (no n28 BHH results from lab1)

Observation 1: For TRP, no outlier value is identified. The reference value is derived by the agreed linear average (with dBm) of all the available results, the maximum deviation between test labs and reference value is 0.76dB for n78 TRP and 1.12dB for n28 TRP.

Observation 2: For TRS, no outlier value is identified. The reference value is derived by the agreed linear average (with dBm) of all the available results, the maximum deviation between test labs and reference value is 1.24dB for n78 TRS and 2.14dB for n28 TRS.

Observation 3: The maximum deviation between test labs and reference value for n78 (LAD1 and LAD2) TRP and TRS are within 0.75*MU, all the 8 labs with available results are well aligned at band n78.

Observation 4: The maximum deviation between test labs and reference value for n28 (LAD3 and LAD4) TRP are within 0.75*MU, all the 7 labs with available results are well aligned at band n28 TRP. However, the maximum deviation between one test lab and reference value for TRS is beyond 0.75*MU, 6 labs are well aligned, 1 lab fail the alignment, 1 lab’s results is not available yet.

Observation 5: Compared with phase 1 results submitted in RAN4#109 meeting, two labs (Lab4 and Lab5) have updated the LAD1 TRS measurement results based on retesting. The updated results are considered in this analysis.

With above analysis, the following proposal can be made as the phase 2 FR1 TRP TRS AC lab alignment outcome:

Proposal 1: 3GPP Rel-18 phase 2 FR1 TRP TRS AC lab alignment activity for BHH can be successfully concluded within the 8 labs with anechoic chamber system are well aligned at n78.

Proposal 2: 3GPP Rel-18 phase 2 FR1 TRP TRS AC lab alignment activity for BHH can be successfully concluded within 6 labs with anechoic chamber system are well aligned at n28.

We also analyse the RC measured results, with the following summary：

Figure 3: NR FR1 TRP and TRS RC lab alignment measurement results from each test lab (no RC results for Lab5, Lab7 and Lab8)

Figure 4: NR FR1 TRP and TRS RC lab alignment analysis, deviation between each test lab and RC reference value (no RC results for Lab5, Lab7 and Lab8)

Observation 6: For n78 and n28 TRP, the reference value is derived by the agreed linear average (with dBm) of all the 5 labs results, the maximum deviation between test labs and RC reference value is 1.47dB for BHH TRP and 1.18dB for BH TRP.

Observation 7: For n78 and n28 TRS, the reference value is derived by the agreed linear average (with dBm) of all the 5 labs results, the maximum deviation between test labs and reference value 1.70dB for BHH TRS and 1.58dB for BH TRS.

Observation 8: The max RC lab alignment deviation is within 0.75*MU, i.e., 0.75*1.9dB for TRP browsing mode, 0.75*2.08 for TRP talk mode, 0.75*2.28dB for TRS browsing mode, 0.75*2.43dB for TRS talk mode.

Proposal 3: 3GPP Rel-18 FR1 TRP TRS RC lab alignment activity can be successfully concluded within 5 labs with Reverb chamber system are well aligned.

In addition, we also analyse the RC vs AC gap of same device from each test lab, summarized as following:

Figure 5: RC vs AC measurement results of same device in each test lab (no RC results for Lab5, Lab7 and Lab8)

Observation 9: The RC vs AC gap of same UE in each test lab is Max. 1.28dB for TRP and Max 1.66dB for TRS.

Regarding RC with AC harmonization, given still lacking two labs results for RC system, it is proposed to further discuss the RC harmonization criteria and pass fail limits.

Proposal 4: RC harmonization can be further discussed until test labs finalize the LADs measurement.

3 Conclusion

Based on the above analysis in this paper, the following outcome of Rel-18 3GPP FR1 TRP TRS AC lab alignment activity (phase 2) and RC harmonization is proposed:

Observation 1: For TRP, no outlier value is identified. The reference value is derived by the agreed linear average (with dBm) of all the available results, the maximum deviation between test labs and reference value is 0.76dB for n78 TRP and 1.12dB for n28 TRP.

Observation 2: For TRS, no outlier value is identified. The reference value is derived by the agreed linear average (with dBm) of all the available results, the maximum deviation between test labs and reference value is 1.24dB for n78 TRS and 2.14dB for n28 TRS.

Observation 3: The maximum deviation between test labs and reference value for n78 (LAD1 and LAD2) TRP and TRS are within 0.75*MU, all the 8 labs with available results are well aligned at band n78.

Observation 4: The maximum deviation between test labs and reference value for n28 (LAD3 and LAD4) TRP are within 0.75*MU, all the 7 labs with available results are well aligned at band n28 TRP. However, the maximum deviation between one test lab and reference value for TRS is beyond 0.75*MU, 6 labs are well aligned, 1 lab fail the alignment, 1 lab’s results is not available yet.

Observation 5: Compared with phase 1 results submitted in RAN4#109 meeting, two labs (Lab4 and Lab5) have updated the LAD1 TRS measurement results based on retesting. The updated results are considered in this analysis.

Proposal 1: 3GPP Rel-18 phase 2 FR1 TRP TRS AC lab alignment activity for BHH can be successfully concluded within the 8 labs with anechoic chamber system are well aligned at n78.

Proposal 2: 3GPP Rel-18 phase 2 FR1 TRP TRS AC lab alignment activity for BHH can be successfully concluded within 6 labs with anechoic chamber system are well aligned at n28.

Observation 6: For n78 and n28 TRP, the reference value is derived by the agreed linear average (with dBm) of all the 5 labs results, the maximum deviation between test labs and RC reference value is 1.47dB for BHH TRP and 1.18dB for BH TRP.

Observation 7: For n78 and n28 TRS, the reference value is derived by the agreed linear average (with dBm) of all the 5 labs results, the maximum deviation between test labs and reference value 1.70dB for BHH TRS and 1.58dB for BH TRS.

Observation 8: The max RC lab alignment deviation is within 0.75*MU, i.e., 0.75*1.9dB for TRP browsing mode, 0.75*2.08 for TRP talk mode, 0.75*2.28dB for TRS browsing mode, 0.75*2.43dB for TRS talk mode.

Proposal 3: 3GPP Rel-18 FR1 TRP TRS RC lab alignment activity can be successfully concluded within 5 labs with Reverb chamber system are well aligned.

Observation 9: The RC vs AC gap of same UE in each test lab is Max. 1.28dB for TRP and Max 1.66dB for TRS.

Proposal 4: RC harmonization can be further discussed until test labs finalize the LADs measurement.

References

- R4-2316995, Working procedure for Rel-18 lab alignment and harmonization campaign, vivo, RAN4#108bis

- R4-2321099, WF on NR_FR1_TRP_TRS_enh, vivo, 3GPP TSG-RAN WG4 Meeting #109

- R4-2318103	Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity-Huawei	Huawei, HiSilicon

- R4-2318970	Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity	vivo

- R4-2319641	3GPP Rel-18 TRP TRS AC lab alignment activity from SRTC	SRTC

- R4-2319920	3GPP Rel-18 TRP TRS LAD measurement for AC lab alignment_n78	OPPO

- R4-2319288	Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity	SGS Wireless

- R4-2320177	CAICT measurement results for 3GPP Rel-18 TRP TRS RC harmonization activity	CAICT

- R4-2318104	Measurement results for 3GPP Rel-18 TRP TRS RC harmonization-Huawei	Huawei, HiSilicon

- R4-2318971	Measurement results for 3GPP Rel-18 TRP TRS RC harmonization activity	vivo

- R4-2319635	3GPP Rel-18 TRP TRS RC harmonization from SRTC	SRTC

- R4-2319921	3GPP Rel-18 TRP TRS LAD measurement for RC harmonization_n78	OPPO

- R4-2320178	CAICT measurement results for 3GPP Rel-18 TRP TRS RC harmonization activity	CAICT

- R4-2400217	3GPP Rel-18 TRP TRS LAD measurements for AC lab alignment - Sporton	Sporton International Inc

- R4-2400551	3GPP Rel-18 TRP TRS LAD measurements for AC lab alignment - Sporton n28	Sporton International Inc

- R4-2400717	ELEMENT - Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment LAD1 and LAD2	Element Materials Technology

- R4-2400718	ELEMENT - Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment LAD3 and LAD4	Element Materials Technology

- R4-2400871	3GPP Rel-18 TRP TRS AC lab alignment activity from SRTC	SRTC

- R4-2401540	Measurement results of 3GPP Rel-18 TRP TRS AC lab alignment activity-full set	vivo

- R4-2401803	3GPP Rel-18 TRP TRS LAD measurement for AC lab alignment_n28	OPPO

- R4-2401834	CAICT measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity-v2	CAICT

- rev of R4-2400980	Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity	SGS Wireless

- R4-2401802	Update on LAD1 measurement data for lab alignment	OPPO

- R4-2400870	3GPP Rel-18 TRP TRS RC lab alignment activity from SRTC	SRTC

- R4-2401541	Measurement results of 3GPP Rel-18 TRP TRS RC harmonization activity-full set	vivo

- R4-2401801	3GPP Rel-18 TRP TRS LAD measurement for RC harmonization_n28	OPPO

- R4-2401835	CAICT measurement results for 3GPP Rel-18 TRP TRS RC harmonization activity-v2	CAICT

- R4-240xxxx	Huawei-Measurement results for 3GPP Rel-18 TRP TRS RC harmonization	Huawei, HiSilicon