3GPP TSG-RAN WG4 Meeting #110                                                       R4-2401547

Athens, Greece, 26th Feb. – 1st Mar. 2024

Agenda Item:	8.9.7.2

Source:	Ericsson

Title:	On PDSCH requirements for ATG network

Document for:	Discussion

# 1	Introduction

In this contribution, we discuss the possibility of introducing PDSCH requirements with 1024QAM.

# 2	Discussion

1024QAM has been already introduced for TN network, and there are corresponding requirements based on TDL-D channel model. Comparing to the existing 1024QAM requirement of TN network, ATG network has similar or even better channel condition so that it gives the worth of considering the possibility. To assist the discussion, we delivered our link budget analysis based on the agreed ATG BS and UE assumptions.

The distance between ATG UE and BS can be 10km ~ 300km. The typical propagation channel is single tap channel, so free space propagation loss can be used. Other parameters for ATG BS and ATG UE are captured in table 2-1 which refers to TS38.876 6.2.2 [3]. Subarray antenna model is used for both FDD and TDD ATG BS in the following calculations.

Table 2-1 System and antenna parameters for ATG BS and UE

| Centre frequency [GHz] |  | 2 | 4 |
| --- | --- | --- | --- |
| CBW [MHz] |  | 10 | 40 |
| PRB number |  | 52 | 106 |
| ATG BS system | Tx max power [dBm] | 43 | 50 |
|  | noise figure [dB] | 5 | 5 |
| ATG BS antenna | Element gain [dBi] | 6.4 |  |
|  | Array configuration (Row × Column) | 4x8 |  |
|  | Sub-array elements | 3 |  |
| ATG UE system | TRP or EIRP [dBm] | 40 | 43 |
|  | noise figure [dB] | 9 | 9 |
| ATG UE antenna | element gain [dBi] | 0 | 5 |
|  | Array configuration (Row × Column) | \ | (8x2) or (16x1) |

The SNR at receiver side for 2GHz and 4GHz are showed in figure 2-1 and 2-2.

Figure 2-1 Received SNR for FDD

Figure 2-2 Received SNR for TDD

As illustrated in the figure 2-1 and figure 2-2, we observed that there is enough SNR margin for some distance. To be specific, 1024QAM is possible to be considered for distance around 50km for TDD and distance around 70km for FDD.

Observation 1: There are applicable scenarios for downlink 1024QAM according to the link budget analysis.

To assist the discussion, figure 2-3 to 2-5 showed our simulations with 1024QAM to see the operating SNR at different level of max Tput. MCS23 with coding rate of 0.79 was used.

Figure 2-3 PDSCH performance of 1024QAM

Figure 2-4 PDSCH performance of TDD with legacy pattern

Figure 2-5 PDSCH performance of TDD with new pattern

Based on the simulation results above, for both FDD and TDD, the operating SNR for 2x2 configuration at 70% and 95% max Tput is around 23.5dB and 24.64dB respectively, which verified that it is possible to consider 1024QAM for PDSCH requirements for ATG network.

Observation 2: According to the simulation results, there is enough margin to consider 1024QAM with MCS23 for PDSCH demodulation requirement for both FDD and TDD.

Proposal 1: Introduce 1024QAM requirements with MCS23 for ATG PDSCH demodulation, following test cases can be considered:

| Test number | Duplex mode, SCS and CBW | Propagation condition | Antenna configuration | MCS | SNR @70% maxTput |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD 15kHz/10MHz | AWGN+220Hz Doppler | 2T2R | 1024QAM Table MCS23 0.79 |  |
| 2 |  |  | 2T4R | 1024QAM Table MCS23 0.79 |  |
| 3 | TDD 30kHz/40MHz 7DS2U | AWGN+500Hz Doppler | 2T2R | 1024QAM Table MCS23 0.79 |  |
| 4 |  |  | 2T4R | 1024QAM Table MCS23 0.79 |  |
| 5 | TDD 30kHz/40MHz 30D4S6U | AWGN+500Hz Doppler | 2T2R | 1024QAM Table MCS23 0.79 |  |
| 6 |  |  | 2T4R | 1024QAM Table MCS23 0.79 |  |

# 3	Summary

In this contribution, we shared our views and simulation results on the possibility of considering 1024QAM in ATG PDSCH requirement. Based on the above, we summarized our observations and proposals as follows:

Observation 1: There is applicable scenarios for downlink 1024QAM according to the link budget analysis.

Observation 2: According to the simulation results, there is enough margin to consider 1024QAM with MCS23 for PDSCH demodulation requirement for both FDD and TDD.

Proposal 1: Introduce 1024QAM requirements for ATG PDSCH demodulation, following test cases can be considered:

| Test number | Duplex mode, SCS and CBW | Propagation condition | Antenna configuration | MCS | SNR @70% maxTput |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD 15kHz/10MHz | AWGN+220Hz Doppler | 2T2R | 1024QAM Table MCS23 0.79 |  |
| 2 |  |  | 2T4R | 1024QAM Table MCS23 0.79 |  |
| 3 | TDD 30kHz/40MHz 7DS2U | AWGN+500Hz Doppler | 2T2R | 1024QAM Table MCS23 0.79 |  |
| 4 |  |  | 2T4R | 1024QAM Table MCS23 0.79 |  |
| 5 | TDD 30kHz/40MHz 30D4S6U | AWGN+500Hz Doppler | 2T2R | 1024QAM Table MCS23 0.79 |  |
| 6 |  |  | 2T4R | 1024QAM Table MCS23 0.79 |  |

# References

[1]	R4-2311795: Summary of simulation results for ATG UE and BS demodulation requirements, CMCC, RAN4 #108, Toulouse

[2]          R4-2316991: WF for ATG demodulation requirements, CMCC, RAN4 #108bis, Xiamen, China

[3]	TS38.876, v0.2.0