3GPP TSG-TSG RAN4 Meeting #110	R4-2401571

# Athens, Greece, 26th Feb – 1st March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | <> | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for channel model on Rel-18 FR2 HST demodulation requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The channel model has been introduced in Rel-18 FR2 HST |  |  |  |  |  |  |  |  |
| Summary of change: |  | The draft CR R4-2319839 has been submitted in RAN4#109 meeting and endorsed Introduce new section for channel model for FR2 HST |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE demodualtion requirement for FR2 HST scenario can not be verfied |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6, 8.1.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Resumbitted R4-2319839 |  |  |  |  |  |  |  |  |

<Start of Change 1>

B.3.4	FR2 HST-DPS Channel Profile

There is an infinite number of RRHs distributed equidistantly along the railway track with the same Cell ID as illustrated in Figure B.3.4.1-1 for Unidirectional, Figure B.3.4.2-1 for Bidirectional, and Figure 8.3.4.X-1 for Bidirectional with Multi-Rx chain reception

The location of RRH k is given as:

xk=k*Ds+j*Dmin	(B.3.4.1)

where:	k∈[-∞,∞], j=sqrt(-1) and Dminis the distance between the RRHs and railway track, while Ds is the distance of two RRHs, both in meters.

The train location is denoted as:

y=a+j*0	(B.3.4.2)

where:	a∈[0-Ds_offset,∞] and a means distance in meters, which means the train is right on the track. a=a0+vt where v (m/s) is the moving speed of the train,  a0=-Ds_offset for B.3.4.1 Unidirectional Deployment Channel Profile and for B.3.4.x Bidirectional Deployment Channel Profile with multi-Rx chain reception, and a0=0 for B.3.4.2 Bidirectional Deployment Channel Profile.

<End of Change 1>

B.3.4.2	Bidirectional Deployment Channel Profile

<Un changed skip>

<Start of Change 2>

B.3.4.x	Bidirectional Deployment Channel Profile with Multi-Rx Chain Reception

The FR2 HST DPS Bidirectional Deployment Channel Profile with Multi-Rx Reception is a single tap propagation channel for each Rx chain, switching transmission point between adjacent RRHs for UE with each Rx Chain when the UE reaches a distance equal to DS_offset from the serving RRH as illustrated in Figure B.3.4.x-1:

Figure B.3.4.x-1: Bidirectional deployment of FR2 HST-DPS with Multi-Rx Chain Reception (To be updated)

RRH k is visible for the train only in the range:

(k-1)*Ds-Dsoffset≤a<(k+1)*Ds+Ds_offset	(B.3.4.x.1)

However, as shown in Figures B.3.4.x-1, RRH k is considered for PDSCH and PDCCH signal transmission only received by UE with right Rx Chain in the range:

(k - 1)*Ds-Ds_offset≤a<k*Ds- Ds_offset	(B.3.4.x.2)

and RRH k is considered for PDSCH and PDCCH signal transmission only received by UE with left Rx Chain in the range:

k*Ds+Dsoffset≤a<k+1*Ds+ Ds_offset	(B.3.4.x.3)

Propagation delay difference are not considered between signals from different RRHs.

Power level Pk (dB) for the signal from each RRH equals to 0.

Doppler shift fst=fd cosθt (Hz) for PDSCH and PDCCH received by train with right Rx Chain is given by:

cosθt=Ds_offset+Ds-vtDmin2+Ds_offset+Ds-vt2,  0<t≤Dsv

cosθ(t)=cost modDsv,  t>Dsv 	(B.3.4.x.4)

Doppler shift fst=fd cosθt (Hz) for PDSCH and PDCCH received by train with left Rx Chain is given by:

cosθt=-Ds-Ds_offset+vtDmin2+Ds-Ds_offset+vt2,  0<t≤2Ds_offsetv--Ds_offset+vtDmin2+-Ds_offset+vt2,  2Ds_offsetv<t≤Dsv

cosθ(t)=cost modDsv,  t>Dsv 	(B.3.4.x.5)

Doppler shift is given by equation B.3.4.x.4 and B.3.4.x.5, where the required input parameters listed in table B.3.4.x-1 and the resulting Doppler shift shown in Figures B.3.4.x-2 and B.3.4.x-3 are applied for all frequency bands.

Table B.3.4.x-1: FR2 HST-DPS Bidirectional scenario

| Parameter | Value |
| --- | --- |
|  | HST-DPS-FR2-BI-B-MR |
| Ds | 700 m |
| Ds_offset | 100 m |
| Dmin | 150 m |
| v | 350 km/h |
| fd | 9722 Hz |

Static channel matrix will be used as defined in Annex B.1.

Figure B.3.4.x-2: Doppler shift trajectory (f_d = 9722 Hz) showing visibility 
of each RRH for FR2 HST-DPS Bidirectional scenario with mutlit-Rx Chain Reception

Figure B.3.4.2-3: Doppler shift trajectory (f_d = 9722 Hz) as seen by PDCCH and PDSCH
for each RRH for FR2 HST-DPS Bidirectional scenario with multi-Rx Chain Reception

<End of Change 2>