3GPP TSG-RAN WG4 Meeting #110             	R4-2401577

Athens, Greece, 26th Feb-1st March, 2024

Agenda item:	8.10.6.2

Source:	Samsung

Title:	Discussion and initial results for BS demodulation requirement for less than 5MHz

Document for:     Discussion

- Introduction

In the last RAN4 meeting, the test scope of performance part of NR support for dedicated spectrum less than 5MHz for FR1 was under discussion. The related agreement was captured in the WF [1] as following

- General - Manufacturer declaration for 3MHz CBW - Introduce new BS manufactory declaration for the support of less than 5MHz CBW - Will BS exist that supports only less than 5MHz CBW - For the purpose of demodulation test coverage in Rel-18, assume there are no BSs that support only less than 5MHz - PUSCH requirement - A need for PUSCH requirement with less than 5MHz CBW - The issue requires further discussion - Option 1: Introduce limited set of PUSCH requirements for less than 5MHz - FFS for requirements in HST condition, including PUSCH demodulation performance and or UL timing adjustment - Option 2: Don’t define PUSCH requirements with 3MHz bandwidth - Testing of PUSCH with/without precoding enabled - If PUSCH requirement for less than 5MHz CBW is decided to be introduced, then only consider CP-OFDM - UCI multiplexing on PUSCH - RAN4 shall not define requirements for UCI multiplexed on PUSCH for the less than 5MHz work item - PUSCH simulation parameters for normal conditions - Consider the following parameters for the evaluation of PUSCH performance - Number of PRBs: 12, 15 - MCS: 2,16, 20 - 1T2R， 1 layer - Mapping type A - DMRS (1+0), (1+1) - Channel Conditions: For MCS 2, TDLB 100-400, For MCS 20, TDLA 30-10, For MCS 16, TDLC 300-100 - Interested companies are encouraged to provide simulation results - Further down selection of parameters is not precluded - PUSCH requirements in HST condition - Introduction of PUSCH requirements in HST conditions require further discussions - 3 DMRS if HST - The existing high speed Train condition (Clause G.3 in TS 38.014) at 500km/h can be reused as a starting point - Ds =700m. Dmin=100m - Maximum Doppler shift f_d =815Hz - Companies are encouraged to bring simulation results for PUSCH demodulation performance - Interested companies can also evaluate UL timing adjustment requirement in Moving propagation conditions (Clause G.4 from TS 38.104), Scenario Z - PUCCH requirement - Introduction of PUCCH requirements - Introduce new PUCCH format 2 for UCI BLER requirements for 3MHz. - Other formats are not precluded - Use the following parameters for the requirement - PUCCH requirement with frequency hopping for 1Tx2Rx antenna configuration - Use TDLC 300-100 channel condition - PRACH - Applicability rule or note for long RACH sequence - The issues require further discussion - Option 1: Add applicability rule or not - Option 1-a: For BS supporting less than 5MHz carrier bandwidth only test requirements relating to short RACH preamble formats with 15KHz SCS, and long PRACH formats with 1.25KHz SCS shall apply - Option 1-b: Unless other wise stated, for the subcarrier spacing to be tested, the test requirements shall apply only for anyone channel bandwidth not less than 20MHz declared to be supported (see D.14 in table 4.6-1) - Option 2: No need to add applicability rule or not

In this contribution, the view on remaining test scope and test setup of BS demodulation requirement for NR less than 5MHz. Meanwhile, the initial simulation results are provided for alignment purpose.

- Test scope of BS demodulation requirement

PUSCH requirement

In the last meeting, the test scope of BS demodulation requirement with less than 5MHz was under discussion. It was agreed to introduce new BS manufactory declaration for the support of less than 5MHz CBW. And for the purpose of demodulation test coverage in Rel-18, assume that there are no BSs that support only less than 5MHz.

For BS side, the requirements defined is manufacture declaration based. BS vendor can declare to support single CBW or multiple CBWs. For BS supporting multiple CBWs including less than 5MHz, to guarantee the requirement of less than 5MHz can be tested and the existing applicability rule be reused, the dedicated PUSCH requirement with less than 5MHz CBW can be introduced.  Given there is no any physical layer enhancement, the limited test cases of PUSCH requirements with 3MHz for 15KHz SCS could be introduced.

Proposal 1: Only limited set of PUSCH requirements for less than 5MHz can be introduced

PUSCH requirement in HST

Based on RAN-P conclusion as following,

RP-230781 Response to LS from RAN4 on spectrum less than 5 MHz RAN Plenary would like to thank RAN WG4 for their LS for spectrum less than 5 MHz. RAN Plenary has discussed question 1 on legacy bands and UE operation, and concluded the following: - In some bands where the <5MHz feature is planned to be deployed there may be legacy NR UEs, whereas in others there are no legacy NR UEs. - In order to limit the impact to any legacy UEs in the same frequency range, it would be helpful if the sync raster can be differentiated for the less-than-5MHz channels. - It is assumed that UE support of the <5MHz feature is band-specific and optional. RAN Plenary has discussed question 2 on the feature list to be considered, and concluded that the less-than-5MHz WI in Rel-18 should consider single-carrier operation, excluding RedCap.In addition, UE speeds up to 500km/h should be targeted for band n100 without impact to RAN1

Based on the WID, 3MHz channel bandwidth was introduced in bands n100, n8, n26 and n28, support different deployment scenarios, in order to be able to start the migration from GSM-R to FRMCS in Europe.UE supporting 3MHz, would be supported high speed up to 500km/h, where the targeting band is n100. Therefore, the PUSCH with high doppler scenario can be considered.

Regarding the test scope of HST condition, both UL timing adjustment and PUSCH with 350km/h and 500km/h were introduced in Rel-16 NR HST WI. Since the UE speed will be up to 500km/h, if the BS can pass the requirement with 500km/h, it should no issue to pass the requirement with 300km/h. Therefore, the UL timing adjustment and PUSCH requirements with related with UE speed 500km/h can be introduced.

Proposal 2: UL timing adjustment and PUSCH requirements with related with UE speed 500km/h could be introduced.

PUCCH requirement

In the last meeting, RAN4 agreed to introduce new PUCCH format2 for UCI BLER requirement for 3MHz. For other formats are not precluded. In general, there is no enhancement are needed for PUCCH to support transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth.

In Rel-15, the PUCCH requirements with all the formats were considered for different CBWs. RAN4 has introduced the test applicability rule for requirements with different formats. To ensure the test coverage of different PUCCH formats, if PUCCH requirement with 3MHz introduced, the same PUCCH formats can be considered to specify the related PUCCH requirement with 3MHz, reusing the existing test applicability rule

Proposal 3: The same PUCCH formats can be considered to specify the related PUCCH requirement with 3MHz

PRACH requirement

Based on RAN1 conclusion, there is no enhancements for PRACH to operate NR on transmission bandwidths of < 5MHz for 3MHz and 5MHz channel bandwidth, which means existing PRACH formats with 15KHz SCS, and long PRACH formats with 1.25KHz SCS can support 3MHz.

As agreed in the last meeting, no new PRACH requirement need to be introduced for less than 5MHz channel bandwidth. The only remaining open issue is that whether applicability rule for long RACH sequence in less than 5MHz bandwidths is needed.

In general, the PRACH requirement is no differentiate with transmission BWs if its configuration can be fitted into the transmission BW. In Rel-15, both long PRACH formats and short formats requirements are introduced. And short formats requirements with sequence length as LRA=1151 for 15KHz SCS was introduced in Rel-16, where the main targeting BW should be 20 MHz channel bandwidth.

Based on the RAN4 RF discussion, the minimum number of RB as 15 for 3MHz was introduced excepting for n100, and the minimum number of RB as 12 for 3MHz with n100. Then, the available number of subcarriers is 180 for 15RB, and 144 for 12RB, respectively, where the existing long PRACH format with LRA=839 and short PRACH formats with LRA=139 can be fitted into the transmission BW of 3MHz, while the existing short PRACH formats with LRA=1151 cannot be fitted into the transmission BW of 3MHz, considering it is targeting for 20MHz channel bandwidth.

Observation 1: Existing long PRACH formats with 1.25KHz SCS with sequence length LRA=839, and long PRACH formats with 15KHz SCS with sequence length LRA=139 can support 3MHz channel bandwidth.

Observation 2: Existing short PRACH formats with 15KHz SCS with sequence length LRA=1151, cannot apply for 3MHz channel bandwidth, considering it is targeting for 20MHz channel bandwidth.

Therefore, not all the PRACH requirements defined in existing spec can be applied for 3MHz channel bandwidth. It was also captured in RAN1 agreement as

- Note: PRACH formats and configurations not fitting into transmission BW are not applicable.

Observation 3: A note is added to indicate PRACH formats and configurations are not fitting into transmission BW are not applicable in RAN1 agreement

So, to align with RAN1 agreement, we think adding an applicability rule is clear in the RAN4 specification. Since it was agreed to introduce new BS manufactory declaration for the support of less than 5MHz CBW, we prefer to add applicability rule as following

- For BS supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply

Proposal 4: Adding an applicability rule for PRACH requirement targeting BS supporting less than 5MHz carrier bandwidth

- For BS supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply

- Test setup of BS demodulation requirement

In this section, the view on test setup for BS demodulation requirement is provided, including the PUSCH and PUCCH requirements.

## 3.1	Test setup of PUSCH requirements

Test Parameters for PUSCH with normal condition

In the last meeting, the initial test parameters are listed for evaluation as following

- Number of PRBs: 12, 15 - MCS: 2,16, 20 - 1T2R， 1 layer - Mapping type A - DMRS (1+0), (1+1) - Channel Conditions: For MCS 2, TDLB 100-400, For MCS 20, TDLA 30-10, For MCS 16, TDLC 300-100

Regarding of PRBs, both 12 and 15 are available pending on different frequency band.

Regarding MCS, MCS2, MCS16 and MCS 20 are considered for PUSCH requirements with different channel bandwidths to guarantee the test coverage of MCS in Rel-15. For 3MHz, since assuming there are no BSs that support only less than 5MHz, there is no necessary to introduce the PUSCH requirement with 3MHz for different MCS, to reduce the test effort, only one MCS is chosen for requirement definition

Regarding the number of DMRS symbol, both 1+0 and 1+1 DMRS configurations are considered for evaluation. 1+1 DMRS configuration should be the typical configuration in the fading channel environment. Meanwhile, there is no requirement with 1+0 DMRS configuration for different channel bandwidth in Rel-15. Therefore, only DMRS configuration with 1+1 is considered for PUSCH requirement definition.

Regarding PUSCH mapping type, both PUSCH mapping type A and type B were considered for PUSCH requirement with larger than 5MHz in Rel-15, we are also open to further discuss the necessity of PUSCH mapping type B, considering the related test applicability rule has introduced in RAN4.

In summary, to reduce the test effort, the following test parameters can be considered for down selection for specifying PUSCH requirement with 3MHz

Proposal 5: The following test parameters can be considered for down selection for specifying PUSCH requirement with 3MHz

- Number of PRBs: 12, 15

- MCS 16

- 1T2R, 1layer

- TDLC300-100

- DMRS 1+1

- Mapping type A

Test Parameters for PUSCH with HST condition

Similar as PUSCH with normal condition, the initial test parameter was discussed for initial evaluation in the last meeting.

Given there is no clear HST deployment modeling for rail communication in less than 5MHz, from the demodulation perspective, to simplify the test setup, the existing Ds (Ds=700m) and Dmin (Dmin=100m) in Rel-16 FR1 NR HST is used as a starting point to derive the Doppler shift trajectory.

Regard MCS, both MCS 2 and MCS 16 were considered for PUSCH definition under HST condition in Rel-16. Similarly, to reduce the test effort, we would like to select only MCS for PUSCH with 3MHz in high-speed condition

For other parameters, the existing test parameters can be reused

Proposal 6: The following test parameters can be considered for specifying PUSCH requirement with 3MHz in high-speed condition

- Number of PRBs: 12, 15

- MCS 16

- 1T2R, 1layer

- DMRS 1+1+1

- Mapping type A

Test Parameters for PUSCH with UL timing adjustment

For less than 5MHz, due to the available PRB is reduced compared with 5MHz. Under this condition, the relevant parameters, such as RB allocation for PUSCH and SRS should be scaled similarly.

Allocated RBs for PUSCH

Proposal 7: Allocated RBs for PUSCH is proposed

- 5MHz CBW/15KHz SCS: 6 contiguously allocated RBs for each UE

- Start RB index

- Moving UE: 0

- Stationary UE: 6

Allocated RBs for SRS

Proposal 8: Allocated RBs for SRS is proposed

- 5MHz CBW/15KHz SCS: 20 contiguously allocated RBs starting from PRB index 0

Base on RAN1 spec, the SRS bandwidth configuration is proposed as

C_SRS =5, B_SRS =0, for 20RB with 15 KHz SCS

Proposal 9: SRS bandwidth configuration is proposed as

C_SRS =5, B_SRS =0, for 20RB with 15 KHz SCS

For other test parameters, the existing test parameters can be reused

Proposal 10: Reusing existing PUSCH test parameters for specifying PUSCH requirement with 3MHz under normal UE speed

Table 1:  Test parameters for PUSCH requirement with normal UE speed if introduced

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 Db |
|  | DM-RS port | [0] |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

Proposal 11: Reusing existing PUSCH test parameters for specifying PUSCH requirement with 3MHz under high-speed condition

Table 2:  Test parameters for PUSCH requirement with UE 500km/h speed if introduced

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | First DM-RS position | Pos 2 or Pos 3 (Note) |
|  | Additional DM-RS position | Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | [0] |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: Either pos2 and pos3 may be selected for conformance testing |  |  |

Proposal 12: Reusing existing PUSCH test parameters for specifying PUSCH requirement with 3MHz with UL timing adjustment

Table 3:  Test parameters for PUSCH requirement with UL timing adjustment if introduced

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD (Note1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 3 MHz CBW/15kHz SCS: 6 RB for each UE |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 6 for 5MHz CBW/15kHz SCS, |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note2) | For FDD: slot #1 in radio frames For TDD: last symbol in slot #3 in radio frames for 15kHz |
|  | SRS resource allocation | 15 kHz SCS: CSRS =5, BSRS =0, for 20 R |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. NOTE 2: The transmission of SRS is optional. And the transmission comb and SRS periodic are configured as KTC = 2, and TSRS = 10 respectively. |  |  |

## 3.2	Test setup of PUCCH requirements

In Rel-15, the PUCCH requirements with all the formats were considered for different CBWs. If PUCCH requirement with 3MHz introduced, the same PUCCH formats can be considered to specify the related PUCCH requirement to ensure the proper test coverage. Although the number of payload size is not related with channel bandwidth directly, the performance will be different for different channel bandwidths due to the frequency hopping operation. In case of frequency hopping disable, there is no performance difference compared with 3MHz and 5MHz. With frequency hopping enable, the PRB location will be different based on the frequency hopping pattern due to the maximum number of PRB in 3MHz and 5MHz.  Therefore, the existing PUCCH test parameters with frequency hopping enabled can be reused to specify the PUCCH requirement with 3MHz.

Proposal 13: Reusing the existing PUCCH test parameters for specifying PUCCH requirement with 3MHz

Table 4:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 for 2 symbols |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 5:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | NACK to ACK probability ACK missed detection probability |

Table 6:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 7:  Test parameters of PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Test metric | BLER |  |

Table 8:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER |

- Simulation results

## 4.1	Simulation Results for PUSCH with normal condition

For this part, the initial simulation results for PUSCH based on the agreed simulation assumption in the last meeting as following table

Table 9:  Simulation assumption for PUSCH with normal condition, DMRS with 1+1

| Test case | Waveform | SCS&BW | RB | MCS | Channel | Mapping type | Length of symbol | Number of Tx and Rx | Number of DMRS | SNR@1% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | CP-OFDM | 15KHz, 3MHz | 12 | 2 | TDLB100-400 | A | 14 | 1T2R | 1+1 (2,11) | -2.8 |
| Case 2 | CP-OFDM | 15KHz 3MHz | 15 | 2 | TDLB100-400 | A | 14 | 1T2R | 1+1 (2,11) | -3.0 |
| Case 3 | CP-OFDM | 15KHz, 5MHz | 25 | 2 | TDLB100-400 | A | 14 | 1T2R | 1+1 (2,11) | -3.2 |
| Case 4 | CP-OFDM | 15KHz, 3MHz | 12 | 16 | TDLC300-100 | A | 14 | 1T2R | 1+1 (2,11) | 8.5 |
| Case 5 | CP-OFDM | 15KHz 3MHz | 15 | 16 | TDLC300-100 | A | 14 | 1T2R | 1+1 (2,11) | 8.4 |
| Case 6 | CP-OFDM | 15KHz, 5MHz | 25 | 16 | TDLC300-100 | A | 14 | 1T2R | 1+1 (2,11) | 8.7 |
| Case 7 | CP-OFDM | 15KHz, 3MHz | 12 | 20 | TDLA30-10 | A | 14 | 1T2R | 1+1 (2,11) | 11.1 |
| Case 8 | CP-OFDM | 15KHz 3MHz | 15 | 20 | TDLA30-10 | A | 14 | 1T2R | 1+1 (2,11) | 11.1 |
| Case 9 | CP-OFDM | 15KHz, 5MHz | 25 | 20 | TDLA30-10 | A | 14 | 1T2R | 1+1 (2,11) | 11.1 |

Table 10:  Simulation assumption for PUSCH with normal condition, DMRS with 1+0

| Test case | Waveform | SCS&BW | RB | MCS | Channel | Mapping type | Length of symbol | Number of Tx and Rx | Number of DMRS | SNR@1% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | CP-OFDM | 15KHz, 3MHz | 12 | 2 | TDLB100-400 | A | 14 | 1T2R | 1+0 (2) | 2.3 |
| Case 2 | CP-OFDM | 15KHz 3MHz | 15 | 2 | TDLB100-400 | A | 14 | 1T2R | 1+0 (2) | 2.6 |
| Case 3 | CP-OFDM | 15KHz, 5MHz | 25 | 2 | TDLB100-400 | A | 14 | 1T2R | 1+0 (2) | 2.1 |
| Case 4 | CP-OFDM | 15KHz, 3MHz | 12 | 16 | TDLC300-100 | A | 14 | 1T2R | 1+0 (2) | 9.6 |
| Case 5 | CP-OFDM | 15KHz 3MHz | 15 | 16 | TDLC300-100 | A | 14 | 1T2R | 1+0 (2) | 9.8 |
| Case 6 | CP-OFDM | 15KHz, 5MHz | 25 | 16 | TDLC300-100 | A | 14 | 1T2R | 1+0 (2) | 11.4 |
| Case 7 | CP-OFDM | 15KHz, 3MHz | 12 | 20 | TDLA30-10 | A | 14 | 1T2R | 1+0 (2) | 11.0 |
| Case 8 | CP-OFDM | 15KHz 3MHz | 15 | 20 | TDLA30-10 | A | 14 | 1T2R | 1+0 (2) | 11.2 |
| Case 9 | CP-OFDM | 15KHz, 5MHz | 25 | 20 | TDLA30-10 | A | 14 | 1T2R | 1+0 (2) | 11.2 |

Observation 4: Compared with 5MHz CWB, the similar performance can be achieved for 3MHz with both 12 RB and 15RB.

## 4.2	Simulation Results for PUSCH with HST condition

For this part, the initial simulation results for PUSCH under high-speed condition based on existing high speed train condition (Clause G.3 in TS 38.104) at 500km/h as a starting point for evaluation.

Table 11:  Simulation assumption for PUSCH with normal condition, DMRS with 1+0

| Test case | SCS&BW | RB | MCS | Doppler | Channel | Mapping type | Length of symbol | Number of Tx and Rx | Number of DMRS | SNR@1% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 15KHz, 3MHz | 12 | 2 | 815 | [HST Scenario 1-NR500] | A | 14 | 1T2R | 1+1+1 (2, 7,11) | -4.5 |
| Case 2 | 15KHz 3MHz | 15 | 2 | 815 | [HST Scenario 1-NR500] | A | 14 | 1T2R | 1+1+1 (2, 7,11) | -4.6 |
| Case 3 | 15KHz, 5MHz | 25 | 2 | 815 | [HST Scenario 1-NR500] | A | 14 | 1T2R | 1+1+1 (2, 7,11) | -4.7 |
| Case 4 | 15KHz, 3MHz | 12 | 16 | 815 | [HST Scenario 1-NR500] | A | 14 | 1T2R | 1+1+1 (2, 7,11) | 5.8 |
| Case 5 | 15KHz, 3MHz | 15 | 16 | 815 | [HST Scenario 1-NR500] | A | 14 | 1T2R | 1+1+1 (2, 7,11) | 7.0 |
| Case 6 | 15KHz, 3MHz | 25 | 16 | 815 | [HST Scenario 1-NR500] | A | 14 | 1T2R | 1+1+1 (2, 7,11) | 7.0 |

## 4.3	Simulation Results for PUCCH

For this part, the initial simulation results for PUCCH format 2 based on the agreed simulation assumption in the last meeting as following table

Table 12:  Simulation assumption for PUCCH format 2

| Parameters | Value |
| --- | --- |
| Modulation order | QPSK |
| First RB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs -1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| Frist symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 13: Initial simulation results for PUCCH format 2

| Test case | Format | SCS&BW | Maximum RB | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | Number of symbols | BLER@1% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | Format 2 | 15KHz, 3MHz | 12 | Enabled | 22 | TDLC300-100 | 9 | 1T2R | 2 | 0.23 |
| Case 2 | Format 2 | 15KHz 3MHz | 15 | Enabled | 22 | TDLC300-100 | 9 | 1T2R | 2 | -0.38 |
| Case 3 | Format 2 | 15KHz, 5MHz | 25 | Enabled | 22 | TDLC300-100 | 9 | 1T2R | 2 | -1.77 |

Observation 5: With limited RB, the frequency diversity gain with frequency hopping is reduced compared with larger number of RB.

- Conclusion

In this paper, the view on remaining issue of test scope and test setup of BS demodulation requirement to support less than 5MHz is provided. Meanwhile, the initial simulation results are provided for alignment purpose.

Proposal 1: Only limited set of PUSCH requirements for less than 5MHz can be introduced

Proposal 2: UL timing adjustment and PUSCH requirements with related with UE speed 500km/h could be introduced.

Proposal 3: The same PUCCH formats can be considered to specify the related PUCCH requirement with 3MHz

Observation 1: Existing long PRACH formats with 1.25KHz SCS with sequence length LRA=839, and long PRACH formats with 15KHz SCS with sequence length LRA=139 can support 3MHz channel bandwidth.

Observation 2: Existing short PRACH formats with 15KHz SCS with sequence length LRA=1151, cannot apply for 3MHz channel bandwidth, considering it is targeting for 20MHz channel bandwidth.

Observation 3: A note is added to indicate PRACH formats and configurations are not fitting into transmission BW are not applicable in RAN1 agreement

Proposal 4: Adding an applicability rule for PRACH requirement targeting BS supporting less than 5MHz carrier bandwidth

- For BS supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply

Proposal 5: The following test parameters can be considered for down selection for specifying PUSCH requirement with 3MHz

- Number of PRBs: 12, 15

- MCS 16

- 1T2R, 1layer

- TDLC300-100

- DMRS 1+1

- Mapping type A

Proposal 6: The following test parameters can be considered for specifying PUSCH requirement with 3MHz in high-speed condition

- Number of PRBs: 12, 15

- MCS 16

- 1T2R, 1layer

- DMRS 1+1+1

- Mapping type A

Proposal 7: Allocated RBs for PUSCH is proposed

- 5MHz CBW/15KHz SCS: 6 contiguously allocated RBs for each UE

- Start RB index

- Moving UE: 0

- Stationary UE: 6

Proposal 8: Allocated RBs for SRS is proposed

- 5MHz CBW/15KHz SCS: 20 contiguously allocated RBs starting from PRB index 0

Proposal 9: SRS bandwidth configuration is proposed as

C_SRS =5, B_SRS =0, for 20RB with 15 KHz SCS

Proposal 10: Reusing existing PUSCH test parameters for specifying PUSCH requirement with 3MHz under normal UE speed

Table 1:  Test parameters for PUSCH requirement with normal UE speed if introduced

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 Db |
|  | DM-RS port | [0] |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

Proposal 11: Reusing existing PUSCH test parameters for specifying PUSCH requirement with 3MHz under high-speed condition

Table 2:  Test parameters for PUSCH requirement with UE 500km/h speed if introduced

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | First DM-RS position | Pos 2 or Pos 3 (Note) |
|  | Additional DM-RS position | Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | [0] |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: Either pos2 and pos3 may be selected for conformance testing |  |  |

Proposal 12: Reusing existing PUSCH test parameters for specifying PUSCH requirement with 3MHz with UL timing adjustment

Table 3:  Test parameters for PUSCH requirement with UL timing adjustment if introduced

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD (Note1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 3 MHz CBW/15kHz SCS: 6 RB for each UE |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 6 for 5MHz CBW/15kHz SCS, |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note2) | For FDD: slot #1 in radio frames For TDD: last symbol in slot #3 in radio frames for 15kHz |
|  | SRS resource allocation | 15 kHz SCS: CSRS =5, BSRS =0, for 20 R |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. NOTE 2: The transmission of SRS is optional. And the transmission comb and SRS periodic are configured as KTC = 2, and TSRS = 10 respectively. |  |  |

Proposal 13: Reusing the existing PUCCH test parameters for specifying PUCCH requirement with 3MHz

Table 4:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 for 2 symbols |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 5:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | NACK to ACK probability ACK missed detection probability |

Table 6:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 7:  Test parameters of PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Test metric | BLER |  |

Table 8:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER |

Observation 4: Compared with 5MHz CWB, the similar performance can be achieved for 3MHz with both 12 RB and 15RB.

Observation 5: With limited RB, the frequency diversity gain with frequency hopping is reduced compared with larger number of RB.

# References

R4-2321064, WF on NR_FR1_lessthan_5MHz_BW_demod