3GPP TSG-RAN4 Meeting #110	R4-2401592

# Athens, Greece, 26th February – 1st March

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_solutions-Core) CR on condtional handover delay for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2024-1-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - TEvent_DU + Tidentify_intra_with_index is a delay unit, which shall not to be compared with an condition of absolute time: T1 or D1. - Align formula description in context - Update the name of condition to match the definition in TS38.331 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update TEvent_DU to an absolute time unit: the time at the end of TEvent_DU - Update formula to : TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index - Update the name of condition to match the definition in TS38.331 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Wrong statement. - Editorial error. - Not match TS38.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1C.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

##### 6.1C.2.2.2	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For intra-frequency handover, the requirements for identifying a new detectable intra frequency cell measured without Time To Trigger (TTT) and L3 filtering, Tidentify_intra_with_index or Tidentify_intra_without_index, defined in clause 9.2C.5.1 and clause 9.2C.6.1 are used.

For time-based conditional intra-frequency handover:

-	If condition ﻿t1-Threshold [2] occurs earlier than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equal to Tidentify_intra_with_index or Tidentify_intra_without_index assuming UE only performs the measurements within SMTC window of the target cell.

-	If condition t1-Threshold occurs later than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until condition t1-Threshold.

For location-based conditional intra-frequency handover:

-	If both condition ﻿referenceLocation1 [2] and ﻿referenceLocation2 [2] are fulfilled earlier than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equal to Tidentify_intra_with_index or Tidentify_intra_without_index assuming UE only performs measurements within SMTC window of the target cell.

-	If both condition referenceLocation1 and referenceLocation2 are fulfilled is later than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index for intra-frequency handover, then the measurement time delay equal to the time from the end of Tevent_DU until time when both condition referenceLocation1 and condition referenceLocation2 are fulfilled.

For inter-frequency handover, the requirements for identifying a new detectable inter frequency cell measured without Time To Trigger (TTT) and L3 filtering, Tidentify_inter_with_index or Tidentify_inter_without_index, defined in clause 9.3C.7.1 are used.

For time-based conditional inter-frequency handover:

-	If condition t1-Threshold occurs earlier than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equal to Tidentify_inter_with_index or Tidentify_inter_without_index assuming that the UE uses only the SMTC window of the target inter-frequency carrier for performing the measurements. In this case Ksatellite=1, CSSFinter=1.

-	If condition t1-Threshold occurs later than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until condition t1-Threshold.

For location-based conditional inter-frequency handover,

-	If both condition referenceLocation1 and condition referenceLocation2 are fulfilled earlier than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equal to Tidentify_inter_with_index or Tidentify_inter_without_index, assuming that the UE uses only the SMTC window of the target inter-frequency carrier for performing the measurements. In this case Ksatellite=1, CSSFinter=1.

-	If both condition referenceLocation1 and condition referenceLocation2 are fulfilled later than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equal to the time from the end of Tevent_DU until time of both condition referenceLocation1 and condition referenceLocation2 are fulfilled.

When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period [Tidentify_intra_without_index] or [Tidentify_intra_with_index] for intra-frequency handover or [Tidentify_inter_without_index] for inter-frequency handover.

<< End of changes >>