3GPP TSG-RAN WG4 Meeting # 110	R4-2401665

# GR, Athens, Feb 26th- March1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0489 | rev |  | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_ext_to_71GHz-Perf) CR on 38.101-4 Correction on test paramters for FR2-2 PDSCH test with 480kHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2024-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | On RAN4 109 meeting, RAN4 agrees to transmit OCNG in first symbol for slot without PDCCH for 480kHz SCS to ensure the constant power in one slot and follow the tradition of RAN4 test setup. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the configuration” Symbols for all unused REs” to be aligned with the agreement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configuration “ Symbols for all unused REs ” is still incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7	Demodulation performance requirements (Radiated requirements)

<Unchanged skipped>

7.2	PDSCH demodulation requirements

The parameters specified in Table 7.2-1 are valid for all PDSCH demodulation tests unless otherwise stated.

Table 7.2-1: Common Test Parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| PTRS epre-Ratio |  |  |  | 0 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 60 or 120 or 480 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-2 [7] for tested channel bandwidth and subcarrier spacing |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot for 120 KHz SCS (Xs, Ys) = (4, 1) for 480 KHz SCS |
|  | Symbols with PDCCH |  |  | 0 |
|  | Number of PRBs in CORESET |  |  | Table 7.2-2 for tested channel bandwidth and subcarrier spacing |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL8 |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI state |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |
| Cross carrier scheduling |  |  |  | Not configured |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 6 for CSI-RS resource 1 and 3 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 for CSI-RS resource 1,2,3,4 120 kHz SCS: 160 for CSI-RS resource 1,2,3,4 480 kHz SCS: 640 for CSI-RS resource 1, 2, 3, 4 |
|  | CSI-RS offset |  | Slots | 60 kHz SCS: 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 120 kHz SCS: 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 480 kHz SCS: 320 for CSI-RS resource 1 and 2 321 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  |  | 3 for 2 CSI-RS ports and 5 for 4 CSI-RS ports |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 12 |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 120 kHz SCS: 160 480 kHz SCS: 640 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4) *4 |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  |  | 5 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 120 kHz SCS: 160 480 kHz SCS: 640 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4) *4 |
| CSI-RS for beam refinement | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 for CSI-RS resource 1,2 120 kHz SCS: 160 for CSI-RS resource 1,2 480 kHz SCS: 640 for CSI-RS resource 1,2 |
|  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | Repetition |  |  | ON |
|  | QCL info |  |  | TCI state #1 |
| PDSCH DMRS configuration | Antenna ports indexes |  |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests |
|  | Position of the first DMRS for PDSCH mapping type A |  |  | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type D |
| PTRS configuration | Frequency density (KPT-RS) |  |  | 2 |
|  | Time density (LPT-RS) |  |  | 1 |
|  | Resource Element Offset |  |  | 2 |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 120 KHz SCS: 4 480 KHz SCS: 16 |
| PUCCH HARQ ACK spaitial bundling |  |  |  | Not configured |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection with Wideband size and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 for FR2-1 tests OP.1 TDD as defined in Annex A.5.2.1 for FR2-1 tests For FR2-2 with 480kHz, transmit OCNG symbols as defined in Annex 5.2.1 on the unused REs of first symbol of each slot without PDCCH, for other unused REs, no OCNG symbols are transmitted. For FR2-2 with 120kHz, no OCNG symbols on unused REs. |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-2 [7] for tested channel bandwidth and subcarrier spacing. Note 3: Refer to Table 7.4.1.5.3-1 in [9] |  |  |  |  |