3GPP TSG-RAN WG4 Meeting # 110	R4-2401669

# Athens, Greece, Feb 26th-March 1st, 2024

Title: 	Discussions on UE 8Rx performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.1.3.1.2

Document for:	Discussion

Background

In RAN4#109 meeting, a WF for UE 8Rx performance requirements was approved [1]. This contribution provides our simulation results for CA.

- Simulation results

We provide our simulation results as follows:

Table 2-1: Simulation results for 8Rx PDSCH CA with Rank2 (FDD 15kHz SCS)

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.3 |
| 10 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.5 |
| 15 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.8 |
| 20 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 11.2 |
| 25 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.9 |
| 30 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 11.1 |
| 35 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.9 |
| 40 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 11.0 |
| 45 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 11.2 |
| 50 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 11.2 |

Table 2-2: Simulation results for 8Rx PDSCH CA with Rank2 (TDD 30kHz SCS)

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.0 |
| 10 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.3 |
| 15 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.3 |
| 20 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.3 |
| 25 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.4 |
| 30 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.3 |
| 40 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.7 |
| 50 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.5 |
| 60 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.4 |
| 80 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 11.1 |
| 90 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 10.8 |
| 100 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | 70 | 11.0 |

Table 2-3: Simulation results for 8Rx PDSCH CA with Rank8 (FDD 15kHz SCS)

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.1 |
| 10 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.3 |
| 15 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.6 |
| 20 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.1 |
| 25 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.1 |
| 30 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.3 |
| 35 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.3 |
| 40 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.7 |
| 45 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.7 |
| 50 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.4 |

Table 2-4: Simulation results for 8Rx PDSCH CA with Rank8 (TDD 30kHz SCS)

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.4 |
| 10 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.3 |
| 15 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.7 |
| 20 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.7 |
| 25 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.7 |
| 30 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.3 |
| 40 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.9 |
| 50 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.8 |
| 60 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.2 |
| 80 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.6 |
| 90 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.8 |
| 100 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 20.1 |

- Conclusion

This contribution provides our simulation results for 8Rx PDSCH CA/

- Reference

[1]   R4-2321199 WF for 8Rx UE performance requirements, Huawei, HiSilicon, MediaTek