3GPP TSG-RAN WG4 Meeting # 110	R4-2401674

# Athens, GR, 26 Feb – 01 Mar, 2024

Title: 	Test parameters and simulation results for advanced receiver for MU-MIMO

Source: 	Huawei, HiSilicon

Agenda item:	8.13.2.2

Document for:	Discussion

Background

In last meeting, a WF [1] on the receiver for MU-MIMO scenario was approved. This contribution provides our views related to test parameter part and simulation results.

- Discussions

The simulation assumptions are captured in Table 2-1:

Table 2-1: Simulation assumptions

| Parameter |  | Unit | Target UE | Co-scheduled UE |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD, TDD |  |
| SCS |  | kHz | FDD:15 TDD:30 |  |
| Bandwidth |  | MHz | FDD:10 TDD:40 |  |
| TDD pattern |  |  | 7D1S2U, S=6D4G4U |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | Rank1+1: 1000 Rank2+2:1000, 1001 | Rank1+1:1001 Rank 2+2:1000,1001 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | Rank1+1: 1 Rank2+2: 2 | Rank1+1: 1 Rank2+2: 2 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. | Random PMI selection: Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12].Any column of precoder matrix is not equal to any column of precoder matrix of Target UE Orthogonal PMI selection: Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH |
| MU-MIMO Beamforming Model |  |  | As specified in B.4.2 in 38.101-4 |  |
| Receiver |  |  | R-ML with modulation order detection R-ML without modulation order detection MMSE-IRC |  |
| Modulation order estimation assumption |  |  | Per PRG detection with range from QPSK to 256QAM |  |
| Number of HARQ Processes |  |  | FDD:4 TDD:8 | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 | N/A |
| Note 1: The DMRS scrambling ID is same for both target UE and Co-scheduled UE. |  |  |  |  |

We provide our simulation results for agreed cases in Table 2-2 to Table 2-5

Table 2-2: Summary of simulation results without modulation order detection for FDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR@70% maxTP(dB) | MMSE-IRC SNR@70% maxTP(dB) | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 13.7 | 22.1 | 8.4 |
|  |  |  |  | ULA Low |  |  | 10.8 | 15.0 | 4.2 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 5.9 | 7.4 | 1.5 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 12.9 | 25.0 | 12.1 |
|  | 2T2R | Orthogonal | TDLC300-100 | ULA medium |  |  | 13.0 | 19.4 | 6.4 |
|  | 2T4R |  |  |  |  |  | 12.1 | 20.5 | 8.4 |
| 2+2 | 4T4R | Orthogonal | TDLA30-10 | ULA Low |  |  | 9.5 | 12.1 | 2.6 |
|  |  |  |  | XP medium |  |  | 10.5 | 13.9 | 3.4 |
|  |  |  |  | ULA Low | 17 | 16QAM | 15.1 | 16.4 | 1.3 |
|  |  |  |  | XP medium |  |  | 16.8 | 18.4 | 1.6 |

Table 2-3: Summary of simulation results without modulation order detection for TDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR@70% maxTP(dB) | MMSE-IRC SNR@70% maxTP(dB) | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 14.3 | 23.7 | 9.4 |
|  |  |  |  | ULA Low |  |  | 11.1 | 16.3 | 5.2 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 6.1 | 7.6 | 1.5 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 13.6 | 27.7 | 14.1 |
|  | 2T2R | Orthogonal | TDLC300-100 | ULA medium |  |  | 13.4 | 20.4 | 7.0 |
|  | 2T4R |  |  |  |  |  | 12.5 | 22.3 | 9.8 |
| 2+2 | 4T4R | Orthogonal | TDLA30-10 | ULA Low |  |  | 9.6 | 12.3 | 2.7 |
|  |  |  |  | XP medium |  |  | 10.6 | 14.1 | 3.5 |
|  |  |  |  | ULA Low | 17 | 16QAM | 15.4 | 16.6 | 1.2 |
|  |  |  |  | XP medium |  |  | 17.0 | 18.7 | 1.7 |

Table 2-4: Summary of simulation results for case with modulation order estimation for FDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR(dB)@70% max TP | MMSE-IRC SNR(dB)@70% max TP | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 14.5 | 22.1 | 7.6 |
|  |  |  |  | ULA Low |  |  | 11.2 | 15.0 | 3.8 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 6.0 | 7.4 | 1.4 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 14.6 | 25.0 | 10.4 |
|  | 2T2R |  | TDLC300-100 | ULA medium | 17 | 16QAM | 22.7 | N/A | N/A |
|  |  |  |  | ULA Low |  |  | 17.7 | 24.3 | 6.6 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 10.8 | 10.8 | 0.0 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 22.7 | N/A | N/A |
|  | 2T2R | Orthogonal | TDLC300-100 | ULA medium | 13 | QPSK | 13.6 | 19.4 | 5.8 |
|  | 2T4R |  |  |  |  |  | 13.5 | 20.5 | 7.0 |
|  | 2T2R |  |  |  | 17 | 16QAM | 20.9 | 26.1 | 5.2 |
|  | 2T4R |  |  |  |  |  | 20.9 | 27.9 | 7.0 |
| 2+2 | 4T4R |  | TDLA30-10 | ULA Low | 13 | QPSK | 10.6 | 12.1 | 1.5 |
|  |  |  |  | XP medium |  |  | 10.7 | 13.9 | 3.2 |
|  |  |  |  | ULA Low | 17 | 16QAM | 15.9 | 16.4 | 0.5 |
|  |  |  |  | XP medium |  |  | 17.0 | 18.4 | 1.4 |

Table 2-5: Summary of simulation results for case with modulation order estimation for TDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR(dB)@70% max TP | MMSE-IRC SNR(dB)@70% max TP | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 15.2 | 23.7 | 8.5 |
|  |  |  |  | ULA Low |  |  | 11.5 | 16.3 | 4.8 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 6.1 | 7.6 | 1.5 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 15.9 | 27.7 | 11.8 |
|  | 2T2R |  | TDLC300-100 | ULA medium | 17 | 16QAM | 24.4 | N/A | N/A |
|  |  |  |  | ULA Low |  |  | 18.4 | N/A | N/A |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 11.1 | 11.0 | -0.1 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 25.4 | N/A | N/A |
|  | 2T2R | Orthogonal | TDLC300-100 | ULA medium | 13 | QPSK | 14.1 | 20.4 | 6.3 |
|  | 2T4R |  |  |  |  |  | 14.3 | 22.3 | 8.0 |
|  | 2T2R |  |  |  | 17 | 16QAM | 22.1 | 28.8 | 6.7 |
|  | 2T4R |  |  |  |  |  | 22.5 | 32.4 | 9.9 |
| 2+2 | 4T4R |  | TDLA30-10 | ULA Low | 13 | QPSK | 10.6 | 12.3 | 1.7 |
|  |  |  |  | XP medium |  |  | 10.6 | 14.1 | 3.5 |
|  |  |  |  | ULA Low | 17 | 16QAM | 15.8 | 16.6 | 0.8 |
|  |  |  |  | XP medium |  |  | 17.0 | 18.7 | 1.7 |

Based on the simulation results, we have following observations:

Observation 1: For cases without modulation order detection.

For Rank 1+1:

Cases with TDLA30-10, ULA low have limited gain.

For 16QAM+QPSK: The performance gain is significant except case with TDLA30-10 and ULA Low

For Rank 2+2:

For 16QAM+QPSK, FDD: the performance gain is 2.6dB for ULA low and 3.4dB for XP medium.

For 16QAM+QPSK, TDD: the performance gain is 2.7dB for ULA low and 3.5dB for XP medium.

For 64QAM+16QAM, FDD: the performance gain is 1.3dB for ULA low and 1.6dB XP medium.

For 64QAM+16QAM, TDD: the performance gain is 1.2dB for ULA low and 1.7dB for XP medium.

Cases with ULA Low have lower performance gain than that with XP medium

Cases with 64QAM+16QAM have lower performance gain than that with 16QAM+QPSK.

Observation 2: For cases with modulation order detection:

For Rank 1+1:

Cases with TDLA30-10, ULA low have limited gain.

For 16QAM+QPSK: The performance gain is significant for all cases except cases with TDLA30-10, ULA low.

For 64QAM+16QAM: The performance gain is significant for all cases except case with TDLA30-10, ULA Low.

For Rank 2+2:

For 16QAM+QPSK, FDD: the performance gain is 1.5dB for ULA low and 3.2dB for XP medium.

For 16QAM+QPSK, TDD: the performance gain is 1.7dB for ULA low and 3.5dB for XP medium.

For 64QAM+16QAM, FDD: the performance gain is 0.5dB for ULA low and 1.4dB for XP medium.

For 64QAM+16QAM, TDD: the performance gain is 0.8dB for ULA low and 1.7dB for XP medium.

Cases with ULA Low have lower performance gain than that with XP medium

Cases with 64QAM+16QAM have lower performance gain than that with 16QAM+QPSK.

For test with modulation order detection, we propose to only consider Rank 1+1. The reason is that there are too many diverse modulation order detection algorithms, which means it’s very challengeable to unify the modulation order detection algorithm and align the simulation results. Taking Rank 2+2 as an example, at least two algorithms are likely to be implemented:

- UE performs MO detection per interference layer with whitening the remaining layers in advance.

- UE performs joint maximum likehood detection for all interference layers.

Obviously b) has better performance than a) but also has higher complexity. Based on that, companies with different implementations may have different results,  considering there is limited time left for this WI, we propose to only introduce Rank 1+1 which is the simplest MU-MIMO case with 1 interference layer, bringing the benefits that different algorithms are expected to have same performance and it's much easier to align the simulation results.

Observation 3:  Defining case with Rank 1+1 can minimize the performance impact of diverse implementation of MO detection, which makes it easier to align the simulation results.

To guarantee the performance gain, we propose following test setups:

Proposal 1: RAN4 to consider following test setup for cases without modulation order detection.

For Rank 1+1: TDLC300-100, Orthogonal PMI selection, 2T2R/2T4R ULA medium, MCS17(Target UE) +16QAM(Co-UE)

For Rank 2+2: TDLA30-10, Orthogonal PMI selection, 4T4R, XPL medium, MCS13(Target UE) +QPSK(Co-UE)

Proposal 2: RAN4 to consider following test setup for cases with modulation order detection.

For Rank 1+1: TDLC300-100, Orthogonal PMI selection, 2T2R/2T4R ULA medium, MCS17(Target UE) +16QAM(Co-UE)

- Additionally, we propose following test setup:

- For cases without modulation order detection: DCI 1~5 is indicated, all RRC signalling are configured.

- For cases with modulation order detection: DCI 6 is indicated, all RRC signalling are configured. MCS table/ maximum modulation order information indicates the maximum modulation order UE to detect is 8 (256QAM)

Proposal 3: Consider following test setup:

For cases without modulation order detection: DCI 1~5 is indicated, all RRC signaling are configured.

For cases with modulation order detection: DCI 6 is indicated, all RRC signaling are configured. MCS table/ maximum modulation order information indicates the maximum modulation order UE to detect is 8 (256QAM)

- Conclusion

This contribution provides our views on test parameters. The proposals and observations are:

Observation 1: For cases without modulation order detection.

For Rank 1+1:

Cases with TDLA30-10, ULA low have limited gain.

For 16QAM+QPSK: The performance gain is significant except case with TDLA30-10 and ULA Low

For Rank 2+2:

For 16QAM+QPSK, FDD: the performance gain is 2.6dB for ULA low and 3.4dB for XP medium.

For 16QAM+QPSK, TDD: the performance gain is 2.7dB for ULA low and 3.5dB for XP medium.

For 64QAM+16QAM, FDD: the performance gain is 1.3dB for ULA low and 1.6dB XP medium.

For 64QAM+16QAM, TDD: the performance gain is 1.2dB for ULA low and 1.7dB for XP medium.

Cases with ULA Low have lower performance gain than that with XP medium

Cases with 64QAM+16QAM have lower performance gain than that with 16QAM+QPSK.

Observation 2: For cases with modulation order detection:

For Rank 1+1:

Cases with TDLA30-10, ULA low have limited gain.

For 16QAM+QPSK: The performance gain is significant for all cases except cases with TDLA30-10, ULA low.

For 64QAM+16QAM: The performance gain is significant for all cases except case with TDLA30-10, ULA Low.

For Rank 2+2:

For 16QAM+QPSK, FDD: the performance gain is 1.5dB for ULA low and 3.2dB for XP medium.

For 16QAM+QPSK, TDD: the performance gain is 1.7dB for ULA low and 3.5dB for XP medium.

For 64QAM+16QAM, FDD: the performance gain is 0.5dB for ULA low and 1.4dB for XP medium.

For 64QAM+16QAM, TDD: the performance gain is 0.8dB for ULA low and 1.7dB for XP medium.

Cases with ULA Low have lower performance gain than that with XP medium

Cases with 64QAM+16QAM have lower performance gain than that with 16QAM+QPSK.

Observation 3:  Defining case with Rank 1+1 can minimize the performance impact of diverse implementation of MO detection, which makes it easier to align the simulation results.

Proposal 1: RAN4 to consider following test setup for cases without modulation order detection.

For Rank 1+1: TDLC300-100, Orthogonal PMI selection, 2T2R/2T4R ULA medium, MCS17(Target UE) +16QAM(Co-UE)

For Rank 2+2: TDLA30-10, Orthogonal PMI selection, 4T4R, XPL medium, MCS13(Target UE) +QPSK(Co-UE)

Proposal 2: RAN4 to consider following test setup for cases with modulation order detection.

For Rank 1+1: TDLC300-100, Orthogonal PMI selection, 2T2R/2T4R ULA medium, MCS17(Target UE) +16QAM(Co-UE)

Proposal 3: Consider following additional test setup:

For cases without modulation order detection: DCI 1~5 is indicated, all RRC signaling are configured.

For cases with modulation order detection: DCI 6 is indicated, all RRC signaling are configured. MCS table/ maximum modulation order information indicates the maximum modulation order UE to detect is 8 (256QAM)

- Reference

[1]    R4-2321114 WF on the advanced receiver for MU-MIMO scenario. China Telcom, Apple