3GPP TSG-RAN WG4 Meeting # 110	R4-2401678

# Athens, GR, 26 Feb – 01 Mar, 2024

Title: 	Discussions on BS requirements for Rel-18 eRedCap

Source: 	Huawei, HiSilicon

Agenda item:	8.23.4.2

Document for:	Discussion

Background

A new WID[1] “Enhanced support of reduced capability NR devices” was approved in last RAN plenary meeting and RAN4 should start discussing the BS performance requirements. In this contribution, we provide our general views on test scope.

- Discussions

- The main requirement of this WID is to reduce the target date rate of R18 eRedCap to 10Mbps, with this target, two RedCap UE types are introduced by RAN1: RedCap UE with reduced peak date rate and reduced baseband bandwidth in FR1 and RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1. For the former one, maximum 5MHz BB (PUSCH) bandwidth with maximum 20MHz RF bandwidth is supported and vLayers·Qm·f is relaxed to 3.2. For the later one, BB bandwidth is not limited and vLayers·Qm·f is relaxed to 0.75 when vLayers = 1 and vLayers·Qm·f  is relaxed to 0.8 when vLayers = 2.

- The only difference from BB side is limitation of actual PUSCH PRBs, which is within 5MHz bandwidth, RAN4 has defined PUSCH demodulation requirements for 5MHz/15kHz SCS, i.e. 25 RBs; At the same time, there is no BS that only supports 5MHz baseband bandwidth, so we don’t see any motivation to define new PUSCH requirements for 5MHz BB bandwidth.

Proposal 1: Don’t define BS requirements for R18 eRedCap.

- Conclusion

- In this contribution we provide our general views on BS requirements for Rel-18 eRedCap. The proposals are:

Proposal 1: Don’t define BS requirements for R18 eRedCap.

- Reference

[1]    RP-233637 Revised WID on Enhanced support of reduced capability NR devices