3GPP TSG-RAN WG4 Meeting #110	R4-2401738

# Athens, Greece, Feb 26 – Mar 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on PSCell addition and release delay of unknown FR1 PSCell for FR1+FR1 NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh3-Perf |  |  |  |  |  | Date: |  |  | 2024-2-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce new test case of PSCell addition and release delay of unknown FR1 PSCell for FR1+FR1 NR-DC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new test case of PSCell addition and release delay of unknown FR1 PSCell for FR1+FR1 NR-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance part for FR1+FR1 NR-DC is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change 1>

### A.6.5.x	PSCell addition and release delay

#### A.6.5.x.1	Addition and Release Delay of unknown NR FR1 PSCell

##### A.6.5.x.1.1	Test purpose and environment

The purpose of this test is to verify that the NR PSCell addition and release delay requirements under NR-DC defined in clauses 8.9.2 and 8.9.3 respectively, for the case where the PSCell is unknown to the UE at the time of addition.

The supported test configurations are shown in A.6.5.x.1.1-1. The test scenario comprises two NR cells, Cell 1 and Cell 2, on radio channel 1 and radio channel 2 in FR1, respectively. Test parameters are given in Tables A.6.5.x.1.1-2 and A.6.5.x.1.1-3 below. The test consists of six successive time periods with duration of T1, T2, T3 and T4 respectively. Cell 1 is the NR PCell, Cell 2 is an NR neighbour cell. The Cell 1 once set up is not changed across time.

At the start of T1, the UE shall be connected to Cell 1 (PCell) on radio channel 1 (PCC) and shall only monitor PCC and hence be unaware of Cell 2 (PSCell-to-be) on radio channel 2. the UE does not have any timing information of Cell 2. At the end of T1, the test system shall send a RRC message instructing the UE to add PSCell (Cell 2), and further instructing the UE to report CSI periodically in the PSCell once it has been added. Reception by the UE of this RRC message defines the start of T2.

During T2, the UE shall identify PSCell (Cell 2) and carry out random access towards the PSCell (Cell 2). Reception by the test system of the PRACH preamble defines the start of T3.

During T3, the UE shall send periodic CSI reports in PSCell (Cell 2). After having received at least one such report, the test system shall send a RRC message instructing the UE to release the PSCell (Cell 2). Reception by the UE of the RRC message defines the start of T4.

During T4, the UE shall release the PSCell (Cell 2).

Table A.6.5.x.1.1-1: Supported test configurations for FR1 PSCell Addition and Release

| Configuration | Description |
| --- | --- |
| 1 | NR SCS 15 kHz, BW 10 MHz, FDD |
| 2 | NR SCS 15 kHz, BW 10 MHz, TDD |
| 3 | NR SCS 30 kHz, BW 40 MHz, TDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.x.1.1-2: General Test Parameters for FR1 PSCell Addition and Release

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two NR FR1 radio channels are used in this test. |
| Initial Condition | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell 2 | Neighbour cell on RF channel number 2. |
| Final Condition | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
|  | Neighbour Cell |  | Cell 2 | PSCell released on RF channel number 2. |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| PRACH configuration on Cell 2 |  |  | FR1 PRACH configuration 1 | Captured in A.3.8.2.1 |
| CSI reporting periodicity and offset configuration for Cell 2 |  | ms | 2 |  |
| T1 |  | s | 5 | During this time the PCell shall be known and Cell 2 shall be unknown. |
| T2 |  | s | 0.5 | During this time the UE adds the PSCell. |
| T3 |  | s | 0.5 | During this time the UE sends CSI reports for PSCell. |
| T4 |  | s | 0.5 | During this time the UE releases the PSCell. |

Table A.6.5.x.1.1-3: Cell Specific Parameters for FR1 PSCell Addition and Release

| Parameter | Unit | Config | Cell 1 | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T3 | T4 |
| Frequency Range |  | 1,2,3 | FR1 |  |  |  |  |  |
| Duplex mode |  | 1 | FDD |  |  |  |  |  |
|  |  | 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  | 1 | Not Applicable |  |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | MHz | 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | 3 | 40: NRB,c = 106 |  |  |  |  |  |
| Data RBs allocated |  | 1,2 | 52 |  |  |  |  |  |
|  |  | 3 | 106 |  |  |  |  |  |
| Initial Downlink BWP configuration |  | 1,2,3 | DLBWP.0.1 |  |  |  |  |  |
| Initial Uplink BWP configuration |  | 1,2,3 | ULBWP.0.1 |  |  |  |  |  |
| Dedicated Downlink BWP configuration |  | 1,2,3 | DLBWP.1.1 |  |  |  |  |  |
| Dedicated Uplink BWP configuration |  | 1,2,3 | ULBWP.1.1 |  |  |  |  |  |
| PDSCH Reference Measurement Channel |  | 1 | SR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET parameters |  | 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET parameters |  | 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  |  |
| OCNG PatternsNote1 |  | 1,2,3 | OP.1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2 | 15 |  |  |  |  |  |
|  |  | 3 | 30 |  |  |  |  |  |
| SSB configuration |  | 1,2 | SSB.1 FR1 |  |  |  |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |  |  |  |
| SMTC configuration |  | 1,2,3 | SMTC.1 |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  |  |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |  |  |
| CSI-RS configuration for CSI reporting |  | 1 | CSI-RS.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CSI-RS.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CSI-RS.2.1 TDD |  |  |  |  |  |
| reportConfigType |  | 1,2,3 | periodic |  |  |  |  |  |
| reportQuantity |  | 1,2,3 | cri-RI-PMI-CQI |  |  |  |  |  |
| CSI reporting periodicity | slot | 1,2 | 5 |  |  |  |  |  |
|  |  | 3 | 10 |  |  |  |  |  |
| CSI reporting offset | slot | 1,2 | 2 |  |  |  |  |  |
|  |  | 3 | 4 |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3 | -100 | N/A |  | -85 |  |  |
| Note2 | dBm/SCS | 1,2 | -100 | N/A |  | -85 |  |  |
|  |  | 3 | -97 | N/A |  | -82 |  |  |
|  |  | 1,2,3 | 12 | -infinity |  | 0 |  |  |
|  |  | 1,2,3 | 12 | -infinity |  | 0 |  |  |
| SS-RSRPNote3 | dBm/SCS | 1,2 | -88 | -infinity |  | -85 |  |  |
|  |  | 3 | -85 | -infinity |  | -82 |  |  |
| IoNote3 | dBm/9.36MHz | 1,2 | -59.78 | N/A |  | -57 |  |  |
|  | dBm/38.1MHz | 3 | -53.68 | N/A |  | -51 |  |  |
| Propagation Condition |  | 1,2,3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

##### A.6.5.x.1.2	Test Requirements

The UE shall transmit the PRACH preamble to PSCell no later than 132 msNote1 from the start of T2.

The UE shall send at least one CSI report for PSCell with non-zero CQI index during T3.

The UE shall periodically send CSI reports for PSCell after the UE has sent first CQI report with non-zero CQI index during T3.

The UE shall stop sending CSI reports for PSCell no later than 20ms from the start of T4.

All the above test requirements shall be fulfilled in order for the observed PSCell addition delay and PSCell release delay to be counted as correct. The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90%.

Note1:	The PSCell addition delay can be expressed as follows as specified in clause 8.9.2:

Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2ms

Where:

TRRC_delay = 10ms

Tprocessing = 20ms

Tsearch 	= 60ms

T∆ = 20ms

TPSCell_ DU = 1*10+10 = 20ms

# < End of change 1>