3GPP TSG-RAN WG4 Meeting #110	R4-2401749

Athens, Greece, 26 February – 1 March 2024

Agenda Item:	8.3.4.2

Source:	Ericsson

Title:	PDSCH demodulation requirements for FR2 UE multi-Rx reception

Document for:	Discussion

# 1	Introduction

RAN4#109 agreed with the way forward and endorsed the draft CRs on UE demodulation and CSI requirements for FR2 multi-Rx DL reception [1][2][3][4]. This contribution provides our PDSCH simulation results for the FR2 UE multi-Rx reception scenario and discusses the remaining open issue.

# 2	Simulation results

## 2.1	sDCI-based transmission

### 2.1.1	Simulation parameters

Table 1 and Table 2 show the simulation parameters according to [2].

Table 1	Simulation parameters for sDCI-based SDM transmission.

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  |  | TCI State #1 |  |
|  |  | CORESETPoolIndex |  |  | 0 |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 160 |  |
|  |  | CSI-RS offset |  | Slots | 80 for CSI-RS resources 1 and 2 81 for CSI-RS resources 3 and 4 | 80 for CSI-RS resources 5 and 6 81 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 |  |
| CSI-RS for beam refinement |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | k0=0 for CSI-RS resource 1,2 | k0=1 for CSI-RS resource 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 | 1 for CSI-RS resource 3,4 |
|  |  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2 | 'No CDM' for CSI-RS resource 3,4 |
|  |  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2 | 3 for CSI-RS resource 3,4 |
|  |  | CSI-RS periodicity |  |  | 160 for CSI-RS resource 1,2 | 160 for CSI-RS resource 3,4 |
|  |  | CSI-RS offset |  |  | 0 for CSI-RS resource 1,2 | 0 for CSI-RS resource 3,4 |
|  |  | Repetition |  |  | ON | ON |
|  |  | QCL info |  |  | TCI state #1 | TCI state #2 |
| Duplex mode |  |  |  |  | TDD |  |
| Active DL BWP index |  |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |
|  | k0 |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  | 1 |  |
|  | Length (L) |  |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  |  | Static |  |
|  | PRB bundling size |  |  |  | 2 |  |
|  | Resource allocation type |  |  |  | Type 1 |  |
|  | RBG size |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | 1000 | 1002 |
|  | TCI state |  |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type D | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type D |
| Resource allocation |  |  |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  | us | -0.0625 for test 1-1 0.25 for test 1-2 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  | Hz | 600 for test 1-1 0 for test 1-2 |  |
| Number of HARQ Processes |  |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Precoding configuration |  |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) |  |  |  |  |  |  |

Table 2	Additional simulation parameters for sDCI-based SDM transmission.

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Rho | PTRS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100 / 120 | 64QAM, 0.43 (MCS17) | FR2.120-1 | TDLA30-35 | 2x4 XP (α=β=0, γ=0.0625) | -12dB | One PTRS port across TRxP |
| 1-2 | 100 / 120 | 64QAM, 0.43 (MCS17) | FR2.120-1 | TDLA30-35 | 2x4 XP (α=β=0, γ=0.0625) | -12dB | One PTRS port per TRxP |

### 2.1.2	Simulation results

Figure 1 shows our simulation result with the separate processing. Table 3 summarizes the simulation results regarding SNR to achieve 70% of the peak rate.

Figure 1	sDCI-based SDM, separate processing.

Table 3	Summary of sDCI-based SDM transmission simulation results.

| Test number | MCS | Rank | FO for TRP2 (Hz) | TO for TRP1 (us) | rho (dB) | SNR (dB) to achieve 70% of peak |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 17 | 1+1 | 600 | -0.0625 | -12dB | 13.3 |
| Test 1-2 | 17 | 1+1 | 0 | 0.25 | -12dB | 12.8 |

## 2.2	mDCI-based full-overlapping transmission

### 2.2.1	Simulation parameters

Table 4 and Table 5 show the simulation parameters according to [3].

Table 4	Simulation parameters for mDCI-based full-overlapping transmission.

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  |  | 0,1 |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 40 |  |
|  |  | CSI-RS offset |  | Slots | 80 for CSI-RS resources 1 and 2 81 for CSI-RS resources 3 and 4 | 80 for CSI-RS resources 5 and 6 81 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 |  |
| CSI-RS for beam refinement |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2 | k0=1 for CSI-RS resources 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 | 1 for CSI-RS resource 3,4 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 160 |  |
|  |  | CSI-RS offset |  | Slots | 0 for CSI-RS resources 1,2 | 0 for CSI-RS resources 3, 4 |
|  |  | QCL info |  |  | TCI state #1 | TCI state #2 |
| Duplex mode |  |  |  |  | TDD |  |
| Active DL BWP index |  |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |
|  | k0 |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  | 2 |  |
|  | Length (L) |  |  |  | 12 |  |
|  | PRB bundling type |  |  |  | Static |  |
|  | PRB bundling size |  |  |  | 2 |  |
|  | Resource allocation type |  |  |  | Type 1 |  |
|  | RBG size |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |
|  | Num CDM groups without data |  |  |  | 2 | 2 |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | {1000} | {1002} |
|  | TCI state |  |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type D | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type D |
| Resource allocation |  |  |  |  | Fully-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  | us | -0.0625 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  | Hz | 600 |  |
| Number of HARQ Processes |  |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Precoding configuration |  |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |  |

Table 5	Additional simulation parameters for mDCI-based full-overlapping transmission.

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Rho |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |
| 1-1 | 64QAM, 0.43 (MCS 17) | 64QAM, 0.43 (MCS17) | 100 / 120 (66PRBs for each TRxP) | FR2.120-1 | TDLA30-75 | 2x4 XP per TRxP (α=β=0, γ=0.0625) | -12dB |

### 2.2.2	Simulation results

Figure 2 shows our simulation result with separate processing. Table 6 summarizes the simulation results regarding SNR to achieve 70% of the peak rate.

Figure 2	mDCI-based full-overlapping, separate processing.

Table 6	Summary of mDCI-based full-overlapping transmission simulation results.

| Test number | MCS | Rank | FO for TRP2 (Hz) | TO for TRP1 (us) | rho (dB) | SNR (dB) to achieve 70% of peak |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 17 | 1+1 | 600 | -0.0625 | -12dB | 10.3 |

## 2.3	mDCI-based non-overlapping transmission

### 2.3.1	Simulation parameters

Table 7 and Table 8 show the simulation parameters according to [4].

Table 7	Simulation parameters for mDCI-based non-overlapping transmission.

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  |  | 0,1 |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 40 |  |
|  |  | CSI-RS offset |  | Slots | 80 for CSI-RS resources 1 and 2 81 for CSI-RS resources 3 and 4 | 80 for CSI-RS resources 5 and 6 81 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 |  |
| CSI-RS for beam refinement |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2 | k0=1 for CSI-RS resources 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 | 1 for CSI-RS resource 3,4 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 160 |  |
|  |  | CSI-RS offset |  | Slots | 0 for CSI-RS resources 1,2 | 0 for CSI-RS resources 3, 4 |
|  |  | QCL info |  |  | TCI state #1 | TCI state #2 |
| Duplex mode |  |  |  |  | TDD |  |
| Active DL BWP index |  |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |
|  | k0 |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  | 2 |  |
|  | Length (L) |  |  |  | 12 |  |
|  | PRB bundling type |  |  |  | Static |  |
|  | PRB bundling size |  |  |  | 2 |  |
|  | Resource allocation type |  |  |  | Type 1 |  |
|  | RBG size |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |
|  | Num CDM groups without data |  |  |  | 2 | 2 |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | {1000, 1001} | {1002, 1003} |
|  | TCI state |  |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type D | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type D |
| Resource allocation |  |  |  |  | Non-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  | us | 0.25 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  | Hz | 0 |  |
| Number of HARQ Processes |  |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Precoding configuration |  |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |  |

Table 8	Additional simulation parameters for mDCI-based non-overlapping transmission

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Rho |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |
| 1-1 | 16QAM, 0.48 (MCS 13) | 16QAM, 0.48 (MCS13) | 100 / 120 (33PRBs for each TRxP) | FR2.120-1 | TDLA30-75 | 2x4 XP per TRxP (α=β=0, γ=0.0625) | -12dB |

### 2.3.2	Simulation results

Figure 3 shows our simulation result, and Table 9 summarizes the simulation results regarding SNR to achieve 70% of the peak rate.

Figure 3	mDCI-based non-overlapping.

Table 9	Summary of mDCI-based non-overlapping transmission simulation results.

| Test number | MCS | Rank | FO for TRP2 (Hz) | TO for TRP1 (us) | rho (dB) | SNR (dB) to achieve 70% of peak |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 13 | 2+2 | 0 | 0.25 | -12dB | 12.5 |

# 3	Discussion

According to the WF [1], the remaining open issue is the Tx EVM assumption.

Issue 1-1-8: TxEVM. - FFS for further study.

When RAN4 sets the NR UE demodulation requirements, companies assume TxEVM with 6% for QPSK, 16QAM and 64QAM, and it is modelled as AWGN. If we observe our simulation results for the FR2 multi-Rx reception scenarios, the required SNRs are about 13dB, 10dB, and 12dB for sDCM, full-overlapped mDCI and non-overlapped mDCI scenarios, respectively. We don’t expect TxEVM gives the significant impacts in such SNR regions. Therefore, we propose to reuse the same assumption of TxEVM as Rel-15 NR UE demodulation requirements.

Proposal: Derive FR2 multi-Rx UE demodulation requirements with the assumption TxEVM=6%.

# 4	Summary

Proposal: Derive FR2 multi-Rx UE demodulation requirements with the assumption TxEVM=6%.

# References

- R4-2321140, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm.

- R4-2321205, “Draft CR on Minimum requirements and FRC definition for sDCI SDM (TS38.101-4, Rel-18)”, Huawei, HiSilicon.

- R4-2321207, 	“DraftCR on PDSCH demod requirements for mDCI fully-overlapping with multi-RX in FR2”, Apple.

- R4-2321208, “DraftCR on Minimum requirements and Reference Channel for mDCI non-overlapping”, Nokia, Nokia Shanghai Bell.