3GPP TSG-RAN4 Meeting #110	R4-2401762

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2113 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR on Introduction of completed SUL band combinations into TS 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SUL_combos_R18-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - R4-2403787, draftCR to 38.101-1: SUL band combinations wit 78C In RAN#102, it was agreed that some SUL band combination with 78C were captured in the WID on Rel-18 SUL basket. After checking, there is no additional triple beat due to UL configurations with SUL band, considering no simultaneous UL transmissions between SUL band and NUL band. And there is no additional MSD for the fallback UL configurations. - The following TP were approved to introduce SUL band combinations in RAN4#110 meeting. R4-2400857 TP to TR 37.718-00-00: SUL_n5A-n84A R4-2403784 TP to TR 37.718-00-00: SUL_n8A-n84A R4-2403785 TP to TR 37.718-00-00 CA_n5A_n78A-n84A, CA_n78A_n5A-n84A and CA_n1A_n78A-n89A R4-2403795 TP to TR 37.718-00-00 CA_n3A_n78A-n84A and CA_n3A_n78C-n84A R4-2403786 TP to TR 37.718-00-00 CA_n8A_n78A-n84A and CA_n78A_n8A-n84A |  |  |  |  |  |  |  |  |
| Summary of change: |  | - R4-2403787, draftCR to 38.101-1: SUL band combinations wit 78C This draftCR is to add CA_n78C_n1A-n89A, CA_n78C_n1A-n80A, CA_n1A_n78C-n80A, CA_n78C_n3A-n84A, CA_n78C_n1A-n81A and CA_n1A_n78C-n81A. - To introduce the following SUL band combinations in RAN4#110 meeting. SUL_n5A-n84A SUL_n8A-n84A n5A_n78A-n84A, CA_n78A_n5A-n84A and CA_n1A_n78A-n89A CA_n3A_n78A-n84A and CA_n3A_n78C-n84A CA_n8A_n78A-n84A and CA_n78A_n8A-n84A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - R4-2403787, draftCR to 38.101-1: SUL band combinations wit 78C The SUL band combinations above cannot be supported in the specification. - The following SUL band combinations can’t be supported in the specificaiton. SUL_n5A-n84A SUL_n8A-n84A n5A_n78A-n84A, CA_n78A_n5A-n84A and CA_n1A_n78A-n89A CA_n3A_n78A-n84A and CA_n3A_n78C-n84A CA_n8A_n78A-n84A and CA_n78A_n8A-n84A |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2C, 5.5C, 6.2C.2, 7.3C.2, 7.3C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.101-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change>>

5.2C	Operating band combination for SUL

NR operation is designed to operate in the operating band combination defined in Table 5.2C-1, Table 5.2C-2, Table 5.2C-3 and Table 5.2C-4, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order band combination.

Table 5.2C-1: Operating band combination for SUL in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n1-n802 | n1, n80 |
| SUL_n1-n812 | n1, n81 |
| SUL_n1-n892 | n1, n89 |
| SUL_n3-n842 | n3, n84 |
| SUL_n5-n842 | n5, n84 |
| SUL_n8-n842 | n8, n84 |
| SUL_n24-n992 | n24, n99 |
| SUL_n41-n802 | n41, n80 |
| SUL_n41-n812 | n41, n81 |
| SUL_n41-n832 | n41, n83 |
| SUL_n41-n952 | n41, n95 |
| SUL_n41-n972 | n41, n97 |
| SUL_n41-n982 | n41, n98 |
| SUL_n41-n992 | n41, n99 |
| SUL_n48-n992 | n48, n99 |
| SUL_n77-n802 | n77, n80 |
| SUL_n77-n842 | n77, n84 |
| SUL_n77-n992 | n77, n99 |
| SUL_n78-n802 | n78, n80 |
| SUL_n78-n812 | n78, n81 |
| SUL_n78-n822 | n78, n82 |
| SUL_n78-n832 | n78, n83 |
| SUL_n78-n842 | n78, n84 |
| SUL_n78-n862 | n78, n86 |
| SUL_n78-n892 | n78, n89 |
| SUL_n79-n802 | n79, n80 |
| SUL_n79-n812 | n79, n81 |
| SUL_n79-n832 | n79, n83 |
| SUL_n79-n842 | n79, n84 |
| SUL_n79-n952 | n79, n95 |
| SUL_n79-n972 | n79, n97 |
| SUL_n79-n982 | n79, n98 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

Table 5.2C-2: Operating SUL band combination with intra-band non-contiguous CA in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n41(*)-n992 | n41, n99 |
| CA_n48(*)-n992 | n48, n99 |
| CA_n77(*)-n992 | n77, n99 |
| CA_n78(*)-n862 | n78, n86 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. NOTE 3: The notation CA_nX(*) in this table indicates intra-band non-contiguous CA for band nX. The configurations for each band are in table 5.5C-2. |  |

Table 5.2C-3: Operating SUL band combination with intra-band contiguous CA in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n41-n80 | n41, n80 |
| CA_n41-n83 | n41, n83 |
| CA_n41-n95 | n41, n95 |
| CA_n41-n98 | n41, n98 |
| CA_n78-n80 | n78, n80 |
| CA_n78-n81 | n78, n81 |
| CA_n78-n84 | n78, n84 |
| CA_n78-n89 | n78, n89 |
| CA_n79-n80 | n79, n80 |
| CA_n79-n83 | n79, n83 |
| CA_n79-n95 | n79, n95 |
| CA_n79-n98 | n79, n98 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

Table 5.2C-4: Operating SUL band combination with inter-band CA in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1_n78-n80 | n1, n78, n80 |
| CA_n1_n78-n81 | n1, n78, n81 |
| CA_n1_n78-n84 | n1, n78, n84 |
| CA_n1_n78-n89 | n1, n78, n89 |
| CA_n3_n41-n80 | n3, n41, n80 |
| CA_n3_n78-n80 | n3, n78, n80 |
| CA_n3_n78-n84 | n3, n78, n84 |
| CA_n3_n79-n80 | n3, n79, n80 |
| CA_n5_n78-n84 | n5, n78, n84 |
| CA_n8_n78-n81 | n8, n78, n81 |
| CA_n8_n78-n84 | n8, n78, n84 |
| CA_n28_n41-n83 | n28, n41, n83 |
| CA_n28_n79-n83 | n28, n79, n83 |
| CA_n41_n79-n80 | n41, n79, n80 |
| CA_n41_n79-n83 | n41, n79, n83 |
| CA_n41_n79-n95 | n41, n79, n95 |
| CA_n41_n79-n97 | n41, n79, n97 |
| CA_n41_n79-n98 | n41, n79, n98 |
| CA_n78_n1-n80 | n1, n78, n80 |
| CA_n78_n1-n81 | n1, n78, n81 |
| CA_n78_n1-n89 | n1, n78, n89 |
| CA_n78_n3-n84 | n3, n78, n84 |
| CA_n78_n5-n84 | n5, n78, n84 |
| CA_n78_n8-n84 | n8, n78, n84 |
| CA_n79_n41-n80 | n41, n79, n80 |
| CA_n79_n41-n83 | n41, n79, n83 |
| CA_n79_n41-n95 | n41, n79, n95 |
| CA_n79_n41-n97 | n41, n79, n97 |
| CA_n79_n41-n98 | n41, n79, n98 |
| CA_n28-n79_n41-n83 | n28, n41, n79, n83 |
| CA_n28-n41_n79-n83 | n28, n41, n79, n83 |
| CA_n41A-n95A_n79A-n98A | n41, n95, n79, n98 |
| CA_n41A-n98A_n79A-n95A | n41, n98, n79, n95 |
| CA_n41A-n83A_n79A-n98A | n41, n83, n79, n98 |
| CA_n41A-n83A_n79A-n95A | n41, n83, n79, n95 |
| CA_n78C_n80A-n84A | n78, n80, n84 |
| CA_n78C_n81A-n84A | n78, n81, n84 |
| CA_n78C_n84A-n89A | n78, n84, n89 |
| NOTE 1: If a UE is configured with a single cell consisting of a NR UL carrier and a corresponding NR SUL carrier, the switching time between NR UL carrier and the corresponding NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

<<Next of Change>>

5.5C	Configurations for SUL

The configuration tables for SUL describe Bandwidth Combination Sets. Bandwidth Combination Set 4 and 5 contains all possible defined channel bandwidths for each band in the combination. The fact that BCS4 and BCS5 contains all channel bandwidths for each band does not alter if a bandwidth is mandatory or optional for a given band. Bandwidths that are identified as optional in Table 5.3.5-1 for a given release are still optional for UEs that support BCS4 or BCS5. , where the bandwidths the UE supports for each band, the maximum bandwidth and/or minimum bandwidth for the band in the band combination are indicated in the UE capabilities. Note that the minimum bandwidth is indicated only in BCS5 and BCS5 shall not be indicated together with BCS4 for a SUL configuration. For SUL band combinations including FR1 intra-band CA and with BCS4 or BCS5, the Bandwidth Combination Sets for the FR1 intra-band CA are BCS4 or BCS5.

For the NR SUL band configurations with inter-band CA in sub-clause 5.5C, when the capability [BandCombination-UplinkTxSwitch-r18] is present, three or four bands can be configured in the uplink with simultaneous uplink transmission on up to two bands, and the corresponding requirements for SUL band configurations with inter-band CA and with uplink assigned to one or two bands shall apply. For each uplink band pair in the NR SUL band configurations with inter-band CA, according to the capability [uplinkTxSwitchingOptionForBandPair],

–	if switchedUL is supported, uplink transmission on any one band of the band pair in the band combination shall be supported according to the scheduling commands, and the corresponding requirements for SUL band configuration with inter-band CA and with uplink assigned to one band on band X or band Y apply;

–	if dualUL is supported, simultaneous uplink transmission on the two NR UL bands from the band pair for which dualUL is declared in the band combination shall be supported according to the scheduling commands, and the corresponding requirements for SUL band configuration with inter-band CA and with uplink CA between the two uplink bands apply.

For SUL band configuration with inter-band CA, band pair(s) of two non-SUL bands with switchedUL or dualUL by the parameter [uplinkTxSwitchingOption] is supported, and any other band pair(s) including SUL with switchedUL is supported, in release 18.

Table 5.5C-1: Supported channel bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n1A-n80A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | n80 | 5, 10, 15, 20, 25, 30 |  |
| SUL_n1A-n81A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | n81 | 5, 10, 15, 20 |  |
| SUL_n1A-n89A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | n89 | 5, 10, 15, 20 |  |
| SUL_n3A-n84A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | n84 | 5, 10, 15, 20 |  |
| SUL_n5A-n84A | n5 | 5, 10, 15, 20 | 0 |
|  | n84 | 5, 10, 15, 20 |  |
| SUL_n8A-n84A | n8 | 5, 10, 15, 20 | 0 |
|  | n84 | 5, 10, 15, 20 |  |
| SUL_n24A-n99A | n24 | 5, 10 | 0 |
|  | n99 | 5, 10 |  |
| SUL_n41A-n80A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n80 | 5, 10, 15, 20, 25, 30 |  |
|  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| SUL_n41A-n81A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n81 | 5, 10, 15, 20 |  |
| SUL_n41A-n83A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  | n83 | 5, 10, 15, 20, 30 |  |
| SUL_n41A-n95A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  | n95 | 5, 10, 15 |  |
| SUL_n41A-n97A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  | n97 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  | n97 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| SUL_n41A-n98A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| SUL_n41A-n99A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  | n99 | 5, 10 |  |
| SUL_n48A-n99A | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n99 | 5, 10 |  |
| SUL_n77A-n80A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n80 | 5, 10, 15, 20, 25, 30 |  |
| SUL_n77A-n84A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n84 | 5, 10, 15, 20 |  |
| SUL_n77A-n99A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n99 | 5, 10 |  |
| SUL_n78A-n80A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n80 | 5, 10, 15, 20, 25, 30 |  |
|  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| SUL_n78A-n81A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n81 | 5, 10, 15, 20 |  |
|  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  | n81 | 5, 10, 15, 20 |  |
| SUL_n78A-n82A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n82 | 5, 10, 15, 20 |  |
| SUL_n78A-n83A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n83 | 5, 10, 15, 20 |  |
|  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  | n83 | 5, 10, 15, 20, 30 |  |
| SUL_n78A-n84A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n84 | 5, 10, 15, 20 |  |
|  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| SUL_n78A-n86A | n78 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  | n86 | 5, 10, 15, 20 |  |
| SUL_n78A-n89A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  | n89 | 5, 10, 15, 20 |  |
| SUL_n79A-n80A | n79 | 40, 50, 60, 80, 100 | 0 |
|  | n80 | 5, 10, 15, 20, 25, 30 |  |
|  | n79 | 40, 50, 60, 80, 100 | 1 |
|  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| SUL_n79A-n81A | n79 | 40, 50, 60, 80, 100 | 0 |
|  | n81 | 5, 10, 15, 20 |  |
| SUL_n79A-n83A | n79 | 40, 50, 60, 80, 100 | 0 |
|  | n83 | 5, 10, 15, 20, 30 |  |
| SUL_n79A-n84A | n79 | 40, 50, 60, 80, 100 | 0 |
|  | n84 | 5, 10, 15, 20 |  |
| SUL_n79A-n95A | n79 | 40, 50, 60, 80, 100 | 0 |
|  | n95 | 5, 10, 15 |  |
| SUL_n79A-n97A | n79 | 40, 50, 60, 80, 100 | 0 |
|  | n97 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  | n79 | 40, 50, 60, 80, 100 | 1 |
|  | n97 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n79 | See n79 channel bandwidths in Table 5.3.5-1 for each carrier | 4 and 5 |
|  | n97 | See n97 channel bandwidths in Table 5.3.5-1 for each carrier |  |
| SUL_n79A-n98A | n79 | 40, 50, 60, 80, 100 | 0 |
|  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |

Table 5.5C-2: Supported channel bandwidths per SUL band combination with intra-band non-contiguous CA

| SUL band combination with intra-band non-contiguous CA | SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41(2A)-n99A | SUL_n41A-n99A | n41 | CA_n41(2A)_BCS0 | 0 |
|  |  | n99 | 5, 10 |  |
| CA_n48(2A)-n99A | SUL_n48A-n99A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n99 | 5, 10 |  |
| CA_n77(2A)-n99A | SUL_n77A-n99A | n77 | CA_n77(2A)_BCS0 | 0 |
|  |  | n99 | 5, 10 |  |
| CA_n78(2A)-n86A | SUL_n78A-n86A | n78 | CA_n78(2A)_BCS0 | 0 |
|  |  | n86 | 5, 10, 15, 20 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

Table 5.5C-3: Supported channel bandwidths per SUL band combination with intra-band contiguous CA

| SUL band combination with CA | SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41C-n80A | SUL_n41A-n80A | n41 | CA_n41C_BCS1 | 0 |
|  | CA_n41C-n80A | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41C-n83A | SUL_n41A-n83A | n41 | CA_n41C_BCS1 | 0 |
|  | CA_n41C-n83A | n83 | 5, 10, 15, 20, 30 |  |
| CA_n41C-n95A | SUL_n41A-n95A | n41 | CA_n41C_BCS1 | 0 |
|  | CA_n41C-n95A | n95 | 5, 10, 15 |  |
| CA_n41C-n98A | SUL_n41A-n98A | n41 | CA_n41C_BCS1 | 0 |
|  | CA_n41C-n98A | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n78C-n80A | SUL_n78A-n80A | n78 | CA_n78C_BCS1 | 0 |
|  | CA_n78C-n80A | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n78C-n81A | SUL_n78A-n81A | n78 | CA_n78C_BCS1 | 0 |
|  | CA_n78C-n81A | n81 | 5, 10, 15, 20 |  |
| CA_n78C-n84A | SUL_n78A-n84A | n78 | CA_n78C_BCS1 | 0 |
|  | CA_n78C-n84A | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n78C-n89A | SUL_n78A-n89A | n78 | CA_n78C_BCS1 | 0 |
|  | CA_n78C-n89A | n89 | 5, 10, 15, 20 |  |
| CA_n79C-n80A | SUL_n79A-n80A | n79 | CA_n79C_BCS0 | 0 |
|  | CA_n79C-n80A | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n79C-n83A | SUL_n79A-n83A | n79 | CA_n79C_BCS0 | 0 |
|  | CA_n79C-n83A | n83 | 5, 10, 15, 20, 30 |  |
| CA_n79C-n95A | SUL_n79A-n95A | n79 | CA_n79C_BCS0 | 0 |
|  | CA_n79C-n95A | n95 | 5, 10, 15 |  |
| CA_n79C-n98A | SUL_n79A-n98A | n79 | CA_n79C_BCS0 | 0 |
|  | CA_n79C-n98A | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

Table 5.5C-4: Supported channel bandwidths per SUL band combination with inter-band CA

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A_n78A-n80A | SUL_n78A-n80A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A_n78C-n80A | SUL_n78A-n80A CA_n78C CA_n1A-n78A CA_n78C-n80A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A_n78A-n81A | SUL_n78A-n81A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n81 | 5, 10, 15, 20 |  |
| CA_n1A_n78C-n81A | SUL_n78A-n81A CA_n78C-n81A CA_n78C CA_n1A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  |  | n81 | 5, 10, 15, 20 |  |
| CA_n1A_n78A-n84A | SUL_n78A-n84A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A_n78C-n84A | SUL_n78A-n84A CA_n78C-n84A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A_n78A-n89A | SUL_n78A-n89A CA_n1A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n89 | 5, 10, 15, 20 |  |
| CA_n1A_n78C-n89A | SUL_n78A-n89A CA_n78C CA_n1A-n78A CA_n78C-n89A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
|  |  | n89 | 5, 10, 15, 20 |  |
| CA_n3A_n41A-n80A | SUL_n41A-n80A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A_n41C-n80A | SUL_n41A-n80A CA_n41C-n80A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A_n78A-n80A | SUL_n78A-n80A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A_n78C-n80A | SUL_n78A-n80A CA_n78C-n80A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A_n78A-n84A | SUL_n78A-n84A CA_n3A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n84 | 5, 10, 15, 20 |  |
| CA_n3A_n78C-n84A | SUL_n78A-n84A CA_n3A-n78A CA_n78C CA_n78C-n84A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
|  |  | n84 | 5, 10, 15, 20 |  |
| CA_n3A_n79A-n80A | SUL_n79A-n80A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A_n79C-n80A | SUL_n79A-n80A CA_n79C-n80A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A_n78A-n84A | SUL_n78A-n84A CA_n5A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n84 | 5, 10, 15, 20 |  |
| CA_n5A_n78C-n84A | SUL_n78A-n84A CA_n5A-n78A CA_n78C CA_n78C-n84A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
|  |  | n84 | 5, 10, 15, 20 |  |
| CA_n8A_n78A-n81A | SUL_n78A-n81A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n81 | 5, 10, 15, 20 |  |
| CA_n8A_n78A-n84A | SUL_n78A-n84A CA_n8A-n78A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n84 | 5, 10, 15, 20 |  |
| CA_n8A_n78C-n84A | SUL_n78A-n84A CA_n78C CA_n8A-n78A CA_n78C-n84A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
|  |  | n84 | 5, 10, 15, 20 |  |
| CA_n28A_n41A-n83A | SUL_n41A-n83A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n28A_n41C-n83A | SUL_n41A-n83A CA_n41C-n83A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n28A_n79A-n83A | SUL_n79A-n83A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n28A_n79C-n83A | SUL_n79A-n83A CA_n79C-n83A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n41A_n79A-n80A | SUL_n79A-n80A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A_n79A-n83A | SUL_n79A-n83A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n41A_n79C-n83A | SUL_n79A-n83A CA_n79C-n83A CA_n41A-n79A CA_n79C | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n41C_n79A-n83A | SUL_n79A-n83A CA_n41C CA_n41A-n79A | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n41C_n79C-n83A | CA_n41C CA_n79C SUL_n79A-n83A CA_n79C-n83A CA_n41A-n79A | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n41A_n79A-n95A | SUL_n79A-n95A | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n95 | 5, 10, 15 |  |
| CA_n41A_n79C-n95A | SUL_n79A-n95A CA_n79C-n95A CA_n41A-n79A CA_n79C | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n95 | 5, 10, 15 |  |
| CA_n41C_n79A-n95A | SUL_n79A-n95A CA_n41C CA_n41A-n79A | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n95 | 5, 10, 15 |  |
| CA_n41C_n79C-n95A | CA_n41C CA_n79C SUL_n79A-n95A CA_n79C-n95A CA_n41A-n79A | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n98 | 5, 10, 15 |  |
| CA_n41A_n79A-n97A | SUL_n79A-n97A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n97 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A_n79A-n98A | SUL_n79A-n98A | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A_n79C-n98A | SUL_n79A-n98A CA_n79C-n98A CA_n41A-n79A CA_n79C | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41C_n79A-n98A | SUL_n79A-n98A CA_n41C CA_n41A-n79A | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41C_n79C-n98A | CA_n41C CA_n79C SUL_n79A-n98A CA_n79C-n98A CA_n41A-n79A | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n78A_n1A-n80A | SUL_n1A-n80A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30 |  |
| CA_n78C_n1A-n80A | SUL_n1A-n80A CA_n78C CA_n1A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30 |  |
| CA_n78A_n1A-n81A | SUL_n1A-n81A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n81 | 5, 10, 15, 20 |  |
| CA_n78C_n1A-n81A | SUL_n1A-n81A CA_n78C CA_n1-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  |  | n81 | 5, 10, 15, 20 |  |
| CA_n78A_n1A-n89A | SUL_n1A-n89A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n89 | 5, 10, 15, 20 |  |
| CA_n78C_n1A-n89A | SUL_n1A-n89A CA_n78C CA_n1A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  |  | n89 | 5, 10, 15, 20 |  |
| CA_n78A_n3A-n84A | SUL_n3A-n84A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n84 | 5, 10, 15, 20 |  |
| CA_n78C_n3A-n84A | SUL_n3A-n84A CA_n78C CA_n3A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  |  | n84 | 5, 10, 15, 20 |  |
| CA_n78A_n5A-n84A | SUL_n5A-n84A CA_n5A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n84 | 5, 10, 15, 20 |  |
| CA_n78C_n5A-n84A | SUL_n5A-n84A CA_n78C CA_n5A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
|  |  | n84 | 5, 10, 15, 20 |  |
| CA_n78A_n8A-n84A | SUL_n8A-n84A CA_n8A-n78A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n84 | 5, 10, 15, 20 |  |
| CA_n78C_n8A-n84A | SUL_n8A-n84A CA_n78C CA_n8A-n78A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |
|  |  | n84 | 5, 10, 15, 20 |  |
| CA_n79A_n41A-n80A | SUL_n41A-n80A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n79A_n41A-n83A | SUL_n41A-n83A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n79A_n41C-n83A | SUL_n41A-n83A CA_n41C-n83A CA_n41A-n79A CA_n41C | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n79C_n41A-n83A | SUL_n41A-n83A CA_n41A-n79A CA_n79C | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n79C_n41C-n83A | CA_n41C CA_n79C SUL_n41A-n83A CA_n41C-n83A CA_n41A-n79A | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n79A_n41A-n95A | SUL_n41A-n95A | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n95 | 5, 10, 15 |  |
| CA_n79C_n41A-n95A | SUL_n41A-n95A CA_n41A-n79A CA_n79C | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n95 | 5, 10, 15 |  |
| CA_n79A_n41C-n95A | SUL_n41A-n95A CA_n41C-n95A CA_n41A-n79A CA_n41C | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n95 | 5, 10, 15 |  |
| CA_n79C_n41C-n95A | CA_n41C CA_n79C SUL_n41A-n95A CA_n41C-n95A CA_n41A-n79A | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n95 | 5, 10, 15 |  |
| CA_n79A_n41A-n97A | SUL_n41A-n97A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n97 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n79A_n41A-n98A | SUL_n41A-n98A | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n79C_n41A-n98A | SUL_n41A-n98A CA_n41A-n79A CA_n79C | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n79A_n41C-n98A | SUL_n41A-n98A CA_n41C-n98A CA_n41A-n79A CA_n41C | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n79C_n41C-n98A | CA_n41C CA_n79C SUL_n41A-n98A CA_n41C-n98A CA_n41A-n79A | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n28A-n79A_n41A-n83A | SUL_n41A-n83A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n28A-n41A_n79A-n83A | SUL_n79A-n83A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

Table 5.5C-5: Supported channel bandwidths per SUL band combination with inter-band CA (two SUL cells)

| SUL band combination with CA | Uplink CA configuration or SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n95A_n79A-n98A | SUL_n41A-n95A SUL_n79A-n98A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n95 | 5, 10, 15 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A-n98A_n79A-n95A | SUL_n41A-n98A SUL_n79A-n95A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n95 | 5, 10, 15 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A-n83A_n79A-n98A | SUL_n41A-n83A SUL_n79A-n98A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n83 | 5, 10, 15, 20,30 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A-n83A_n79A-n95A | SUL_n41A-n83A SUL_n79A-n95A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n83 | 5, 10, 15, 20,30 |  |
|  |  | n95 | 5, 10, 15 |  |
| CA_n78C_n80A-n84A | SUL_n78A-n80A SUL_n78A-n84A CA_n78C2 | n78 | CA_n78C_BCS1 | 0 |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n78C_n81A-n84A | SUL_n78A-n81A SUL_n78A-n84A CA_n78C2 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 CA_n78C_BCS1 | 0 |
|  |  | n81 | 5, 10, 15, 20 |  |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n78C_n84A-n89A | SUL_n78A-n84A SUL_n78A-n89A CA_n78C | n78 | CA_n78C_BCS1 | 0 |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n89 | 5, 10, 15, 20 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 2: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

<<Next of Change>>

6.2C.2	ΔTIB,c

For the UE which supports SUL band combination, ΔTIB,c in Tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

Table 6.2C.2-1: ΔTIB,c due to SUL

| Band combination for SUL | ΔTIB,c for NR bands (dB)3 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration4 |  |
| SUL_n1-n80 | 0.3 | 0.3 |
| SUL_n1-n81 | 0.3 | 0.3 |
| SUL_n1-n89 | 0.3 | 0.3 |
| SUL_n3-n84 | 0.3 | 0.3 |
| SUL_n5-n84 | 0.3 | 0.3 |
| SUL_n8-n84 | 0.3 | 0.3 |
| SUL_n41-n80 | 0.31 / 0.82 | 0.5 |
| SUL_n41-n81 | 0.3 | 0.3 |
| SUL_n41-n83 | 0.3 | 0.3 |
| SUL_n41-n97 | 0.5 | 0.5 |
| SUL_n41-n98 | 0.5 | 0.5 |
| SUL_n41-n99 | 0.41 / 0.92 | 0.3 |
| SUL_n48-n99 | 0.6 | 0.8 |
| SUL_n77-n80 | 0.8 | 0.6 |
| SUL_n77-n84 | 0.8 | 0.6 |
| SUL_n77-n99 | 0.6 | 0.8 |
| SUL_n78-n80 | 0.8 | 0.6 |
| SUL_n78-n81 | 0.8 | 0.6 |
| SUL_n78-n82 | 0.8 | 0.6 |
| SUL_n78-n83 | 0.8 | 0.5 |
| SUL_n78-n84 | 0.8 | 0.3 |
| SUL_n78-n86 | 0.8 | 0.6 |
| SUL_n78-n89 | 0.8 | 0.6 |
| SUL_n79-n83 | 0.8 | 0.5 |
| SUL_n79-n97 | 0.8 | 0.3 |
| SUL_n79-n98 | 0.8 | 0.3 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2515MHz. NOTE 3: “-” denotes ΔTIB,c = 0. NOTE 4: The component band order in the configuration should be listed by the order of NR bands such as for SUL_n41-n81 the order of band is n41 and n81. |  |  |

Table 6.2C.2-2: ΔTIB,c for SUL band combination (Three bands)

| Band combination for SUL | ΔTIB,c for NR bands (dB)3 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration4 |  |  |
| CA_n1_n78-n80 | 0.6 | 0.8 | 0.6 |
| CA_n1_n78-n81 | 0.3 | 0.8 | 0.6 |
| CA_n1_n78-n84 | 0.6 | 0.8 | 0.6 |
| CA_n1_n78-n89 | 0.6 | 0.8 | 0.6 |
| CA_n3_n41-n80 | 0.5 | 0.31 / 0.82 | 0.5 |
| CA_n3_n78-n80 | 0.6 | 0.8 | 0.6 |
| CA_n3_n78-n84 | 0.6 | 0.8 | 0.6 |
| CA_n3_n79-n80 | 0.3 | 0.8 | 0.3 |
| CA_n5_n78-n84 | 0.6 | 0.8 | 0.6 |
| CA_n8_n78-n81 | 0.6 | 0.8 | 0.6 |
| CA_n8_n78-n84 | 0.6 | 0.8 | 0.3 |
| CA_n28_n41-n83 | 0.3 | 0.3 | 0.3 |
| CA_n28_n79-n83 | 0.5 | 0.8 | 0.5 |
| CA_n41_n79-n80 | 0.31 / 0.82 | 0.8 | 0.3 |
| CA_n41_n79-n83 | 0.3 | 0.8 | 0.5 |
| CA_n41_n79-n95 | 0.3 | 0.8 | 0.3 |
| CA_n41_n79-n97 | 0.5 | 0.5 | 0.5 |
| CA_n41_n79-n98 | 0.3 | 0.8 | 0.3 |
| CA_n78_n1-n80 | 0.8 | 0.6 | 0.6 |
| CA_n78_n1-n81 | 0.8 | 0.3 | 0.6 |
| CA_n78_n1-n89 | 0.8 | 0.6 | 0.6 |
| CA_n78_n3-n84 | 0.8 | 0.6 | 0.6 |
| CA_n78_n5-n84 | 0.8 | 0.6 | 0.6 |
| CA_n78_n8-n84 | 0.8 | 0.6 | 0.3 |
| CA_n78_n80-n84 | 0.8 | 0.6 | 0.6 |
| CA_n78_n81-n84 | 0.8 | 0.6 | 0.3 |
| CA_n78_n84-n89 | 0.8 | 0.6 | 0.6 |
| CA_n79_n41-n80 | 0.31 / 0.82 | 0.8 | 0.3 |
| CA_n79_n41-n83 | 0.3 | 0.8 | 0.5 |
| CA_n79_n41-n95 | 0.3 | 0.8 | 0.3 |
| CA_n79_n41-n97 | 0.5 | 0.5 | 0.5 |
| CA_n79_n41-n98 | 0.3 | 0.8 | 0.3 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515MHz. NOTE 3: “-” denotes ΔTIB,c = 0. NOTE 4: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

Table 6.2C.2-3: ΔTIB,c for SUL band combination (Four bands)

| Band combination for SUL | ΔTIB,c for NR bands (dB)1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |  |  |
| CA_n28-n79_n41-n83 | 0.3 | 0.3 | 0.5 | 0.3 |
| CA_n28-n41_n79-n83 | 0.3 | 0.3 | 0.5 | 0.3 |
| CA_n41A-n95A_n79A-n98A | 0.3 | 0.8 | - | 0.3 |
| CA_n41A-n98A_n79A-n95A | 0.5 | 0.8 | 0.5 | - |
| CA_n41A-n83A_n79A-n98A | 0.3 | 0.8 | 0.3 | 0.3 |
| CA_n41A-n83A_n79A-n95A | 0.3 | 0.8 | 0.3 | - |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n28-n79_n41-n83 the band order from left to right is n28, n41, n79 and n83. |  |  |  |  |

<<Next of Change>>

7.3C.2	Reference sensitivity power level for SUL

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in Table 7.3.2-1a, Table 7.3.2-1b and Table 7.3.2-2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 or supplementary uplink transmission bandwidth less than or equal to that specified in Table 7.3C.2-1 with reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1), unless sensitivity degradation is allowed in this clause of this specification. These exceptions also apply to any higher order CA or DC combination containing one of the exception combinations in this clause as subset.

For SUL operation with downlink CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 or supplementary uplink transmission bandwidth less than or equal to that specified in Table 7.3C.2-1 with reference measurement channels as specified in Annexes A.2.2.2,  A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1), unless sensitivity degradation is allowed in this clause of this specification. These exceptions also apply to any higher order CA or DC combination containing one of the exception combinations in this clause as subset.

Table 7.3C.2-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n80 | 15 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |
| n1 | n81 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |
| n1 | n841 | 15 | 25 | 50 | 75 | 100 | 128 | 128 | 128 | 128 |  |  |  |  |  |
| n1 | n89 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |
| n3 | n801 | 15 | 25 | 50 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |  |  |
| n3 | n84 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |
| n5 | n84 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| n8 | n84 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| n8 | n811 | 15 | 25 | 25 | 20 | 20 |  |  |  |  |  |  |  |  |  |
| n24 | n99 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |
| n28 | n831 | 15 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |  |  |  |
| n41 | n80 | 15 |  | 160 | 160 | 160 |  | 160 | 160 | 160 | 160 |  | 160 | 160 | 160 |
| n41 | n81 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n41 | n83 | 15 |  | 100 | 100 | 100 |  | 100 | 100 | 100 | 100 |  | 100 | 100 | 100 |
|  |  | 30 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n41 | n95 | 15 |  | 75 | 75 | 75 |  | 75 | 75 | 75 | 75 |  | 75 | 75 | 75 |
| n41 | n97 | 30 |  | 216 | 216 | 216 |  | 216 | 216 | 216 | 216 | 216 | 216 | 216 | 216 |
| n41 | n98 | 15 |  | 216 | 216 | 216 |  | 216 | 216 | 216 | 216 |  | 216 | 216 | 216 |
| n41 | n99 | 15 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n48 | n99 | 15 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n77 | n80 | 15 |  | 160 | 160 | 160 |  |  | 160 | 160 | 160 |  | 160 | 160 | 160 |
| n77 | n84 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n77 | n99 | 15 |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| n78 | n80 | 15 |  | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |
| n78 | n81 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | n82 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n78 | n83 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | n84 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | n86 | 15 |  | 216 | 216 | 216 |  |  | 216 | 216 | 216 |  | 216 | 216 | 216 |
| n78 | n89 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n79 | n80 | 15 |  |  |  |  |  |  | 160 | 160 | 160 |  | 160 |  | 160 |
| n79 | n83 | 15 |  |  |  |  |  |  | 100 | 100 | 100 |  | 100 |  | 100 |
|  |  | 30 |  |  |  |  |  |  | 50 | 50 | 50 |  | 50 |  | 50 |
| n79 | n81 | 15 |  |  |  |  |  |  | 100 | 100 | 100 |  | 100 |  | 100 |
| n79 | n84 | 15 |  |  |  |  |  |  | 100 | 100 | 100 |  | 100 |  | 100 |
| n79 | n95 | 15 |  |  |  |  |  |  | 75 | 75 | 75 |  | 75 |  | 75 |
| n79 | n97 | 15 |  |  |  |  |  |  | 270 | 270 | 270 |  | 270 |  | 270 |
| n79 | n98 | 15 |  |  |  |  |  |  | 216 | 216 | 216 |  | 216 |  | 216 |
| NOTE 1: The Tx-Rx carrier center frequency separation between SUL band and DL band is the same as the Tx-Rx carrier center frequency separation of DL band specified in table 5.4.4-1 from TS 38.101-1. The channel bandwidth of SUL band is the same as DL band. This restriction of REFSENS configurations applies also for these carriers when applicable SUL configuration is part of a higher order configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For the UE that supports any of the SUL operation given in Table 7.3C.2-2, exceptions to the requirements specified in Table 7.3.2-1a and Table 7.3.2-1b are allowed for different combinations of UL configurations and DL channel bandwidths when the uplink is active in a lower frequency band and is within a specified frequency range such that transmitter harmonics fall within the downlink transmission bandwidth assigned in a higher band as noted in Table 7.3C.2-2. For these exceptions, only the listed test points in Table 7.3C.2-2 are needed to be tested.

Table 7.3C.2-2: Reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to uplink harmonic issue)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n80 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n80 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n81 | n41 | 5 | 15 | 16 (RBstart=4) | 10 | 13 | NOTE 3 | UL3/DL1 direct-hit |
| n81 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 3.5 | NOTE 3 | UL3/DL1 direct-hit |
| n81 | n78 | 5 | 15 | 16 (RBstart=4) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n81 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n81 | n79 | 5 | 15 | 25 (RBstart=0) | 40 | 6.8 | NOTE 5 | UL5/DL1 direct-hit |
| n81 | n79 | 5 | 15 | 25 (RBstart=0) | 100 | 4.4 | NOTE 5 | UL5/DL1 direct-hit |
| n82 | n78 | 5 | 15 | 16 (RBstart=4) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n82 | n78 | 5 | 15 | 20 (RBstart=2) | 100 | 1.0 | NOTE 4 | UL4/DL1 direct-hit |
| n83 | n78 | 5 | 15 | 10 (RBstart=8) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n83 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| n84 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n84 | n77 | 10 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n84 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n86 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n86 | n78 | 10 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n86 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n89 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
| n89 | n78 | 20 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n97 | n79 | 5 | 15 | 100 (RBstart=0) | 40 | 29.4 | NOTE 2 | UL2/DL1 direct-hit |
| n97 | n79 | 5 | 15 | 270 (RBstart=0) | 100 | 25.3 | NOTE 2 | UL2/DL1 direct-hit |
| n99 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n99 | n77 | 10 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n99 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |

Table 7.3C.2-3: Void

For the UE that supports any of the SUL operation given in Table 7.3C.2-4, reference sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths when a DL band is impacted by UL band due to cross band isolation issues. For these exceptions, only the listed test points in Table 7.3C.2-4 are needed to be tested.

Table 7.3C.2-4: Reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to cross band isolation)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n80 | n41 | 1765 | 40 | 15 | 50 (RBstart=166) | 2501 | 10 | 0.7 | >ACLR2 |
| n80 | n41 | 1765 | 40 | 15 | 50 (RBstart=166) | 2546 | 100 | 0.7 | >ACLR2 |
| n84 | n3 | 1945 | 50 | 15 | 128 (RBstart=0) | 1877.5 | 5 | 19.7 | ACLR1 |
| n95 | n41 | 2017.5 | 15 | 15 | 75 (RBstart=4) | 2501 | 10 | 3.2 | >ACLR2 |
| n95 | n41 | 2017.5 | 15 | 15 | 75 (RBstart=4) | 2546 | 100 | 3.2 | >ACLR2 |
| n97 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 28.1 | ACLR2 |
| n97 | n41 | 2360 | 80 | 30 | 216 (RBstart=1) | 2546 | 100 | 10.6 | ACLR2 |
| n98 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.3 | >ACLR2 |

Table 7.3C.2-5: Void

7.3C.3	ΔRIB,c for SUL

7.3C.3.1	General

For a UE supporting a SUL configuration, the ΔRIB,c applies for both SC and SUL operation.

7.3C.3.2	SUL band combination

For the UE which supports SUL band combiantion, the minimum requirement for reference sensitivity in clause 7.3C.2 shall be increased by the amount given in ΔRIB,c defined in clause 7.3C.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3] for the applicable operating bands.

7.3C.3.2.1	ΔRIB,c  for two bands

Table 7.3C.3.2.1-1: ΔRIB,c due to SUL (two bands)

| Band combination for SUL | ΔRIB,c for NR band (dB)2 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration3 |  |
| SUL_n41-n80 | 0.51 | N/A |
| SUL_n41-n95 | 0.2 | N/A |
| SUL_n41-n98 | 0.2 | N/A |
| SUL_n48-n99 | 0.5 | N/A |
| SUL_n77-n80 | 0.5 | N/A |
| SUL_n77-n84 | 0.5 | N/A |
| SUL_n77-n99 | 0.5 | N/A |
| SUL_n78-n80 | 0.5 | N/A |
| SUL_n78-n81 | 0.5 | N/A |
| SUL_n78-n82 | 0.5 | N/A |
| SUL_n78-n83 | 0.5 | N/A |
| SUL_n78-n84 | 0.5 | N/A |
| SUL_n78-n86 | 0.5 | N/A |
| SUL_n79-n83 | 0.5 | N/A |
| SUL_n79-n97 | 0.5 | N/A |
| SUL_n79-n98 | 0.5 | N/A |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 2: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE 3: The component band order in the configuration should be listed by the order of NR band, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |

7.3C.3.2.2	ΔRIB,c  for three bands

Table 7.3C.3.2.2-1: ΔRIB,c due to SUL (three bands)

| Band combination for SUL | ΔRIB,c for NR bands (dB)2 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration3 |  |  |
| CA_n1_n78-n80 | 0.2 | 0.5 | N/A |
| CA_n1_n78-n81 | - | 0.5 | N/A |
| CA_n1_n78-n84 | 0.2 | 0.5 | N/A |
| CA_n1_n78-n89 | 0.2 | 0.5 | N/A |
| CA_n3_n41-n80 | - | 0.51 | N/A |
| CA_n3_n78-n80 | 0.2 | 0.5 | N/A |
| CA_n3_n78-n84 | 0.2 | 0.5 | N/A |
| CA_n3_n79-n80 | - | 0.5 | N/A |
| CA_n5_n78-n84 | 0.2 | 0.5 | N/A |
| CA_n28_n41-n83 | 0.2 | - | N/A |
| CA_n8_n78-n81 | 0.2 | 0.5 | N/A |
| CA_n8_n78-n84 | 0.2 | 0.5 | N/A |
| CA_n28_n79-n83 | 0.2 | 0.5 | N/A |
| CA_n41_n79-n80 | 0.5 | 0.5 | N/A |
| CA_n41_n79-n83 | 0.5 | 0.5 | N/A |
| CA_n41_n79-n95 | - | 0.5 | N/A |
| CA_n41_n79-n97 | - | 0.8 | N/A |
| CA_n41_n79-n98 | - | 0.5 | N/A |
| CA_n78_n1-n80 | 0.5 | 0.2 | N/A |
| CA_n78_n1-n81 | 0.5 | - | N/A |
| CA_n78_n1-n89 | 0.5 | 0.2 | N/A |
| CA_n78_n3-n84 | 0.5 | 0.2 | N/A |
| CA_n78_n5-n84 | 0.5 | 0.2 | N/A |
| CA_n78_n8-n84 | 0.5 | 0.2 | N/A |
| CA_n79_n41-n80 | 0.5 | 0.5 | N/A |
| CA_n78_n80-n84 | 0.5 | - | N/A |
| CA_n78_n81-n84 | 0.5 | - | N/A |
| CA_n78_n84-n89 | 0.5 | - | N/A |
| CA_n79_n41-n83 | 0.5 | 0.5 | N/A |
| CA_n79_n41-n95 | - | 0.5 | N/A |
| CA_n79_n41-n97 | - | 0.8 | N/A |
| CA_n79_n41-n98 | - | 0.5 | N/A |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 2: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE 3: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

7.3C.3.2.3	ΔRIB,c  for four bands

Table 7.3C.3.2.3-1: ΔRIB,c due to SUL (four bands)

| Band combination for SUL | ΔRIB,c for NR bands (dB)1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |  |  |
| CA_n28-n79_n41-n83 | 0.2 | - | 0.8 | N/A |
| CA_n28-n41_n79-n83 | 0.2 | - | 0.8 | N/A |
| CA_n41A-n95A_n79A-n98A | 0.5 | 0.5 | - | - |
| CA_n41A-n98A_n79A-n95A | 0.5 | 0.5 | - | - |
| CA_n41A-n83A_n79A-n98A | 0.5 | 0.5 | - | - |
| CA_n41A-n83A_n79A-n95A | 0.5 | 0.5 | - | - |
| NOTE 1: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE 2: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n28-n79_n41-n83 the order of band is n28, n41, n79 and n83. |  |  |  |  |

<<End of Change>>