3GPP TSG-RAN WG4 Meeting # 110                               R4-2401764

Greece, Athens, February 26 – March 1, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on MSD for CA_n78A-n104A

Agenda Item:	7.1.1.2

Document for:	Approval

Introduction

In the RAN4#109 meeting, a WF [1] for CA_n78-n104 has been approved with the following way forward.

TX/RX concurrency <Way forward>: - Companies are encouraged to provide analysis for Harmonic MSD, Harmonic Mixing MSD, and Cross-band MSD for simultaneous TX/RX - Companies are encouraged to evaluate ΔTIB and ΔRIB impacts using different RF filtering attenuation assumptions and the ΔTIB and ΔRIB difference between Simultaneous and non-simultaneous TX/RX RF filter frequency ranges <Way forward>: - n78 RF filter frequency range is 3300-4200 MHz in this analysis - n104 RF filter frequency range has three alternatives in this analysis - 5150-7125 MHz - 5925-7125 MHz - 6425-7125 MHz - Companies are encouraged to also discuss potential limitations in features and use cases related to antenna and RF front end architectures

In this paper, we’d like to provide our technical analysis on this band combination.

RF architecture

Since the frequency separation between band n78 (3300~3800MHz) and band n104 (6425~7125 MHz) is more than 2GHz, one possible way is to implement this combo by using separate antenna RF architecture as shown below for CA_n78-n104.

figure 1 Separate antenna RF architecture 1 for CA_n78-n104

In addition, the diversity chain can be shared between band n78 and n104 as shown in figure 2. This implementation may increase the difficulties of diversity antenna design, but UE can reduce the antenna number for real implementation.

figure 2 common antenna RF architecture 2 in diversity chain for CA_n78-n104

The third possible way is to share antenna in both main and diversity path for band n78-n104 as shown in figure 3. For non-simultaneous Rx/Tx capability, the challenge is from antenna design, which is similar to common antenna RF architecture 2 in diversity chain. However, if UE has to support simultaneous Rx/Tx capability, that means the duplexer between band n78 and n104 should be designed to achieve good isolation performance.

figure 3 common antenna RF architecture 3 in both main and diversity chain for CA_n78-n104

Generally, RAN4 assume 10~15 dB antenna isolation among different antennas. As the frequency range for both bands is higher than 3GHz and the frequency separation between two bands is larger than 2GHz, probably 15~20 dB antenna isolation between band n78 and n104 can be assumed. If common antenna RF architecture 3 is considered, the diplexer isolation between band n78 and n104 is not more than 10dB in main path.

Observation 1: Generally, there are three RF architecture for CA_n78-n104 at least. i.e. separate antenna RF architecture, common antenna in diversity path and common antenna in both main and diversity path. The antenna isolation could be assumed as 10~15dB. The diplexer isolation between band n78 and n104 is not more than 10dB in main path.

Filter assumption

Currently, both band n78 and n77 shared the same full frequency filter (3300~4200MHz) in the real UE implementation. In addition, most of band n77/n78 filter didn’t optimize the attenuation performance at frequency range 6425~7125MHz, so the attenuation performance can be assumed as 20~25dB at frequency range 6425~7125MHz.

Observation 2: for band n77/n78 filter, the attenuation performance can be assumed as 20~25dB at frequency range 6425~7125MHz.

For band n104, there are three alternatives based on the way forward in last meeting. We list the relative bandwidth as below.

Table 1 relative bandwidth analysis for different filter alternative

| Filter frequency range | 5150-7125 MHz | 5925-7125 MHz | 6425-7125 MHz | 3300~4200MHz |
| --- | --- | --- | --- | --- |
| Relative bandwidth | 32% | 18.3% | 10.3% | 24% |

It can be observed that Alt.1 (5150-7125 MHz) is the most difficult implementation for filter due to 32% relative bandwidth. Although the relative bandwidth of Alt.2 (5925-7125 MHz) are less than band n77/n78 filter (3300~4200MHz), quality factor (Q) values of filters for 6GHz are lower than 3GHz. It’s very hard to say that the filter implementation of Alt.2 (5925-7125 MHz) is easier than band n77/n78 filter (3300~4200MHz). However, it’s obvious that Alt.3 (6425-7125 MHz) is the easiest implementation for band n104 and good performance can be achieved more easily.

Observation 3: it’s obvious that Alt.3 (6425-7125 MHz) is the easiest implementation for band n104 and good performance can be achieved more easily than Alt 1 and Alt 2.

Currently, 3GPP specified band n104 (6425-7125 MHz) for IMT licensed service and band n96/n102 for unlicensed service. However, the maximum output power for band n104 and n96 are different. Generally, 20dBm/23dBm maximum output power for unlicensed band was specified. But for band n104, at least 26dBm will be implemented same as band n78/n77. Considering larger path loss due to higher frequency range, it’s precluded to implement 29dBm using single PA for band n104 in the future.

Based on the analysis above, the difference of MOP between licensed and unlicensed service will result the difference of power tolerance in the whole RF chain. Thus, it’s very hard to share the same filter between licensed band n104 and unlicensed band n96.

Observation 4: due to the RF specification difference of band licensed band n104 and unlicensed band n96, it’s very hard to share the same filter.

Observation 5: for band n104 filter, the attenuation performance can be assumed as 20~25dB at frequency range 3300~3800MHz.

MSD issues

Based on the way forward, the MSD issues should be analysed for simultaneous Rx/Tx operation. From network perspective, it’s very hard to guarantee non-simultaneous Rx/Tx operation for CA_n78-n104 due to the uncertainty of colocation deployment and TDD UL/DL pattern for both band n78 and n104.

Proposal 1: it’s necessary to consider the simultaneous Rx/Tx operation when CA_n78-n104 is specified.

The harmonic/harmonic mixing analysis is listed in table 2.

Table 2 harmonic/harmonic mixing analysis for CA_n78-n104

| UE UL/DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL/DL frequency (MHz) | 3300 | 3800 | 6425 | 7125 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6600 | 7600 | 12850 | 14250 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 9900 | 11400 | 19275 | 21375 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 13200 | 15200 | 25700 | 28500 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 16500 | 19000 | 32125 | 35625 |

Observation 6: due to the 2nd harmonic frequency of band n78 UL overlapping with DL band n104, the MSD due to 2nd harmonic interference should be investigated for band n104 DL.

Observation 7: due to the 2nd harmonic frequency of band n78 DL overlapping with UL band n104, the MSD due to 2nd harmonic mixing interference should be investigated for band n78 DL.

In addition, since the filter isolation (25dB) between band n78 and n104 is not enough, the MSD due to cross band isolation for DL band n78 and DL band n104 should be investigated considering both filter isolation and antenna isolation.

The following diagram shows the relationship between n78 frequency range and n104 frequency range.

Figure 4 The frequency range relationship between n78 and n104

If the carriers are deployed in region A for band n78 and region D for band n104, the harmonic/harmonic mixing interference could happen. However, if one carrier is deployed in region B or region C, the harmonic/harmonic mixing interference will not exist.

Before we go through the specific MSD analysis, it’s better to align the assumption in working group.

Proposal 2: To consider the following assumption for CA_n78-n104 MSD analysis.

1) MSD due to cross band isolation:

PA output noise PSD for both n78 and n104:  -120~-130dBm/Hz

Both n78 and n104 filter attenuation: 25dB

Antenna isolation: 10dB

Diplexer isolation between band n78 and n104 is no more than 10dB in main path.

2) MSD due to harmonic interference:

n78 PA 2nd harmonic attenuation:  30dB

Both n78 and n104 filter attenuation: 25dB

Antenna isolation: 10dB

Diplexer isolation between band n78 and n104 is no more than 10dB in main path.

3) MSD due to harmonic mixing interference:

n78 LO 2nd harmonic attenuation:  50dB

Both n78 and n104 filter attenuation: 25dB

Antenna isolation: 10dB

Diplexer isolation between band n78 and n104 is no more than 10dB in main path.

It’s highlighted that the minimum channel bandwidth for band n104 is 20MHz and the minimum channel bandwidth for band n78 is 10MHz. The following test configurations can be considered.

Proposal 3: To consider the following test configurations for CA_n78-n104 MSD analysis.

1) MSD due to cross band isolation:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | n104 | 3750 | 100 | 30 | 270 (RBstart=0) | 6435 | 20 | TBD | >ACLR2 |
| n104 | n78 | 6475 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | TBD | >ACLR2 |

2) MSD due to harmonic interference:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n104 | 10 | 30 | 24 (RBstart=0) | 20 | TBD | NOTE 2 | UL2/DL1 direct-hit |

3) MSD due to harmonic mixing interference:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n104 | n78 | 20 | 30 | 50 (RBstart=0) | 20 | TBD | NOTE 1 | UL1/DL2 |

- Summary

Observation 1: Generally, there are three RF architecture for CA_n78-n104 at least. i.e. separate antenna RF architecture, common antenna in diversity path and common antenna in both main and diversity path. The antenna isolation could be assumed as 10~15dB. The diplexer isolation between band n78 and n104 is not more than 10dB in main path.

Observation 2: for band n77/n78 filter, the attenuation performance can be assumed as 20~25dB at frequency range 6425~7125MHz.

Observation 3: it’s obvious that Alt.3 (6425-7125 MHz) is the easiest implementation for band n104 and good performance can be achieved more easily than Alt 1 and Alt 2.

Observation 4: due to the RF specification difference of band licensed band n104 and unlicensed band n96, it’s very hard to share the same filter.

Observation 5: for band n104 filter, the attenuation performance can be assumed as 20~25dB at frequency range 3300~3800MHz.

Proposal 1: it’s necessary to consider the simultaneous Rx/Tx operation when CA_n78-n104 is specified.

Observation 6: due to the 2nd harmonic frequency of band n78 UL overlapping with DL band n104, the MSD due to 2nd harmonic interference should be investigated for band n104 DL.

Observation 7: due to the 2nd harmonic frequency of band n78 DL overlapping with UL band n104, the MSD due to 2nd harmonic mixing interference should be investigated for band n78 DL.

Proposal 2: To consider the following assumption for CA_n78-n104 MSD analysis.

1) MSD due to cross band isolation:

PA output noise PSD for both n78 and n104:  -120~-130dBm/Hz

Both n78 and n104 filter attenuation: 25dB

Antenna isolation: 10dB

Diplexer isolation between band n78 and n104 is no more than 10dB in main path.

2) MSD due to harmonic interference:

n78 PA 2nd harmonic attenuation:  30dB

Both n78 and n104 filter attenuation: 25dB

Antenna isolation: 10dB

Diplexer isolation between band n78 and n104 is no more than 10dB in main path.

3) MSD due to harmonic mixing interference:

n78 LO 2nd harmonic attenuation:  50dB

Both n78 and n104 filter attenuation: 25dB

Antenna isolation: 10dB

Diplexer isolation between band n78 and n104 is no more than 10dB in main path.

Proposal 3: To consider the following test configurations for CA_n78-n104 MSD analysis.

1) MSD due to cross band isolation:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | n104 | 3750 | 100 | 30 | 270 (RBstart=0) | 6435 | 20 | TBD | >ACLR2 |
| n104 | n78 | 6475 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | TBD | >ACLR2 |

2) MSD due to harmonic interference:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n104 | 10 | 30 | 24 (RBstart=0) | 20 | TBD | NOTE 2 | UL2/DL1 direct-hit |

3) MSD due to harmonic mixing interference:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n104 | n78 | 20 | 30 | 50 (RBstart=0) | 20 | TBD | NOTE 1 | UL1/DL2 |

- References

[1] R4-2321930, WF on CA_n78A-n104A, Qualcomm France