# 3GPP TSG-RAN WG4 Meeting #110	R4-2401810

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda Item:	8.22.1.1.2

Source: 	OPPO

Title: 	MPR results for PSFCH

Document for:	Approval

- Introduction

During the RAN4#109 meeting, the MPR and A-MPR for all the channels have been agreed. Due to time constrain, the MPR results for PSFCH from OPPO has not been finished and submitted at that time. In this paper, we submit our MPR results for PSFCH and after comparison it is found the results are aligned with the agreed number. Hence a further TP to TR is proposed to capture the results.

- Discussion

The SL-U MPR for PSFCH has been agreed as:

For PSFCH transmission with single RB set the allowed MPR for the maximum output power is 10dB for power class 5 NR sidelink UE.

For PSFCH transmission with multiple RB sets the allowed MPR for the maximum output power is specified in Table 6.2E.2F-4 for power class 5 NR sidelink UE.

Table 6.2E.2F-4 Maximum power reduction (MPR) for PSFCH transmission for NR SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 10.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 6.2E.2F-3 apply. |  |  |

For the PSFCH MPR simulation, it has been fully discussed during the last meeting and hence we would like to directly provide our results and to compare with the agreed PSFCH values.

For the PSFCH MPR, the simulation cases are listed as below table 1.

Table 1 simulation cases

|  | case | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 | 105RB0 | 30 |
|  | 2 | CP-OFDM | 40 | 50RB0 | 15 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 30 |
|  | 4 | CP-OFDM | 60 | 105RB0 | 30 |
| Wide band operation Interlaced RB allocation | 5 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 6 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 7 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 8 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 9 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 10 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 11 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 12 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 13 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 14 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 15 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 16 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 17 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 18 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 19 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 20 | CP-OFDM | 100 | Bitmap 00100 | 30 |

Table 2 MPR simulation result for PSFCH single CC

| Single CC interlace | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| QPSK | 10.24 | 13.82 | 12.98 | 15.60 |

Table 3 MPR simulation result for PSFCH Wideband Interlace

| Wideband Interlace | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 9.41 | 9.98 | 13.16 | 9.33 | 12.49 | 9.41 | 14.28 | 9.98 |
| Wideband Interlace | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| QPSK | 13.07 | 9.82 | 12.98 | 14.86 | 9.73 | 12.73 | 14.52 | 9.65 |

From table 2 and table 3 above, it can be observed that for larger channel bandwidth that larger MPR is needed. There is related to the P-APR of multiple repeated sub-channels which is similar also to the S-SSB MPR simulation result.

To also account for some implementation margin, it can be observed that the MPR simulation result of OPPO’s are aligned with the MPR requirement defined in the RAN4#109 meeting.

Observation:  The MPR simulation result of OPPO’s are aligned with the MPR requirement defined in the RAN4#109 meeting.

Proposal: To capture the simulation results table 2 and table 3 into TR 38.786.

3	Conclusions

Observation:  The MPR simulation result of OPPO’s are aligned with the MPR requirement defined in the RAN4#109 meeting.

Proposal : To capture the simulation results into TR 38.786.

4 References