# 3GPP TSG-RAN WG4 Meeting #110	R4-2401830

# Athens, Greece, 26th Feb – 1st March, 2024

Title:	Reply LS to RAN1 on the system parameters for NTN above 10GHz

Response to:	R1-2312553

Release:	Rel-18

Work Item:	NR_NTN_enh_Core

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:

Contact Person:

Name:  Derrick Chao

E-mail Address:  derrick.chao@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN4 would like to thank RAN1 for the LS on R1-2312553. RAN4 discussed the questions asked by RAN1 and has the following answers.

In the previous meeting, we RAN4 RRM has already discussed the timing requirements and we can list the reached timing requirements as below:

### Te_NTN for 60kHz and 120kHz Agreement: - UL 60kHz SCS: - 13 Ts for all cases with 120kHz/240kHz SSB - UL 120kHz SCS: - Case 1 and case 2: 7.5 Ts - FFS for the applicable side condition on case 2 - Case 3: Higher than 7.5 Ts, FFS for the exact value ### NTA-offset Agreement: - RAN4 to define the exact value of NTA,offset for NR NTN band above 10 GHz. - Option 1: the value of NTA-offset defined in Table 7.1.2-2 for FR2 - Other options are not precluded. ### UE Timing Advance adjustment accuracy Agreement: - RAN4 to define the timing advance adjustment accuracy requirement for NR NTN band above 10 GHz. The values for 60kHz and 120kHz UL SCSs are the same as those defined in Table 7.3A.2.2-1. ### UL timer accuracy requirements Agreement: - RAN4 to define the UL timer accuracy requirements for NR NTN band above 10 GHz. The values are the same as those defined in Table 7.2C.2-1. ### Gradual timing adjustment requirements Agreement: - RAN4 to define the gradual timing adjustment requirements for NR NTN band above 10 GHz. The values are the same as those defined in Table 7.1.2.1-1.

We just discussed the Te, UE timing advance adjustment, UL timer and gradual timing adjustment requirements. However, the Te requirements for UL SCS=120kHz is still under consideration and the cases on the above table can be seen as below:

Case-1: Stationary UE for GSO

Case-2: Stationary UE for LEO

Case-3: Mobile UE for GSO

And all the tables on above are contained in TS 38.133.

# 2. Actions:

To RAN1:

ACTION: RAN4 respectfully ask RAN1 to take above RAN4 responses into consideration.

# 3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #110	26 Feb - 01 Mar 2024          Athens , GR

TSG-RAN WG4 Meeting #110-bis	15 - 19  April 2024            Changsha , CN