3GPP TSG-RAN WG4 Meeting #110	R4-2401832

Athens, Greece 26th Feb – 1st May 2024

Source: 	Nokia, Nokia Shanghai Bell

Title: 					FWA PC1 n7 NS_46 A-MPR proposal

Agenda Item: 			7.15.2

Document for:	Approval

# 1	Introduction

In this contribution we present simulations results and A-MPR proposal for NR band n7 and power class PC1. A-MPR is required due to the additional spurious emission mask signaled with NS_46. The proposal is also captured in a CR [3].

# 2	Simulation assumptions and scenario

- Operating band n7 (2500 MHz – 2570 MHz)

- Power class PC1

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc, CIM5 -70 dBc

- Only additional spurious emission mask is simulated. Other gating factors (ACLR, SEM, etc.) are excluded.

- Additional spurious emission mask for NS_46:

Table 6.5.3.3.25-1: Additional requirements for “NS_46”

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1, 2 |
| Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1, 2 |
| Frequency range | 2595 | - | 2620 | -40 | 1 | 1 |
| NOTE 1: This requirement is applicable for all carriers confined in 2500-2570 MHz. Sepcial restrictions apply for channel bandwidths up to 20MHz: For carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB with the minimum supported SCS of 15KHz. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

# 3	Simulation results

In this section we provide a brief description of the simulation results. In the appendix we provide a complete set of the simulated triangles at the center frequencies relevant for our proposal. The simulation results differ slightly from our previous contribution [1] because more gating factors (ACLR etc.) were simulated there and the backoff due to such gating factors exceeded the MPR due to issues in the calibration configuration [2].

5 MHz and 10 MHz CBWs do not require A-MPR. Furthermore, A-MPR can be avoided for certain center frequencies for up to 30 MHz CBW, as detailed in Table 1.

Table 1: The center frequencies where A-MPR is not needed according to simulations. For higher CBWs A-MPR cannot be avoided anywhere in the operating band. Lower CBWs do not need A-MPR anywhere in the operating band.

| Channel Bandwidth [MHz] | Fc, noAMPR [MHz] |
| --- | --- |
| 15 | ≤ 2559.84 |
| 20 | ≤ 2547.67 |
| 25 | ≤ 2535.50 |
| 30 | ≤ 2523.35 |

The largest backoff values in the lower left corners of the triangles are due to CIM3 hitting the -40 dBm mask segment and they can reach up to 25 dB for OFDMA. In Fig. 1 we provide examples of simulated A-MPR for 50 MHz channel at both lower and upper edge of the operating band.

Fc = 2545 MHz

Fc = 2525 MHz

Figure 1: Examples of the simulated backoff to satisfy the additional spurious emission limit. The triangles in the upper row are for the upper edge of the operating band while the figures in the lower row are for the lower edge.

# 4	A-MPR proposal

Based on our simulation results we propose A-MPR shown in tables 2 and 3. In Fig. 2 we show examples of the proposed A-MPR as triangles for 50 MHz channel at both lower and upper edge of the operating band. In the appendix we provide a complete set of the proposal triangles.

Proposal 1: RAN4 shall consider defining A-MPR for n7 PC1 due to NS_46 as shown in tables 2 and 3.

Table 2: The proposed A-MPR regions.

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 15 MHz | Fc > 2559.5 | ≥ 11.8 | ≥ -1.8*12*SCS*RBend + 32.5 | A1 |
|  |  | ≥ 13.3 | ≤ 1.62 | A2 |
| 20 MHz | Fc > 2547.5 | ≥ 15.3 | ≥ -1.5*12*SCS*RBend + 36.5 | A1 |
|  |  | ≥ 15 | ≤ 5.7 | A2 |
| 25 MHz | Fc > 2547.5 | ≥ 17 | ≥ -1.5*12*SCS*RBend + 43.5 | A3 |
|  |  | ≥ 18.5 | ≤ 6.7 | A2 |
|  | 2535.5 < Fc ≤ 2547.5 | ≥ 21.6 | ≥ 21.6 | A5 |
|  |  | ≥ 20.5 | ≤ 5.6 | A2 |
| 30 MHz | 2523 < Fc ≤ 2545 | ≥ 22.5 | ≥ -0.7*12*SCS*RBend + 41.0 | A5 |
|  |  | ≥ 23.4 | ≤ 7.2 | A2 |
|  | Fc > 2545 | ≥ 20.0 | ≥ -1.5*12*SCS*RBend + 50.5 | A8 |
|  |  | ≥ 21.6 | ≤ 7.2 | A9 |
|  |  | ≤ 9.6 | ≥ 1.1*12*SCS*RBend - 1.5 | A7 |
| 35 MHz | Fc ≤ 2532.5 | ≥ 28 | ≤ 7.2 | A2 |
|  | 2532.5 < Fc ≤ 2542.5 | ≥ 27.7 | ≥ -1.2*12*SCS*RBend + 59.4 | A4 |
|  |  | ≥ 24.5 | ≤ 7.2 | A2 |
|  | Fc > 2542.5 | ≤ 18.4 | ≥ 1.15*12*SCS*RBend – 3.4 | A7 |
|  |  | ≥ 21.6 | ≥ 9, ≥ -1.2*12*SCS*RBend + 47 | A8 |
|  |  | ≥ 23 | < 9 | A9 |
| 40 MHz | Fc > 2540 | ≤ 24 | ≥ 1.15*12*SCS*RBend - 6 | A7 |
|  |  | > 24, < 35 | ≥ -0.75*12*SCS*RBend + 40.5 | A9 |
|  |  | ≥ 34 | > 7.2 | A9 |
|  |  | ≥ 27 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 27 | ≤ 2.2 | A9 |
|  | 2530 < Fc ≤ 2540 | ≤ 10.8 | ≥ 1.2*12*SCS*RBend – 2 | A7 |
|  |  | ≥ 29 | ≤ 7.2 | A9 |
|  |  | ≥ 28 | ≥ -1*12*SCS*RBend + 56 | A10 |
|  | Fc ≤ 2530 | ≥ 30 | ≤ 7.2 | A2 |
| 50 MHz | Fc > 2535 | ≤ 19 | ≥ 1.17*12*SCS*RBend – 10 | A7 |
|  |  | > 19, ≤ 29.5 | ≥ 1.17*12*SCS*RBend – 10 | A11 |
|  |  | > 29.5 | > 12.8, ≥ -1.15*12*SCS*RBend + 60 | A9 |
|  |  | > 41 | > 7.2, ≤ 12.8 | A12 |
|  |  | ≥ 33 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 33 | ≤ 2.2 | A9 |
|  | 2525 < Fc ≤ 2535 | ≤ 14 | ≥ 1.17*12*SCS*RBend – 6 | A7 |
|  |  | > 14, ≤ 26 | ≥ 1.17*12*SCS*RBend – 6 | A11 |
|  |  | ≥ 34 | > 22.5, ≥ -1.25*12*SCS*RBend + 77.5 | A8 |
|  |  | ≥ 43 | > 7.2, ≤ 22.5 | A12 |
|  |  | ≥ 35 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 35 | ≤ 2.2 | A9 |
|  | Fc = 2525 | ≥ 47 | ≥ -1*12*SCS*RBend + 94 | A13 |
|  |  | ≤ 9 | ≥ 1.14*12*SCS*RBend – 2 | A7 |
|  |  | ≥ 37.4 | > 2.2, ≤ 7.6 | A8 |
|  |  | ≥ 37.4 | ≤ 2.2 | A9 |

Table 3: The proposed A-MPR values.

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 | A11 | A12 | A13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 2 |  | 2 | 1 | 1 | 3 | 6.5 | 5 | 6 | 4 |  | 4 | 2.5 |
|  | QPSK | 3 | 4 | 3 | 2 | 2 | 6 | 11.5 | 6 | 6 | 5 |  | 4 | 4 |
|  | 16 QAM | 3 | 6.5 | 3 |  |  | 6 | 15 | 6 | 6.5 | 5 |  | 4 | 4 |
|  | 64 QAM | 3 | 7 | 3 |  |  | 6 | 16 | 6 | 7 | 5 |  | 4 | 4 |
|  | 256 QAM |  | 7 |  |  |  | 6 | 16.5 | 6 | 7 | 5 |  |  |  |
| CP-OFDM | QPSK | 4 | 12.5 | 6 | 4 |  | 6.5 | 25 | 9 | 12.5 | 5.5 | 10 | 5 | 4 |
|  | 16 QAM | 4 | 12.5 | 6 | 4 |  | 6.5 | 25 | 9 | 13 | 5.5 | 10 | 5 | 4 |
|  | 64 QAM | 4 | 13 | 6 |  |  | 6.5 | 25 | 9 | 13 | 5.5 | 10 | 5 | 4 |
|  | 256 QAM |  | 13 |  |  |  |  | 25 | 9 | 13 |  | 10 |  |  |

Fc = 2525 MHz

Fc = 2545 MHz

Figure 2: Examples of the proposed A-MPR. The triangles in the upper row are for the upper edge of the operating band while the figures in the lower row are for the lower edge.

# 5	Conclusions

We have provided A-MPR simulation results for PC1 n7 taking into account NS_46 and we have made a proposal based on the results. The proposal is also captured in a CR [3]. Complete sets of the simulated and proposed A-MPR triangles are presented in the appendix.

Proposal 1: RAN4 shall consider defining A-MPR for n7 PC1 due to NS_46 as shown in tables 2 and 3.

Table 2: The proposed A-MPR regions.

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 15 MHz | Fc > 2559.5 | ≥ 11.8 | ≥ -1.8*12*SCS*RBend + 32.5 | A1 |
|  |  | ≥ 13.3 | ≤ 1.62 | A2 |
| 20 MHz | Fc > 2547.5 | ≥ 15.3 | ≥ -1.5*12*SCS*RBend + 36.5 | A1 |
|  |  | ≥ 15 | ≤ 5.7 | A2 |
| 25 MHz | Fc > 2547.5 | ≥ 17 | ≥ -1.5*12*SCS*RBend + 43.5 | A3 |
|  |  | ≥ 18.5 | ≤ 6.7 | A2 |
|  | 2535.5 < Fc ≤ 2547.5 | ≥ 21.6 | ≥ 21.6 | A5 |
|  |  | ≥ 20.5 | ≤ 5.6 | A2 |
| 30 MHz | 2523 < Fc ≤ 2545 | ≥ 22.5 | ≥ -0.7*12*SCS*RBend + 41.0 | A5 |
|  |  | ≥ 23.4 | ≤ 7.2 | A2 |
|  | Fc > 2545 | ≥ 20.0 | ≥ -1.5*12*SCS*RBend + 50.5 | A8 |
|  |  | ≥ 21.6 | ≤ 7.2 | A9 |
|  |  | ≤ 9.6 | ≥ 1.1*12*SCS*RBend - 1.5 | A7 |
| 35 MHz | Fc ≤ 2532.5 | ≥ 28 | ≤ 7.2 | A2 |
|  | 2532.5 < Fc ≤ 2542.5 | ≥ 27.7 | ≥ -1.2*12*SCS*RBend + 59.4 | A4 |
|  |  | ≥ 24.5 | ≤ 7.2 | A2 |
|  | Fc > 2542.5 | ≤ 18.4 | ≥ 1.15*12*SCS*RBend – 3.4 | A7 |
|  |  | ≥ 21.6 | ≥ 9, ≥ -1.2*12*SCS*RBend + 47 | A8 |
|  |  | ≥ 23 | < 9 | A9 |
| 40 MHz | Fc > 2540 | ≤ 24 | ≥ 1.15*12*SCS*RBend - 6 | A7 |
|  |  | > 24, < 35 | ≥ -0.75*12*SCS*RBend + 40.5 | A9 |
|  |  | ≥ 34 | > 7.2 | A9 |
|  |  | ≥ 27 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 27 | ≤ 2.2 | A9 |
|  | 2530 < Fc ≤ 2540 | ≤ 10.8 | ≥ 1.2*12*SCS*RBend – 2 | A7 |
|  |  | ≥ 29 | ≤ 7.2 | A9 |
|  |  | ≥ 28 | ≥ -1*12*SCS*RBend + 56 | A10 |
|  | Fc ≤ 2530 | ≥ 30 | ≤ 7.2 | A2 |
| 50 MHz | Fc > 2535 | ≤ 19 | ≥ 1.17*12*SCS*RBend – 10 | A7 |
|  |  | > 19, ≤ 29.5 | ≥ 1.17*12*SCS*RBend – 10 | A11 |
|  |  | > 29.5 | > 12.8, ≥ -1.15*12*SCS*RBend + 60 | A9 |
|  |  | > 41 | > 7.2, ≤ 12.8 | A12 |
|  |  | ≥ 33 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 33 | ≤ 2.2 | A9 |
|  | 2525 < Fc ≤ 2535 | ≤ 14 | ≥ 1.17*12*SCS*RBend – 6 | A7 |
|  |  | > 14, ≤ 26 | ≥ 1.17*12*SCS*RBend – 6 | A11 |
|  |  | ≥ 34 | > 22.5, ≥ -1.25*12*SCS*RBend + 77.5 | A8 |
|  |  | ≥ 43 | > 7.2, ≤ 22.5 | A12 |
|  |  | ≥ 35 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 35 | ≤ 2.2 | A9 |
|  | Fc = 2525 | ≥ 47 | ≥ -1*12*SCS*RBend + 94 | A13 |
|  |  | ≤ 9 | ≥ 1.14*12*SCS*RBend – 2 | A7 |
|  |  | ≥ 37.4 | > 2.2, ≤ 7.6 | A8 |
|  |  | ≥ 37.4 | ≤ 2.2 | A9 |

Table 3: The proposed A-MPR values.

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 | A11 | A12 | A13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 2 |  | 2 | 1 | 1 | 3 | 6.5 | 5 | 6 | 4 |  | 4 | 2.5 |
|  | QPSK | 3 | 4 | 3 | 2 | 2 | 6 | 11.5 | 6 | 6 | 5 |  | 4 | 4 |
|  | 16 QAM | 3 | 6.5 | 3 |  |  | 6 | 15 | 6 | 6.5 | 5 |  | 4 | 4 |
|  | 64 QAM | 3 | 7 | 3 |  |  | 6 | 16 | 6 | 7 | 5 |  | 4 | 4 |
|  | 256 QAM |  | 7 |  |  |  | 6 | 16.5 | 6 | 7 | 5 |  |  |  |
| CP-OFDM | QPSK | 4 | 12.5 | 6 | 4 |  | 6.5 | 25 | 9 | 12.5 | 5.5 | 10 | 5 | 4 |
|  | 16 QAM | 4 | 12.5 | 6 | 4 |  | 6.5 | 25 | 9 | 13 | 5.5 | 10 | 5 | 4 |
|  | 64 QAM | 4 | 13 | 6 |  |  | 6.5 | 25 | 9 | 13 | 5.5 | 10 | 5 | 4 |
|  | 256 QAM |  | 13 |  |  |  |  | 25 | 9 | 13 |  | 10 |  |  |

# 6	References

[1] R4-2316886, FWA PC1 n7 NS_46 A-MPR simulation results, Nokia

[2] R4-2311151, PA Calibration using 40MHz CBW waveforms for FR1, Skyworks

[3] R4-2400617, draftCR for 38.101-1 addition of PC1 operation for n7

# 7	Appendix: A-MPR triangles

### 7.1	Simulation results

Note that for other CBWs than 50 MHz the backoff search was performed only up to 20 dB. This only affects the CIM3 region for OFDMA. In the proposal the 25 dB A-MPR for OFDMA CIM3 region is used for all CBWs.

### 7.2	Proposal