3GPP TSG-RAN WG4 Meeting #110	R4-2401868

# Athens, Greece, February 26 – March 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RRM performance requirements for Pre-MG, concurrent MG and NCSG

Agenda item:	8.5.3

Document for:	Discussion

Introduction

This paper presents our view on RRM aspects related to further enhancements of measurement gaps for Rel-18 [1]. In particular, it presents our view on RRM performance requirements for Case 1 and Case 2 requirements, i.e., the gap combinations

- concurrent MG with Pre-MG

- Pre-MG + Pre-MG

- concurrent MG with NCSG and

- NCSG + NCSG.

Discussion

RAN4 #109 has discussed test cases for Case 1 and Case 2 requirements and has listed open issues in [2] [3]. These are discussed in the following.

Performance principles for Case 1 and Case 2

RAN4 discussed the following issues related to performance principles for Case 1 and Case 2 requirements.

Issue 5-1-1: Which general configuration shall be defined for the test cases? < Way Forward > - FFS the options: - Option 1: - Only define test case in NR SA in both FR1 and FR2: support - Do not introduce the test for L1 impact: support - Do not introduce test cases for intra-freq measurement without gap: [Not support, both intra-frequency inter-frequency, and deactivated SCell measurement shall be introduced] - Define a minimum set of test cases for SSB-based measurement: support - Only define test case under non-DRX: support - Define test case without SBI reporting: support - On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations: support - Do not define test cases with simultaneously FR1 and FR2 gaps configured: support - Test cases are limited to single serving carrier: support - Only use mandatory gap patterns to define test cases: support - Focus on only fully non-overlp and partially partial overlap in the test case design: support - Verify gap dropping behaviour without introducing additional test cases: support - Option 2: - Only inter-frequency measurement - Only non-DRX - without SSB time index detection - Only define test case in NR SA in both FR1 and FR2 - Do not introduce the test for L1 impact - Do not introduce test cases for intra-freq measurement without gap - Define a minimum set of test cases for SSB-based measurement - On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations - Do not define test cases with simultaneously FR1 and FR2 gaps configured. - Test cases are limited to single serving carrier - Only use mandatory gap patterns to define test cases - Focus on only partially partial overlap in the test case design - Option 3: - Only define test case for SA mode; - No need to introduce test case for intra-frequency without gap; - Only define test case of non-DRX; - Only consider per-UE + per-UE MGs or per-FR + per-FR MGs configuration - Option 4: - It is necessary to define the test cases for NR standalone scenario only; - Test cases for SSB measurement with Pre-MG shall be defined at least - Test cases for the intra-frequency with Pre-MG gap can be defined with high priority - In Rel18, the test cases under single carrier when the pre-MG activation is based on UE autonomous pre-MG activation shall be defined - BWP switching trigger event can be tested ONLY - Non-DRX cases will be tested only in Rel18 - Only the test case when the colliding happened needs to be defined. And in the test case for colliding, the separated sub test cases with different Pre-MG priority can be considered

In our view, there are some commonalities between these options, however option 1 is most detailed and hence can serve as baseline. In addition, we propose to limit the tests to BWP switching trigger events for Pre-MG activation and deactivation procedures, as proposed in option 4.

The following proposal is made.

RAN4 to proceed with the following baseline for general configuration for NR_MG_enh2 tests:

    - Only define test case in NR SA in both FR1 and FR2

    - Do not introduce the test for L1 impact

    - Introduce test cases for both intra-frequency, inter-frequency, and deactivated SCell measurement

    - Define a minimum set of test cases for SSB-based measurement

    - Only define test case under non-DRX

    - Define test case without SBI reporting

    - On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations

    - Do not define test cases with simultaneously FR1 and FR2 gaps configured

    - Test cases are limited to single serving carrier

    - Only use mandatory gap patterns to define test cases

    - Focus on only fully non-overlap and partially partial overlap in the test case design

    - Verify gap dropping behaviour without introducing additional test cases

    - Limit the tests to BWP switching trigger events for Pre-MG activation and deactivation procedures.

RAN4 further discussed the related issue on the testing procedure.

Issue 5-1-2: Testing procedure < Way Forward > - FFS the options: - Option 1: - RAN4 can use the testing procedure below to reduce the overall testing efforts for Pre-MG and Concurrent MGs testing.

The testing procedure for measurements by Pre-MG which is transited from deactivation to activation can consist of three successive time periods, with time durations of T1, T2 and T3 respectively. - During the duration of T1, UE can be configured with Pre-MG but being deactivated and the other Type2 MG within concurrent MGs. The configuration of them can be: - The periodicity of Pre-MG and Type2 MG can be Ttype2MG =2* TpreMG - The occasion of Type2 MG can be fully overlapped with these of Pre-MG - Pre-MG activated has higher priority than that of Type2 MG - At the start of time duration T1, Pre-MG was configured but not activated. The measurements with Type2 MG will be prioritized while colliding with Pre-MG deactivated and the reporting delay will be tested. The measurement results by Pre-MG deactivated will be reported based on the occasions which is not collided with other Pre-MG/MGs. - At the start of time duration T2, the serving gNB can trigger Pre-MG activation. And UE is expected to complete the Pre-MG activation within T2. - The dynamic collision rule shall be verified within T2. For an example, the measurement by Type2 MG will be kept during T2. - At the start of time duration T3, Pre-MG shall be activated. - During T3, Type2 MG will be dropped because it was collided with the activated Pre-MGs. The measurement by Pre-MG activated will be tested only. The testing procedure for measurements by Pre-MG which is transited from activation to deactivation can also consist of three successive time periods, with time durations of T1, T2 and T3 respectively. - During the duration of T1, UE can be configured with Pre-MG but being activated and the other Type2 MG within concurrent MGs. The configuration of them can be: - The periodicity of Pre-MG and Type2 MG can be Ttype2MG =2* TpreMG - The occasions of Type2 MG can be fully overlapped with these of Pre-MG - Pre-MG activated has higher priority than that of Type2 MG - At the start of time duration T1, Pre-MG was configured and activated. The measurements with Pre-MG will be prioritized when colliding with other MGs and the reporting delay will be tested. The measurements with Pre-MG deactivated will be reported based on the occasion which is not collided with other MGs. - At the start of time duration T2, the serving gNB can trigger Pre-MG activation. And UE is expected to complete the Pre-MG activation within T2. - The dynamic collision rule defined in Rel18 can be verified within T2 - During T2, Pre-MG can be deactivated. - During T3, the deactivated Pre-MGs collide Type2 MGs will be dropped. - The gapless measurement on MO which is associated with Pre-MG will be tested ONLY.

In our view, the proposed testing procedure in option 1 can be a candidate for verifying Case 1 requirements. However, more detailed discussion is needed once the scenarios are agreed (e.g. fully non-overlapped gaps, partially partial overlapped gaps, etc.). Hence it is too early to agree the above testing procedure for Pre-MG activation / deactivation.

The following proposal is made.

Revisit suitability of option 1 of issue 5-1-2 for testing Pre-MG activation/deactivation once the scenarios for Case 1 test cases are agreed.

Test cases for Case 1

RAN4#109 further discussed Case 1 test cases:

Issue 5-2-1: [Case 1] Which topics shall RAN4 RRM define test cases for Pre-MG and concurrent MG (Case 1) < Way Forward > - FFS the options: - Option 1: - Pre-MG + Pre-MG, Pre-MG + Type-2 MG and Pre-MG + Type-1 MG. - Option 2: - Pre-MG + Pre-MG, Pre-MG + Type-2 MG. - Option 3: - Concurrent MG and 1 Pre-MG activation / deactivation in same FR - Pre-MGs with activation / deactivation in the same FR. - Option 4: - Prioritize testing Type-2 + Pre-MG - If Pre-MG + Pre-MG is tested, prioritize test scenarios without dynamic collisions - Deprioritize test scenarios with dynamic collisions - If dynamic collisions are tested, test them with Type-2 + Pre-MG - Configure only two MGs in each test case - Option 5: - Pre-MG (de)activation - Pre-MG (de)activation is based on the associated MO or the associated pre-configured status - In non-simultaneous case, the (de)activation delay is same as in Rel-17 - In simultaneous case, the (de)activation delay is extended by 2ms - Collision handling - Deactivated pre-MG is not considered in collision - Dynamic collision including at least scenario 1 and 2

We do not support option 1, as combination Pre-MG + Type-1 MG was considered to not require any testing. Hence, we support option 2. We also support option 3 that both Type-2 MG and Pre-MG as well as both Pre-MGs should be in same FR. Regarding option 4, we disagree and believe test scenarios should include dynamic collisions, for both gap combinations Type 2 MG + Pre-MG and for Pre-MG + Pre-MG. In any case, only two MGs should be tested. Regarding option 5, we agree with the proposals except for dynamic collisions. These should be tested for Pre-MG (de)activation with higher and lower priority of Pre-MG vs. Type-2 MG. In addition, both network-controlled and UE-autonomous Pre-MG (de)activation should be verified.

The following proposal is made.

RAN4 to proceed with following test cases for Case 1.

    - Pre-MG + Pre-MG, Pre-MG + Type-2 MG. Hence, configure only two MGs in each test case.

    - Concurrent MG and 1 Pre-MG activation / deactivation in same FR

    - Pre-MGs with activation / deactivation in the same FR.

    - Pre-MG (de)activation is based on the associated MO or the associated pre-configured status

    - For Pre-MG (de)activation, assume either higher or lower priority of Pre-MG vs. Type-2 MG.

    - Design test scenarios without and with dynamic collisions, for both gap combinations Type 2 MG + Pre-MG and for Pre-MG + Pre-MG.

    - For some scenarios test network-controlled Pre-MG (de)activation, for other test UE-autonomous Pre-MG (de)activation.

    - Test the (de)activation delay for simultaneous and non-simultaneous Pre-MG (de)activation.

Test cases for Case 2

RAN4#109 also discussed Case 2 test cases:

Issue 5-3-1: [Case 2] Which topics shall RAN4 RRM define test cases for NCSG and concurrent MG (Case 2) < Way Forward > - FFS the options: - Option 1: - NCSG + NCSG, NCSG + Type-2 MG and NCSG + Type-1 MG. - Option 2: - NCSG + NCSG, NCSG + Type-2 MG. - Option 3: - 1 Concurrent MG and 1 NCSG in the same FR - 2 NCSGs (non-overlapping) in the same FR - Measurement for deactivated SCells with 1 Concurrent MG (w/o gap or with NCSG, as being discussed for Rel-17 maintenance). - Option 4: - Collision definition - Total NCSG duration, including both the VILs and the ML are considered in the proximity - [Deactivated SCell measurement - Deactivated SCell is measured in NCSG regardless of the gap association] - Option 5: - Prioritize testing NCSG + Pre-MG

We do not support option 1, as combination NCSG + Type-1 MG was considered to not require any testing. Hence, we support option 2. We also support option 3 that the gap combinations should be in the same FR. For measurement of deactivated SCells in option 3 and 4, we propose to wait on the conclusion of the Rel-17 discussion. In our understanding the collision definition in option 4 has already been used in the normative work in clause 9.1.13. It can be used for collision testing. Option 5 is outside the Rel-18 scope.

The following proposal is made.

RAN4 to proceed with following test cases for Case 2:

    - NCSG + NCSG, NCSG + Type-2 MG. Hence, configure only two MGs in each test case.

    - 1 Concurrent MG and 1 NCSG in the same FR

    - 2 NCSGs (non-overlapping) in the same FR

    - For collision test scenarios, test the proximity condition.

    - The use of concurrent MG and/or NCSG for measurement of deactivated SCells depends on the outcome of the Rel-17 discussion.

Conclusion

In this paper, our views on open issues related to RRM performance requirements for Case 1 and Case 2 requirements [2] [3] are presented.

The following proposals are made.

- RAN4 to proceed with the following baseline for general configuration for NR_MG_enh2 tests:

    - Only define test case in NR SA in both FR1 and FR2

    - Do not introduce the test for L1 impact

    - Introduce test cases for both intra-frequency, inter-frequency, and deactivated SCell measurement

    - Define a minimum set of test cases for SSB-based measurement

    - Only define test case under non-DRX

    - Define test case without SBI reporting

    - On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations

    - Do not define test cases with simultaneously FR1 and FR2 gaps configured

    - Test cases are limited to single serving carrier

    - Only use mandatory gap patterns to define test cases

    - Focus on only fully non-overlap and partially partial overlap in the test case design

    - Verify gap dropping behaviour without introducing additional test cases

    - Limit the tests to BWP switching trigger events for Pre-MG activation and deactivation procedures.

RAN4 to revisit suitability of option 1 of issue 5-1-2 for testing Pre-MG activation/deactivation once the scenarios for Case 1 test cases are agreed.

RAN4 to proceed with following test cases for Case 1.

    - Pre-MG + Pre-MG, Pre-MG + Type-2 MG. Hence, configure only two MGs in each test case.

    - Concurrent MG and 1 Pre-MG activation / deactivation in same FR

    - Pre-MGs with activation / deactivation in the same FR.

    - Pre-MG (de)activation is based on the associated MO or the associated pre-configured status

    - For Pre-MG (de)activation, assume either higher or lower priority of Pre-MG vs. Type-2 MG.

    - Design test scenarios without and with dynamic collisions, for both gap combinations Type 2 MG + Pre-MG and for Pre-MG + Pre-MG.

    - For some scenarios test network-controlled Pre-MG (de)activation, for other test UE-autonomous Pre-MG (de)activation.

    - Test the (de)activation delay for simultaneous and non-simultaneous Pre-MG (de)activation.

RAN4 to proceed with following test cases for Case 2.

    - NCSG + NCSG, NCSG + Type-2 MG. Hence, configure only two MGs in each test case.

    - 1 Concurrent MG and 1 NCSG in the same FR

    - 2 NCSGs (non-overlapping) in the same FR

    - For collision test scenarios, test the proximity condition.

    - The use of concurrent MG and/or NCSG for measurement of deactivated SCells depends on the outcome of the Rel-17 discussion.

- References

- RP-232440, “Revised WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, Intel, MediaTek

- R4-2321640, “WF for NR_MGE-2_part1”, Moderator (MediaTek inc.)

- R4-2318166, “Topic summary for [109][210] NR_MG_enh2_part1”, Moderator (MediaTek inc.)