3GPP TSG-RAN WG4 Meeting # 110	R4-2401917

Athens, Greece, Feb. 26 – Mar. 1, 2024

Agenda Item: 8.24.2

Source: MediaTek Inc.

Title: Discussion on RRM performance requirements for R18 SL Relay

Document for: Discussion

- Introduction

Last meetings, RAN4 had some discussion on the RRM performance requirements for R18 SL relay [1]. Now all the issues are open. In this contribution, we would like to share our view.

- Discussion

2.1 Accuracy requirements

In R17, intra-frequency discovery signal measurement accuracy requirements are defined for U2N remote UEs. There is no difference on the measurement of SD-RSRP or SL-RSRP. Legacy accuracy requirements is applicable to remote UE in multipath scenario too. In current spec, it is limited to U2N scenario. Some update is needed to extend to multipath scenario.

Proposal 1: Legacy discovery signal measurement accuracy requirements are applicable to multipath scenario.

2.2 Test case

Last meeting, the discussion focus on whether define test cases for the following two:

- Delay of selection/reselection of relay UE in U2U relay scenario

- Interruptions caused by the remote UE and relay UE due to the SL DRX operation between the remote UE and the relay UE in multi-path relay scenario.

Regarding the first bullet, UE behavior on selection/reselection of relay UE is the same in U2U and U2N scenarios. The question is whether UE supporting U2U is mandatory to support U2N. In our understanding, the answer is no. So we think it is better to define a test case on selection/reselection of relay UE for U2U scenario. For UE capable of both U2U and U2N, it is required to pass only one of the two test cases.

Proposal 2: Define a test case on selection/reselection of relay UE for U2U scenario. UE capable of both U2U and U2N is required to pass only one of the two test cases (U2U and U2N).

Regarding the second bullet, the question is whether the remote UE and relay UE need to pass the test cases defined for general SL UEs. In our understanding, the answer is yes. As the interruption caused by the remote UE and relay UE due to the SL DRX operation is exactly the same as legacy interruption caused by SL UE due to DRX operation which is tested in A.9.1.6.2. We don’t think it is necessary to define a new test case.

Proposal 3: Not to define a new test case to test the interruptions caused by the remote UE and relay UE due to the SL DRX operation between the remote UE and the relay UE in multi-path relay scenario.

- Summary

In this paper, we have some discussion on R18 SL relay performance requirements. We have the following proposals:

Proposal 1: Legacy discovery signal measurement accuracy requirements are applicable to multipath scenario.

Proposal 2: Define a test case on selection/reselection of relay UE for U2U scenario. UE capable of both U2U and U2N is required to pass only one of the two test cases (U2U and U2N).

Proposal 3: Not to define a new test case to test the interruptions caused by the remote UE and relay UE due to the SL DRX operation between the remote UE and the relay UE in multi-path relay scenario.

4 Reference

R4-2321416, WF on SL Relay RRM core/performance requirements, RAN4#109 meeting, Nov. 13 – Nov. 17, 2023