3GPP TSG-RAN WG4 Meeting #110 	R4-2401931

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on UL transmit timing requirements for R18 LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. For the following agreements in RAN4 #109, capture the details of the ‘time gap’ as ‘TCI state activation before/during cell switch’, for the reasons stated in P1, P2 of R4-2401930 RAN4 to define a time gap between TCI state activation and PDCCH order RACH or cell switch. If PDCCH order or cell switch cmd is received before the time gap, additional time for T/F tracking in PDCCH order RACH delay or cell switch delay requirement is needed. 2. The applicability of uplink timing accruacy requirements should be clarified as that they only apply to the case when the SSB for the PRACH is associated to one of known TCI state, as stated in P4 of R4-2401930. 3. The applicability of RRM requirements needs to be clarified for the collision cases between SSBs for early-Sync in LTM, and the SSBs for legacy L1/L3 RRM procedures, as in P5 and P6 of R4-2401930. 4. For non-collocated CA, if one DL-only SCell is configured as a candidate cell for LTM, and early RACH before cell switch is triggered on this SCell, then UE should also use the SSB in the SCell as the DL timing reference, as stated in P7 of R4-2401930. 5. RRM requirements for UE-based TA are needed according to RAN1/2 latest conclusions, as stated in P8 of R4-2401930. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Capture the details of the ‘time gap’ as ‘TCI state activation before cell switch’. 2. Clarify the applicability of UL timing accuracy requirements if the SSB for PRACH is associated to one SSB that is unknown for the UE. 3. Clarify the applicability of RRM requirements for the collision cases between SSBs for early-Sync in LTM, and the SSBs for legacy L1/L3 RRM procedures. 4. Specify RRM requirements for the case when candidate cell is one of the serving cell and PDCCH ordered RACH is triggered. 5. Capture RRM requirements for UE-based TA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. The agreements are not captured and the requirements are in-sufficient. 2. Requirement applicability is not clear. 3. Requirement applicability is not clear. 4. In-sufficient RRM requirements for the PCell/SCell switch scenario. 5. No RRM requirements for UE-based TA, and the feature may not work well. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8, 7.1, 9.5.4.1, 9.13.4.1, 9.14.5.1, 9.15.6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

8.X	Downlink LTM TCI state activation delay requirements

8.X.1	Introduction

The requirements in this clause apply for a UE configured with ltm-DL-OrJointTCI-StateToAddModList-r18 configurations on a candidate cell for LTM. The LTM TCI state can be activated:

- by a MAC CE indicating LTM TCI state activation before cell switch command as specified in clause 6.1.3.75 of TS 38.321, or

- by a MAC CE incidating an LTM cell switch, and LTM TCI activation during the cell switch, as specified in clause 6.1.3.74 of TS 38.321.

UE shall complete the activation of downlink LTM TCI state within the delay defined in this clause.

When the target DL TCI state refers to a candidate cell and the activation of TCI is performed before or during cell switch, the requirements in this clause are applicable provided that following conditions are met:

-	For a UE supports [LTM RTD > CP capability], the requirements apply when Rx timing difference between active TCI states on one frequency layer is up to MRTD as specified in clause [7.6.4].

-	For a UE which do not support [LTM RTD > CP capability], the requirements apply when Rx timing difference between active TCI states on any frequency layer is less than CP.

-	Editor’s Note: TBD other conditions

### 8.X.2	Known conditions for downlink TCI state

The downlink TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state activation, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state

-	The MAC CE command indicating the activation of downlink TCI state is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received.

-	The target downlink TCI state remains detectable during the downlink TCI state activation period

-	The SSB associated with the downlink TCI state remain detectable during the downlink TCI activation period

-	SNR of the downlink TCI state ≥ -3dB

Otherwise, the downlink TCI state is unknown.

### 8.X.3	 MAC-CE based downlink LTM TCI state activation delay

The requirements in this clause shall apply for downlink LTM TCI state activation using separate DL TCI state or joint TCI state of unified TCI state switch framework. The requirements specified in this clause are applicable if

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, and a MAC CE activates one or more than one target separate TCIs, and at least one DL TCI is included, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, and a MAC CE activates one or more than one target joint TCI.

The requirements in this clause shall apply if the candidate cell satisfies the LTM known cell condition defined in 6.3.1.2 or 8.20.2. If the known cell condition is not met, longer delay may be expected.

For downlink LTM TCI activation before cell switch, as defined in 8.X.1:

- If all the target TCI states are known, upon receiving PDSCH carrying MAC CE activation command in slot n, UE shall be able to complete the activation of the TCI state at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk* TL1-RSRP / NR slot length

- If any target TCI state is unknown, upon receiving PDSCH carrying MAC CE activation command in slot n, UE shall be able to complete the activation of the TCI state at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ [2 * TL1-RSRP] / NR slot length

- No RRM requirements are applicable if all target TCI states are unknown.

Note: Upon reception of the MAC CE command, UE may or may not also activate the uplink LTM TCI before cell switch. There is no additional impact to the delay of completing downlink LTM TCI state activation.

where

-	THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

[-	TL1-RSRP is measurement period for SSB-based L1-RSRP as specified in clause 9.14 and 9.15; In case more than one TCIs are activated, TL1-RSRP is longest measurement period for SSB-based L1-RSRP among all the associated SSBs]

-	TOk = 1 if target TCI state is not in the active TCI state list for candidate cell, 0 otherwise.

After completion of LTM TCI state(s) activation, the TCI states indicated in the LTM TCI activation command are in the active TCI list.

For downlink LTM TCI activation during cell switch, as defined in 8.X.1:

- If the target TCI state is known, or if the target TCI is indicated based on the SSB index reported in L3 measurement report of the known target cell, upon receiving PDSCH carrying MAC CE based cell switch command, UE shall be able to complete the activation of the downlink TCI state at the time UE completes the LTM cell switch as specified in clause 6.3.1.3 or 8.20.3.

- In case of joint TCI state switch, if the target PL-RS is not maintained, UE is not expected to receive on DL based on the target TCI state before UE completes the DL and UL TCI state switch.

<End of Change #1>

<Start of Change #2>

7.1	UE transmit timing

7.1.1	Introduction

The UE shall have capability to follow the frame timing change of the reference cell in connected state or when transmiting PUSCH on CG resources for SDT in RRC_Inactive. The uplink frame transmission takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. For serving cell(s) in sTAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the sTAG. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

In the requirements of clause 7.1.2, the term reference cell on a carrier frequency subject to CCA is not available at the UE refers to when at least one SSB is configured by gNB, but the first two successive candidate SSB positions for the same SSB index within the discovery burst transmission window are not available during at least one discovery burst transmission window, at the UE due to DL CCA failures at gNB during the last 1280 ms; otherwise the reference cell on the carrier frequency subject to CCA is considered as available at the UE.

[For UE supporting [RACH-based early TA acquisition] for LTM, and if the candidate cell is a neighbor cell or a secondary serving cell without uplink carrier,

-	UE shall have capability to follow the frame timing of the reference cell. The PRACH transmission take place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For the neighbor cell to which PRACH is transmitted, UE shall use this neighbor cell as the reference cell for deriving transmit timing. UE initial transmit timing accuracy is defined in the following requirements.]

Editor’s Note: The above requirements for RACH-based early TA acquisition can be revisited if any further agreements in other WG have impacts on the DL reference timing

For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the first detected path (in time) of the corresponding downlink reference signal associated with UL-TCIState or DLorJointTCIState (if unifiedTCI-StateType is indicated as Joint) , where NTA  is commanded by the network independently for each TAG [TS 38.331].

In the requirements of clause 7.1.2, the SSB applies for both CD-SSB and NCD-SSB if the UE supports [UE capability IE for FG 53-3].

7.1.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission, or it is the first transmission sent on the PSCell for activating the deactivated SCG without RACH.

-	when it is the transmission for PUSCH on CG resources for SDT in RRC_Inactive.

-	when it is the first transmission on target cell after UE receives LTM cell switch command.

In case UE supports [UE-based TA capability] for LTM, if UE-MeasuredTA-ID is configured, and the RTD between cells with the same UE-MeasuredTA-ID is no larger than CP, Te requirements apply to the first transmission on target cell after LTM cell switch. The transmit timing adjustment requirement is defined in 7.1.2.5.

When the UL SCS is 120 kHz or smaller,

- If UE supports [RACH-based early TA acquisition] for LTM, and if the candidate cell is a neighbor cell or a secondary serving cell without uplink carrier, the UE shall meet the Te requirement for an initial transmission at the PRACH occasion defined in clause 8.1 of TS 38.213, provided:

  - For FR2,

    - if the SSB index indicated in PDCCH order is associated to a known TCI state of the target cell according to 8.X.2, and the TCI state is in the active TCI state list according to 8.X.3, and measurement period of L1-RSRP for the SSB is no longer than 160ms, the delay for time frequency tracking TSSB  = [0];

    - if the SSB index indicated in PDCCH order is associated to a known TCI state of the target cell according to 8.X.2, but the TCI state is not in the active TCI state list according to 8.X.3, the delay for time frequency tracking TSSB  = Tfirst_SSB + TSSB_proc;

  - For FR1,

    - if the SSB index indicated in PDCCH order is associated to a known TCI state of target cell according to 8.X.2, and the TCI state is in the active TCI state list according to 8.X.3, and measurement period of L1-RSRP for the SSB is no longer than 160ms, the delay for time frequency tracking TSSB  = 0;

    - if the SSB index indicated in PDCCH order is associated to a known TCI state of target cell according to 8.X.2, but the TCI state is not in the active TCI state list according to 8.X.3, the delay for time frequency tracking TSSB  = Tfirst_SSB + TSSB_proc;

  - where

    - Tfirst_SSB is the delay of the first SSB based on the SSB_Periodicity of the target cell after UE decodes the PDCCH order, i.e. after NT,2 in [8.1, TS 38.213].

    - TSSB_proc is the delay of the SSB processing, which is 2ms.

    - Note: Te requirements are NOT applicable if the first SSB after UE decodes the PDCCH order collides with SSB-based tracking in serving cell TCI activation or SSB-based tracking for another simultaneous PDCCH ordered RACH.

  - For the case of PRACH bandwidth within active UL BWP, the delay for RF/BB preparation and RF retuning after time-frequency tracking ∆RF/BB_preparation = 0.

  - For the case of PRACH bandwidth outside active UL BWP but within one of configured UL BWPs of any active serving cell, the delay for RF/BB preparation and RF retuning after time-frequency tracking ∆RF/BB_preparation is DCI based BWP switching delay specified in clause 8.6.

  - For the case of PRACH bandwidth not within any of the configured UL BWPs of any active serving cell, the delay for RF/BB preparation and RF retuning after time-frequency tracking is based on the reported value in [UE capability for RF/BB retuning].

- Otherwise, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms.

When the UL SCS is 480 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 80 ms. When the UL SCS is 960 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 40 ms.

The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna. NTA for PRACH is defined as 0.

(in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA for other channels is not changed until next timing advance is received. The value ofdepends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). is defined in Table 7.1.2-2.

For multi-DCI based multi-TRP operation with two TAs, UE initial transmission timing error requirements specified in this clause is applicable for each TAG and shall be met for each TAG separetely. The reference point described in this clause for each TAG is the the first detected path (in time) of the corresponding downlink reference signal associated with UL-TCIState or DLorJointTCIState (if unifiedTCI-StateType is indicated as Joint) [TS 38.331].

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2-1 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| 2-2 | 120 | 120 | 3.5*64*Tc |
|  |  | 480 | 1.58*64*Tc |
|  | 480 | 120 | 2.86*64*Tc |
|  |  | 480 | 1.35*64*Tc |
|  |  | 960 | 0.90*64*Tc |
|  | 960 | 120 | 2.80*64*Tc |
|  |  | 480 | 1.13*64*Tc |
|  |  | 960 | 0.86*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.1.2-2: The Value of

| Frequency range and band of cell used for uplink transmission | (Unit: TC) |
| --- | --- |
| FR1 FDD or TDD band with neither E-UTRA–NR nor NB-IoT–NR coexistence case | 25600 (Note 1) |
| FR1 FDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case | 0 (Note 1) |
| FR1 TDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case | 39936 (Note 1) |
| FR2 | 13792 |
| Note 1: The UE identifies based on the information n-TimingAdvanceOffset as specified in TS 38.331 [2]. If UE is not provided with the information n-TimingAdvanceOffset, the default value of is set as 25600 for FR1 band. In case of multiple UL carriers in the same TAG, UE expects that the same value of n-TimingAdvanceOffset is provided for all the UL carriers according to clause 4.2 in TS 38.213 [3] and the value 39936 of can also be provided for a FDD serving cell. Note 2: Void |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell except when the timing advance in clause 7.3 is applied.

Table 7.1.2-3: void

If the UE uses a reference cell on a carrier frequency subject to CCA for deriving the UE transmit timing, then the UE shall meet all the transmit timing requirements defined in clause 7.1.2 provided that the reference cell is available at the UE. If the reference cell is not available at the UE on a carrier frequency subject to CCA, then the UE is allowed to transmit in the uplink provided that the UE meets all the transmit timing requirements defined in clause 7.1.2; otherwise the UE shall not transmit any uplink signal.

If a reference cell on a carrier frequency belonging to the PTAG, which is subject to CCA, is not available at the UE then the UE is allowed to use any of available activated SCell(s) at the UE in PTAG as a new reference cell. If the SCell used as reference cell is deactivated, or becomes not available, the UE is allowed to use another active serving cell in PTAG as new reference cell.

If a reference cell on a carrier frequency belonging to the STAG, which is subject to CCA is not available at the UE then the UE is allowed to use any of available activated SCell(s) at the UE in STAG as a new reference cell.

7.1.2.1	Gradual timing adjustment

Requirements in this section shall apply regardless of whether the reference cell is on a carrier frequency subject to CCA or not.

When the transmission timing error between the UE and the reference timing exceeds Te then the UE shall adjust the timing such that timing error is to within Te. The reference timing shall be  before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200 ms for SCS of UL signals smaller or equal to 120 kHz and 100 ms for SCS of upling signals larger or equal to 480 kHz.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 7.1.2.1-1.

For multi-DCI based multi-TRP operation with two TAs, gradual timing adjustment specified in this clause applies for each TAG and shall be met for each TAG. The reference timing described in this clause for each TAG is the the first detected path (in time) of the corresponding downlink reference signal associated with UL-TCIState or DLorJointTCIState (if unifiedTCI-StateType is indicated as Joint) [TS 38.331].

Table 7.1.2.1-1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2-1 | 60 | K*64*Tc | 2.5*64*Tc |
|  | 120 | K*64*Tc | 2.5*64*Tc |
| 2-2 | 120 | 2.5*64*Tc | 2.5*64*Tc |
|  | 480 | 0.8*64*Tc | 0.8*64*Tc |
|  | 960 | 0.8*64*Tc | 0.8*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] NOTE 2: When highSpeedMeasFlagFR2-r17 is configured for UE supporting power class 6, K = 4.5; otherwise, K = 2.5. |  |  |  |

7.1.2.2	Void

Table 7.1.2.2-1: Void

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE

When highSpeedMeasFlagFR2-r17 is configured and highSpeedLargeOneStepUL-TimingFR2-r17 is enabled for UE supporting FR2 power class 6 and ue-OneShotUL-TimingAdj-r17 capability, the following requirements apply to the UE:

If the UE also indicates to support [highSpeedTCISwitchEnhMAC-CE-FR2-r18] and [R18 enhanced MAC-CE indication] is indicated as ‘1’ for the TCI state switch

-	the UE transmit timing immediately after TCI state switch shall be Tnew-(NTA+NTA offset)+2 (Told-Tnew) and clause 7.1.2.1 requirements don’t apply.

-	The UE UL transmission timing error after the TCI state switching procedure shall be less than or equal to ±Te as specified in clause 7.1.2 if the new target TCI state is within active TCI state list, otherwise ±7*64*Tc, and the reference point is Tnew-(NTA+NTA offset)+2 (Told-Tnew).

If the UE also indicates to support [highSpeedTCISwitchEnhMAC-CE-FR2-r18] and [R18 enhanced MAC-CE indication] is indicated as ‘0’ for the TCI state switch

-	the requirement in clause 7.1.2.1 apply to the first UL transmission after a TCI state switch.

If the UE does not indicate to support [highSpeedTCISwitchEnhMAC-CE-FR2-r18] or [R18 enhanced MAC-CE indication] is not indicated for the TCI state switch

-	If the absolute value Told-Tnew≤CP/4, the requirement in clause 7.1.2.1 apply to the first UL transmission after a TCI state switch.

-	Otherwise, the UE transmit timing immediately after TCI state switch shall be Tnew-(NTA+NTA offset)+2 (Told-Tnew) and clause 7.1.2.1 requirements don’t apply.

-	The UE UL transmission timing error after the TCI state switching procedure shall be less than or equal to ±Te as specified in clause 7.1.2 if the new target TCI state is within active TCI state list, otherwise ±7*64*Tc, and the reference point is Tnew-(NTA+NTA offset)+2 (Told-Tnew).

Above,

-	Tnew (in Tc units) is the DL timing defined as the time when UE receives downlink frame with new target TCI state.

-	Told  (in Tc units) is the DL timing defined as the time when UE receives downlink frame with old source TCI state.

7.1.2.4	UE transmit timing for positioning measurements

If cell reselection occurs in RRC_Inactive within srs-PosRRC-InactiveValidityArea area, the UE shall have capability to follow the frame timing change of the reference cell when transmiting SRS for positioning. The uplink frame transmission takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. The UE shall use the current camping cell as reference cell for deriving the UE transmit timing. Gradual timing adjustment requirements are defined in the following requirements.

When ueAotonomousTaAdjustment is configured, the following requirements apply to the UE supporting [AutonomousAdjustOneStepUL-Timing-r18]:

If the DL timing difference is ≥ CP/4, UE autonomously adjusts the TA based on twice of the DL timing difference

-	The UE  UL transmission timing error after autonomous TA adjustment shall be less than or equal to Te in clause 7.1.2.3.

If the DL timing difference is < CP/4, UE follows the DL timing of the new camping cell by performing gradual timing adjustment as defined in clause 7.1.2.1.

The above DL timing difference is timing difference between the last camping cell and current camping cell.

7.1.2.5	UE transmit timing for UE-based TA adjustment in LTM

If UE supports [UE-based TA capability] for LTM, and UE-MeasuredTA-ID is configured, the UE shall follow the frame timing change of the target cell when transmiting the first the uplink transmission to target cell after LTM cell switch. The uplink frame transmission takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the target cell. The UE shall use the source SpCell as one of the reference cell for deriving DL timing difference the UE transmit timing.

If the DL timing difference is ≥ CP/4, UE autonomously adjusts the TA based on twice of the DL timing difference

-	The UE UL transmission timing error after autonomous TA adjustment shall be less than or equal to Te in clause 7.1.2.3.

If the DL timing difference is < CP/4, UE follows the DL timing of the target cell by performing gradual timing adjustment as defined in clause 7.1.2.1.

The above DL timing difference is timing difference between the sourc SpCell and the target cell.

<End of Change #2>

<Start of Change #3>

9.5.4.1	SSB based L1-RSRP Reporting

When groupBasedBeamReporting-r17 is not configured, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

When groupBasedBeamReporting-r17 is configured, the UE shall be capable of performing L1-RSRP measurements based on the two configured SSB resource sets for L1-RSRP, and the UE physical layer shall be capable of reporting group-based L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

<…Unchanged part omitted…>

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and GAP configurations does not meet pervious conditions.

For UE supports [RACH-based early TA acquisition] for LTM, if UE needs to perform the SSB-based time frequency tracking for the PRACH triggered by PDCCH order on one of the SSBs configured for SSB-based L1 measurement, longer evaluation period would be expected.

When UE is configured with aperiodic MUSIM gap and the aperiodic MUSIM gap is overlapping with SSB resource occasion for L1-RSRP, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and SSB resource occasions for L1-RSRP are fully overlapped with MUSIM gap(s) or fully overlapped with the union of MUSIM gap(s) and GAPs, no requirement applies for the SSB based L1-RSRP measurement.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

<…Unchanged part omitted…>

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting

If a cell with PCI different from serving cell is known according 9.13.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_CDP. The requirements specified in this clause are only applicable when

-	highSpeedMeasFlag-r16 is not configured, and

-	highSpeedMeasFlagFR2-r17 is not configured, and

-	highSpeedMeasCA-Scell-r17 is not configured, and

-	SSBs of CDP outside SMTCs for L1-RSRP measurement are not overlapped with concurrent gaps.

<…Unchanged part omitted…>

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For UE supports [RACH-based early TA acquisition] for LTM, if UE needs to perform the SSB-based time frequency tracking for the PRACH triggered by PDCCH order on one of the SSBs configured for SSB-based L1 measurement, longer evaluation period would be expected.

For either an FR1 or FR2 cell with PCI different from serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

When UE is configured with aperiodic MUSIM gap and the aperiodic MUSIM gap is overlapping with SSB resource occasion for inter-cell L1-RSRP, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and SSB resource occasions for inter-cell L1-RSRP are fully overlapped with MUSIM gap(s) or fully overlapped with the union of MUSIM gap(s) and GAPs, no requirement applies for the SSB based inter-cell L1-RSRP measurement.

For either an FR1 or FR2 cell with PCI different from serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

9.14.5.1	SSB based L1-RSRP Reporting

The requirements specified in this clause are only applicable when

-	highSpeedMeasFlag-r16 is not configured, and

-	highSpeedMeasFlagFR2-r17 is not configured, and

-	highSpeedMeasCA-Scell-r17 is not configured.

-	The SSB from the neighbor cell completely contained in the active BWP of the UE

<…Unchanged part omitted…>

Longer measurement period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For UE supports [RACH-based early TA acquisition] for LTM, if UE needs to perform the SSB-based time frequency tracking for the PRACH triggered by PDCCH order on one of the SSBs configured for SSB-based L1 measurement, longer evaluation period would be expected.

For either an FR1 or FR2 cell, longer measurement period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

9.15.6.1.1	Inter-frequency SSB based L1-RSRP measurement

When configured by the network and the inter-frequency cell’s SSB is completely contained in the DL active BWP, the UE shall be able to perform L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap.

<…Unchanged part omitted…>

Longer measurement period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet previous conditions.

For UE supports [RACH-based early TA acquisition] for LTM, if UE needs to perform the SSB-based time frequency tracking for the PRACH triggered by PDCCH order on one of the SSBs configured for SSB-based L1 measurement, longer evaluation period would be expected.

For either an FR1 or FR2 cell, longer measurement period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

<End of Change #3>