3GPP TSG-RAN WG4 Meeting #110 	R4-2401935

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on cell switch delay requirements for R18 LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. For LTM cell switch, the TCI activated by cell switch command could be based on SSB index reported in the L3 reports as discussed in P2 of R4-2401934. 2. Move the known condition of TCI according to P3 of R4-2401934. 3. The conditions for Tfirst_SSB and TSSB_proc equals zero is not correct. Actually the TCI activation should be LTM TCI activation, but not serving cell activation. The TCI activation delay requirements should be defined in 8.X |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add the requirement applicability to the scenario where TCI activated by cell switch command is based on SSB index reported in the L3 reports. 2. Remove the known condition of TCI as it is captured in 8.X.3. 3. The conditions for Tfirst_SSB and TSSB_proc equals zero are removed and refers to 8.X. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are redundant and not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3, 8.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

6.3	L1/L2-Triggered Mobility

6.3.1	PCell Cell Switch

6.3.1.1	Introduction

The purpose of LTM cell switch is to switch the PCell or PSCell to another cell. The requirements in this section are applicable to LTM PCell switch.

The requirements in this clause are applicable to SA and NR-DC, and to both intra-frequency and inter-frequency LTM cell switch.

The requirements in this clause are applicable to SA for the following scenarios:

PCell switch to a neighboring cell.

-	FR1 cell to FR1 cell

-	FR1 cell to FR2 cell

-	FR2 cell to FR2 cell

-	FR2 cell to FR1 cell

PCell switch to a serving SCell in MCG

-	FR1 cell to FR1 cell

-	FR2 cell to FR2 cell

The requirements in this clause are applicable to NR-DC for the following scenarios:

PCell switch to a neighboring cell

-	FR1 cell to FR1 cell

PCell switch to a serving SCell in MCG

-	FR1 cell to FR1 cell

6.3.1.2	LTM Cell Switch delay

LTM cell switch delay DLTM is the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.

When

- the target cell, and the target joint UL/DL TCI state or separate UL and DL TCI states activated by the MAC-CE LTM cell switch command if exists, are known, or

- the target cell is known, and the target joint UL/DL TCI state or separate UL and DL TCI states activated by the MAC-CE LTM cell switch, if exists, is based on the reported SSB index in L3 report,

the LTM cell switch delay is defined as:

DLTM = Tcmd  + TLTM-interrupt

Where:

Tcmd equals to THARQ + 3ms, where THARQ is the timing between cell switch command and acknowledgement as specified in TS 38.213.

TLTM-interrupt is as stated in section 6.3.1.2.1.

The target cell in the LTM cell switch command is known if the following conditions are met:

-	During the last 5 seconds before the reception of the cell switch command:

-	The UE has sent a valid L1 or L3 measurement report for the target cell, and

-	One of the SSBs measured from the NR target cell being configured remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

-	One of the SSBs measured from the target cell also remains detectable during the cell switch delay according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell.

Otherwise, the cell is unknown.

The target joint DL/UL TCI state or separate DL and UL TCI states activated by the LTM cell switch command are known according to known conditions of LTM TCI defined in 8.X.2.

6.3.1.3	Interruption time

The interruption time TLTM-interrupt is the time between the end of the last TTI containing the MAC-CE command for LTM cell switch until the time the UE transmits the first UL message on the target cell, excluding Tcmd stated in section 6.3.1.2.

TLTM-interrupt  = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms,

Where:

TLTM-RRC-processing is the time for early ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command.

TLTM-RRC-processing  = 0, if the the UE supporting capability [earlyDecodingAndValidityCheck] has performed early ASN.1 decoding and validity/compliance check of the complete LTM candidate cell configuration prior to the cell switch command for the LTM target cell indicated in the cell switch command.

Editor’s note: FFS the conditions under which the UE with new capability can work with early ASN.1 decoding and validity/compliance check

Otherwise TLTM-RRC-processing = 10 ms.

TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

The value of TLTM-processing is 20 ms for FR1 to FR1 and FR2 to FR2 LTM cell switch.

The value of TLTM-processing is 40 ms for FR1 to FR2 and FR2 to FR1 LTM cell switch.

Editor’s note: FFS whether a smaller value can be considered in some scenarios or under certain conditions, or whether to introduce a new UE capability to support smaller value of TLTM-processing.

Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

TRS-proc is the time for SSB processing.

Tfirst-RS = 0 and TRS-proc= 0 under the following conditions:

-	The target TCI state indicated in the LTM cell switch command is in the LTM candidate cell active TCI state list or in the serving cell active TCI state list, as defined in 8.X.3

3Nslotsubframe,µ

Otherwise,

Tfirst-RS is the time to the first SSB transmission on the target cell [after Tcmd].

Editor’s note: FFS whether TRS transmission is also considered.

TRS-proc = 2 ms.

TLTM-IU is the interruption uncertainty during LTM cell switch.

For RACH-based LTM cell switch, TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TLTM-IU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

For RACH-less LTM cell switch, TLTM-IU_is the uncertainty on transmitting the first uplink transmission on the target cell.

<End of Change #1>

<Start of Change #2>

8.20	PSCell Cell Switch

8.20.1	Introduction

The purpose of LTM cell switch is to switch the PCell or PSCell to another cell. The requirements in this section are applicable to LTM PSCell switch.

The requirements in this clause are applicable to both intra-frequency and inter-frequency LTM cell switch.

The requirements in this clause are applicable to NR-DC for the following scenarios:

PSCell switch to a neighboring cell

-	FR1 cell to FR1 cell

-	FR1 cell to FR2 cell

-	FR2 cell to FR2 cell

-	FR2 cell to FR1 cell

PSCell switch to a serving SCell in SCG

-	FR1 cell to FR1 cell

-	FR2 cell to FR2 cell

8.20.2	LTM Cell Switch delay

LTM cell switch delay DLTM is the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.

When

- the target cell, and the target joint UL/DL TCI state or separate UL and DL TCI states activated by the MAC-CE LTM cell switch command, if exists, are known, or

- the target cell is known, and the target joint UL/DL TCI state or separate UL and DL TCI states activated by the MAC-CE LTM cell switch, if exists, is based on the reported SSB index in L3 report,

the LTM cell switch delay is defined as:

DLTM = Tcmd + TLTM-interrupt

Where:

Tcmd is as stated in section 6.3.1.2.

TLTM-interrupt is as stated in section 8.20.1.3.

The definition of known LTM target cell is as stated in section 6.3.1.2. The target joint DL/UL TCI state or separate DL and UL TCI states activated by the LTM cell switch command are known according to known conditions of LTM TCI defined in 8.X.2.

8.20.3	Interruption time

The interruption time TLTM-interrupt is the time between the end of the last TTI containing the MAC-CE command for LTM cell switch until the time the UE transmits the first UL message on the target cell, excluding Tcmd stated in section 8.20.1.2.

TLTM-interrupt  = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms,

Where:

TLTM-RRC-processing, TLTM-processing, Tfirst-RS, TRS-proc and TLTM-IU are as stated in section 6.3.1.3.

<End of Change #2>