3GPP TSG-RAN4 Meeting #110		R4-2401941

# Athens, Greece, February 26-March 01, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_SL_relay_enh-Perf) Draft CR on applicability of SD-RSRP and SL-RSRP accuracy requirements in multipath scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To define applicable conditions for a remote sidelink UE in multipath relay scenario to meet the SD-RSRP and SL-RSRP accuracies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is defined that the SD-RSRP and SL-RSRP accuracies are applicable for a remote sidelink UE in multipath relay scenario provided that the remote UE: - is synchronised to the sidelink relay UE that is measured and - is in-coverage on the frequency used for sidelink if both the direct path and the sidelink on the indirect path are on the same frequency or - is out of coverage on the frequency used for sidelink if the direct path and the sidelink on the indirect path are on different frequencies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SD-RSRP and SL-RSRP accuracy performance of the remote UE in multipath relay operation cannot be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

### 10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements

The requirements in this clause are applicable for a remote sidelink UE in U2N relay scenario provided that the remote UE:

-	is out of coverage on the frequency used for sidelink, and

-	that is synchronised to the sidelink relay UE that is measured.

The requirements in this clause are applicable for a remote sidelink UE in multipath relay scenario provided that the remote UE:

- is synchronised to the sidelink relay UE that is measured and

- is in-coverage on the frequency used for sidelink if both the direct path and the sidelink on the indirect path are on the same frequency or

- is out of coverage on the frequency used for sidelink if the direct path and the sidelink on the indirect path are on different frequencies.

----------------------END OF CHANGE----------------------------