3GPP TSG-RAN WG4 Meeting # 110                                    R4-2401978

# Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.16.1.1.2

# Title: 	Discussion on L1-RSRP measurement requirements

# Source: 	ZTE Corporation

# Document for:	Approval

Introduction

L1-RSRP measurement requirements of L1/L2 based inter-cell mobility have been discussed widely for several meetings. During RAN4 #108bis meeting, we have reached lots of consensus on L1/L2 inter-cell mobility delay requirements for inter-cell mobility in WF [1]. There are still some remaining issues that need to be addressed. In this contribution, we want to share further considerations on these open issues.

Discussion

Applicability rule for L1-RSRP measurement

For intra-frequency L1-RSRP or inter-frequency L1-RSRP on candidate target cell, if network enables deriveSSB-IndexFromCell or deriveSSB-IndexFromCellInter, UE can assume SFN and frame boundary alignment across cells on the serving frequency. Meanwhile, we have the common understanding that if deriveSSB-IndexFromCell or deriveSSB-IndexFromCellInter is enabled, UE can derive SSB index according to serving cell timing and skip the DL synchronization for obtaining frame/slot/symbol level synchronization and SFN level synchronization. For intra-frequency L1-RSRP or inter-frequency L1-RSRP on candidate target cell, if deriveSSB-IndexFromCell or deriveSSB-IndexFromCellInter is not enabled, which means that the RTD between the SSBs of serving cell and neighbour cell on the same carrier is larger than 2 SSB symbols, we can discuss it on whether UE has performed L3 measurement with SSB index reading. In our understanding, no additional time is needed if UE has performed L3 measurement with SSB index reading on the to-be-measured neighbor cell, otherwise, additional time for reading SSB index is needed.

Issue 2-1-4: How to get SSB index information (on which symbols the RS to-be-measured) of the to-be-measured neighbor cell before L1-RSRP measurement? For information RAN4#108 Common understanding: If deriveSSB-IndexFromCell or deriveSSB-IndexFromCellInter is enabled, UE can derive SSB index according to serving cell timing.. <Agreement> - If deriveSSB-IndexFromCell and deriveSSB-IndexFromCellInter-r17 are not enabled, but UE has performed L3 measurement with SSB index on the candidate cell, no additional time is needed - If deriveSSB-IndexFromCell and deriveSSB-IndexFromCellInter-r17 are not enabled, and UE has not performed L3 measurement without SSB index reading on the candidate cell, additional time for time index detection is needed.

For the second bullet, UE has not performed L3 measurement without SSB index reading on the candidate cell. There is a wording error, it should be “UE has not performed L3 measurement with SSB index reading on the candidate cell”.

- Proposal 1: Change “UE has not performed L3 measurement without SSB index reading on the candidate cell” to “UE has not performed L3 measurement with SSB index reading on the candidate cell”.

2.2 Intra-frequency L1-RSRP Measurement delay

Issue 2-3-1-1: whether to support the case that SSB periodicity of FR2 intra-frequency neighbour cell equals to SMTC periodicity in R18 LTM Ad hoc Agreement < Agreement> - Scenario of SSB periodicity of FR2 intra-frequency neighbour cell equals to SMTC periodicity in R18 LTM is supported. - Agree the following as baseline: - [When the SSB periodicity of FR2 intra-frequency cell is fully overlapped with SMTC periodicity of inter-frequency neighbour cell, the existing sharing factor P used for L1/L3 measurements can be reused, i.e., P =3 for L1 measurement and P=1.5 for L3 measurement.] - Further discuss whether to support the following optimization: - In FR2, L1-RSRP measurement period of less than 160ms is only possible under following conditions. RAN4 to discuss the feasibility of it and methods to achieve 160ms L1-RSRP measurement period. - L3 measurements are suspended after TCI state activation - N is 1 or reduced to some other value smaller than 8 (i.e., beam sweeping or reduced after TCI state activation for certain time) - RAN4 to find a method to achieve less than 160ms measurement period or a method to skip fine time tracking (e.g., by performing fine time tracking in parallel to UE processing). If RAN4 did not find a method to achieve less than 160ms L1-RSRP periodicity or a method to remove fine time tracking from the cell switch delay, for at least one configuration, RAN4 to send LS to RAN1 and RAN2 to convey pre-sync or pre-TCI state activation is not suitable/applicable for FR2.

As mentioned by other company in [2], we should handle the issue that L1-RSRP measurement period for FR2 is always larger than 160ms. We can refer the anylsis blew.

RAN4#108bis meeting agreement - When # of neighboring cells configured/activated to be measured is equal to or larger than 2 - When TCI state of neighbor cell is activated, UE performs L1-RSRP measurement on the neighbor cell whose TCI state is activated and the serving cell. UE may measure any other cell(s) based on UE implementation - The measurement period of serving cell is R15/R16 SSB based L1-RSRP measurement period scaled by 3 - The measurement period of the neighbor cell whose TCI state is activated is R15/R16 SSB based L1-RSRP measurement period scaled by 3 - For the other neighbor cells: no measurement delay requirements The above principle and requirements apply when the NW activate TCI state(s) from only one neighbor cell. FFS: the requirements when TCI states are activated on neighbor cells in multiple bands. - When TCI state of all the neighbor cells are not activated, UE performs L1-RSRP measurement on the serving cell and neighbor cell(s). - The measurement period of serving cell is R15/R16 SSB based L1-RSRP measurement period scaled by 3 - The measurement period of the neighbor cells is R15/R16 SSB based L1-RSRP measurement period scaled by 3*(# of neighbor cells) - When # of neighboring cells configured/activated to be measured is 1, reuse R17 ICBM measurement delay requirements.

Based on RAN4#108bis meeting agreement, L1-RSRP measurement period of neighbor cell whose TCI state is activated for FR2 is given by 3*M*P*N*TSSB. If we consider M is 1 and N is 8, then measurement period is 24*P*TSSB. If we consider legacy sharing factor of P=3 for L1-RSRP and P=1.5 for L3-RSRP, then measurement period is 72*TSSB. Even with last meeting agreement of prioritising cell whose TCI state was activated, even for TSSB of 5ms, measurement period will not be less than 160ms (e.g., 3*1*3*8*5ms is 360ms). Meanwhile, even if we assume equal sharing between L3 and L1 after TCI state is activated, the measurement period is also lager than 160ms (e.g., 3*1*2*8*5ms is 240ms). Assume LTM L1-RSRP measurement period is less than 160ms, 3*M*P*N*TSSB <160ms should be satisfied. even for TSSB of 5ms, if we consider M is 1 and N is 8, P<4/3.

Based on above analysis, if we require LTM L1-RSRP measurement period of less than 160ms, it seems not suitable to reuse existing sharing factor P used for L1/L3 measurements can be reused, i.e., P =3 for L1 measurement and P=1.5 for L3 measurement.

To reduce the measurement delay, P and/or N should be reduce. It’s ok for us to reduce beam sweeping factor and this method has been applied to R17 positioning.

Proposal 2: To reduce L1-RSRP measurement period, N should be reduced to some other value smaller than 8.

  - 2.3 Inter-frequency L1-RSRP Measurement delay

To cover the scenario that time offset of serving cell and neighbour cell is more than CP, we introduced the UE capability to indicate whether UE supporting SSB based intra-frequency L1 measurement on neighbor cell when RTD of serving cell and neighbor cell is larger than CP. However, we just discussed this capability for intra-frequency L1-RSRP measurement.

RAN4#106 (R4-2303175) Issue 3-2-2: Whether to consider RTD of serving cell and neighbour cell larger than one CP for intra-frequency L1-RSRP measurement <Agreement>: - For SSB based intra-frequency L1 measurement, support the scenario that RTD between the SSBs of serving cell and neighbour cell on the same carrier is larger than CP length of the corresponding SCS with additional UE capability. - Note: the need for UE capability can be further discussed subject to the outcome of the discussion on measurement framework

In our understanding, the probability of RTD> CP occurring in inter-f scenarios is higher, we should consider whether the capability of supporting RTD>CP should be applicable to inter-frequency L1-RSRP measurement, both inter-frequency L1-RSRP measurement with Type 1 measurement gap and inter-frequency L1-RSRP measurement without gap.

In our understanding, for inter-f L1-RSRP measurement with Type 1 measurement gap, the total number of FFT uesd for serving cell and neighbour cell is not one. Therefore, RTD>CP could be supported to inter-frequency L1-RSRP measurement with type1 gap whether the capability of supporting RTD>CP be applicable or not. However, for inter-frequency L1-RSRP measurement without gap, if there is no scheduling restriction and only one FFT used for serving cell and neighbour cell, the capability of supporting RTD>CP should be applicable.

Proposal 3: The capability of supporting RTD>CP can be applicable to inter-frequency L1-RSRP measurement without gap if there is only one FFT.

3 Conclusion

In this contribution, we give some discussions on L1/L2 inter-cell mobility delay requirements mobility, The conclusions are:

Proposal 1: Change “UE has not performed L3 measurement without SSB index reading on the candidate cell” to “UE has not performed L3 measurement with SSB index reading on the candidate cell”.

Proposal 2: To reduce L1-RSRP measurement period, N should be reduced to some other value smaller than 8.

Proposal 3: The capability of supporting RTD>CP can be applicable to inter-frequency L1-RSRP measurement without gap if there is only one FFT.

4	Reference

[1] R4-2321621,WF on NR mobility enhancements (part 1), MediaTek.

[2] R4-2320774, On L1 measurement requirements of LTM, Ericsson.