3GPP TSG-RAN WG4 Meeting #109	R4-2402017

Chicago, US, Nov 13 – Nov 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Feasibility study of RRM TDCP accuracy requirements

Agenda item:	8.29.2.1

Document for:	Discussion

Introduction

In this paper we discuss Nokia’s view on TDCP RRM requirements.

TDCP accuracy requirements discussion

In the last RAN4 meeting the following options we discussed regarding TDCP requirements

Issue 1-1-1: Is it feasible to define TDCP accuracy requirement for TDCP? Agreement: - No RRM core requirement to be defined, and further discuss the following options: - Option 1: Define accuracy and test cases as part of performance requirement - Option 2: Do not define accuracy requirement but define test cases for particular configurations as part of performance requirements - Option 3: Do not define accuracy requirement and test cases as part of performance requirements

In this session we present simulation results with different configurations. The purpose of this analysis is to find the best combination of parameters that would result in good discrimination between Doppler of 30 Hz, indicating low mobility, and 200 Hz, indicating high mobility.

| - No averaging | - 2 samples averaging |
| --- | --- |
| - 4 samples averaging | - 8 samples averaging |

Figure 1 CDF for TDCP reports with different averaging

Figure 1 shows one example of how averaging is helping for the discrimination of low and high mobility. In the examples above, there is a high overlap between low Doppler and High Doppler TDCP statistics when 2 samples are used. This indicates that mobility detection would provide many errors when using magnitude and only 2 samples averaging. On the other hand, when the averaging is increased to 4 or 8 samples the separation between high and low Doppler is much more pronounced, which indicates that 4 or 8 samples averaging is helping significantly for the detection of mobility using TDCP.

Define TDCP requirements with 4 samples averaging

| - D=1slot, 4 samples averaging | - D=10slots, 4 samples averaging |
| --- | --- |

Figure 2 CDF for TDCP reports with different lags between TRS samples

| - D=1slot, no averaging | - D=10slots, no averaging |
| --- | --- |

Figure 3 CDF for TDCP reports with SNR variation and different lags between TRS samples

Figure 2 shows one example comparing lags between TRS of 1 and 10 slots. From those plots we can notice that the overlap between 30 Hz and 200 Hz is reduced to a good extent when using D=10 slots. So far, the companies in RAN4 have only provided simulation results with D=1. However, the results comparing D=1 and D=10 show that this is an important parameter to improve the mobility detection using TDCP. More gain of adding higher delay between TRS lags observed in the presence of noise, Figure 3. As multiple companies noticed, high doppler cases experience less effect of the noise, the same is effect is present for lower Doppler cases when delay between lags is increased. Therefore, we suggest using D=10 slots.

With lag of 1 slot, there is no significant difference in TDCP reports with low and high Doppler.

Lower noise effect is experienced for higher Doppler and longer delays Di between lags.

With lag of 10 slots, the difference in TDCP reports with low and high Doppler is increased in comparison to lag of 1.

| - D=1slot, no averaging | - D=10slots, no averaging |
| --- | --- |
| - D=1slot, averaging=4 | - D=10slots, averaging=4 |

Figure  Polar plot showing class separation

In  we present polar plots for representing magnitude and phase of the TDCP reports for 100 samples of high and low Doppler. The blue samples are the TDCP reports for Doppler=30 Hz, whereas the red samples are the TDCP reports for Doppler=200 Hz. In this figure we can notice that magnitude in combination with D=10 slots is helping greatly to distinguish between TDCP samples for high and low mobility. In the CDF plots shown in Figure 1 and  we can notice that only with magnitude there is a significant overlap between the CDFs for Doppler = 30 Hz and 300 Hz. On the other hand, the polar plot shows that by combining D=10 slots, averaging and the phase information, the detection error of mobility could be significantly reduced. Therefore, we believe that RAN4 should also consider the phase information for the TDCP reports.

With lag of 10 slots, the difference in TDCP reports with low and high Doppler is increased in comparison to lag of 1.

Use of phase in combination with magnitude increases the differentiation between low and hight Doppler using TDCP reports.

Define TDCP requirements with lag of 10 slots

Define TDCP requirements with amplitude and phase

Conclusion

Within this contribution we RRM requirements for TDCP. As part of this discussion we have derived the following observations and proposals:

Proposal 1: Define TDCP requirements with 4 samples averaging

Observation 4: Use of phase in combination with magnitude increases the differentiation between low and hight Doppler using TDCP reports.

Proposal 2: Define TDCP requirements with lag of 10 slots

Proposal 3: Define TDCP requirements with amplitude and phase

- References

- R4-2316401 - Discussion on the TDCP measurement accuracy requirements (Nokia, RAN4#108bis)

- Appendix A: Simulation results

Simulation parameters

The following simulation parameters were fronted in the RAN4#108bis as a starting point:

Table 2: Simulation parameters

| Parameter | Value |
| --- | --- |
| Delay (between TRS symbols) | 1 slot |
| Channel model | TDL-A, delay spread=30ns |
| Doppler Spread | 10, 30, 75, 100, 200, 300 |
| SNR | 5:5:20 |
| Number of averaging samples: | one shot |
| Channel BW | 10MHz |
| SCS | 30KHz |
| Reference Channel estimation | LS base CE for TRS as baseline |
| Correlation matrix and antenna configuration | 1x2 Low |

As a preliminary position, these parameters are reasonable for use with simulating TDCP accuracy requirements.

Simulation results

Using the simulation parameters above, the following results were obtained. Selected results are provided bellow.

| Doppler | SNR | 10% |  |  |  |  |  | 90% |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 slot |  |  | 10 slots |  |  | 1 slot |  |  | 10 slots |  |  |
|  |  | 1 samp. | 2 samp. | 4 samp. | 1 samp. | 2 samp. | 4 samp. | 1 samp. | 2 samp. | 4 samp. | 1 samp. | 2 samp. | 4 samp. |
| 10 | 5 | 0,44 | 0,44 | 0,43 | 0,43 | 0,44 | 0,45 | 0,88 | 0,87 | 0,87 | 0,88 | 0,87 | 0,86 |
|  | 10 | 0,71 | 0,71 | 0,71 | 0,70 | 0,70 | 0,71 | 0,96 | 0,96 | 0,96 | 0,96 | 0,95 | 0,95 |
|  | 15 | 0,89 | 0,89 | 0,89 | 0,87 | 0,87 | 0,88 | 0,99 | 0,99 | 0,99 | 0,99 | 0,98 | 0,98 |
|  | 20 | 0,96 | 0,96 | 0,96 | 0,95 | 0,95 | 0,95 | 1,00 | 1,00 | 1,00 | 0,99 | 0,99 | 0,99 |
| 30 | 5 | 0,44 | 0,44 | 0,43 | 0,39 | 0,43 | 0,49 | 0,88 | 0,87 | 0,87 | 0,87 | 0,85 | 0,79 |
|  | 10 | 0,71 | 0,71 | 0,71 | 0,63 | 0,65 | 0,70 | 0,96 | 0,96 | 0,96 | 0,95 | 0,94 | 0,91 |
|  | 15 | 0,89 | 0,88 | 0,88 | 0,78 | 0,78 | 0,82 | 0,99 | 0,99 | 0,99 | 0,98 | 0,97 | 0,96 |
|  | 20 | 0,96 | 0,96 | 0,96 | 0,85 | 0,84 | 0,86 | 1,00 | 1,00 | 1,00 | 0,99 | 0,99 | 0,97 |
| 75 | 5 | 0,43 | 0,44 | 0,44 | 0,26 | 0,29 | 0,34 | 0,88 | 0,87 | 0,87 | 0,83 | 0,72 | 0,63 |
|  | 10 | 0,71 | 0,71 | 0,71 | 0,39 | 0,38 | 0,43 | 0,96 | 0,96 | 0,95 | 0,92 | 0,84 | 0,75 |
|  | 15 | 0,88 | 0,88 | 0,88 | 0,47 | 0,41 | 0,47 | 0,99 | 0,99 | 0,98 | 0,95 | 0,89 | 0,81 |
|  | 20 | 0,95 | 0,95 | 0,95 | 0,50 | 0,43 | 0,49 | 1,00 | 1,00 | 0,99 | 0,97 | 0,91 | 0,84 |
| 100 | 5 | 0,43 | 0,44 | 0,45 | 0,23 | 0,19 | 0,19 | 0,88 | 0,87 | 0,86 | 0,79 | 0,68 | 0,57 |
|  | 10 | 0,70 | 0,70 | 0,71 | 0,34 | 0,23 | 0,23 | 0,96 | 0,95 | 0,95 | 0,89 | 0,80 | 0,69 |
|  | 15 | 0,87 | 0,87 | 0,88 | 0,39 | 0,26 | 0,25 | 0,99 | 0,98 | 0,98 | 0,93 | 0,85 | 0,74 |
|  | 20 | 0,95 | 0,95 | 0,95 | 0,42 | 0,27 | 0,25 | 1,00 | 0,99 | 0,99 | 0,95 | 0,88 | 0,77 |
| 200 | 5 | 0,42 | 0,43 | 0,48 | 0,23 | 0,14 | 0,11 | 0,87 | 0,86 | 0,83 | 0,77 | 0,63 | 0,50 |
|  | 10 | 0,67 | 0,68 | 0,71 | 0,32 | 0,17 | 0,14 | 0,95 | 0,95 | 0,94 | 0,87 | 0,74 | 0,59 |
|  | 15 | 0,84 | 0,84 | 0,85 | 0,37 | 0,19 | 0,16 | 0,98 | 0,98 | 0,97 | 0,92 | 0,79 | 0,64 |
|  | 20 | 0,91 | 0,90 | 0,91 | 0,40 | 0,20 | 0,16 | 0,99 | 0,99 | 0,99 | 0,93 | 0,81 | 0,66 |
| 300 | 5 | 0,39 | 0,43 | 0,49 | 0,23 | 0,14 | 0,11 | 0,87 | 0,85 | 0,79 | 0,77 | 0,61 | 0,47 |
|  | 10 | 0,63 | 0,65 | 0,70 | 0,32 | 0,18 | 0,14 | 0,95 | 0,94 | 0,91 | 0,87 | 0,72 | 0,57 |
|  | 15 | 0,78 | 0,78 | 0,82 | 0,37 | 0,19 | 0,16 | 0,98 | 0,97 | 0,96 | 0,92 | 0,78 | 0,61 |
|  | 20 | 0,85 | 0,84 | 0,87 | 0,39 | 0,19 | 0,16 | 0,99 | 0,99 | 0,97 | 0,93 | 0,80 | 0,63 |