3GPP TSG-WG4 Meeting #110 	R4-2402021

# Athens, Greece, February 26 - March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR correcting TCI switching requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections of TCI switching requirements for MIMO evo |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of the names of information elements which are not existing in RAN2 specification. Those include - DLorJointTCIState, which is replaced by dl-OrJointTCI-StateList - UL-TCIState, which is replaced by TCI-UL-State Behavior when SSBs are adjacent in mDCI - Definition of the behavior with overlaping SSBs in FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect TCI switching requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.21, 8.22, 8.23, 8.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change #1>

## 8.21	Active downlink TCI state switching delay for unified TCI for single-DCI mTRP

### 8.21.1	Introduction

The requirements in this clause apply for a UE configured with dl-OrJointTCI-StateList configurations for DL channels on a serving cell and UE reports its capability of S-DCI based MTRP. Further the requirements also apply for all the list of serving cells in simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 in MR-DC or standalone NR provided all serving cells in the list are configured with the same serving cell and BWP in unifiedTCI-StateRef-r17. UE shall complete the switch of active downlink TCI state within the delay defined in this clause.

### 8.21.2	Known conditions for downlink TCI state

The downlink TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state

-	Downlink TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target downlink TCI state before the downlink TCI state switch command

-	The target downlink TCI state remains detectable during the downlink TCI state switching period

-	The SSB associated with the downlink TCI state remain detectable during the downlink TCI switching period

-	SNR of the downlink TCI state ≥ -3dB

-	The SSB can be associated with the serving cell PCI.

Otherwise, the downlink TCI state is unknown.

### 8.21.3	MAC-CE based downlink TCI state switch delay

The requirements in this clause shall apply for DL TCI state switch using separate DL TCI state or joint TCI state of unified TCI state switch framework.

In case of joint TCI state switch, if the target PL-RS is not maintained, UE is not expected to receive on DL based on the target TCI state before UE completes the DL and UL TCI state switch.

When a MAC CE for DL TCI state switch indicates single TCI state switch, requirements specified in clause 8.15.3 apply.

When a MAC CE for DL TCI state switch indicates dual TCI state switch,

- If both the target TCI states are known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ max{TOk1*(Tfirst-SSB1 + AD1*TSSB1  + TSSB-proc), TOk2*(Tfirst-SSB2 + AD2*TSSB2 + TSSB-proc)} / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ where THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3];

-	Tfirst-SSB1 is time to the first SSB transmission of the first TCI state of the pair of TCI states after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state; Tfirst-SSB2 is time to the first SSB transmission of the second TCI state of the pair of TCI states after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state;

-	TSSB-proc = 2 ms;

-	TOk1 = 1 if first target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise.

-	TOk2 = 1 if second target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise.

-	AD1 = AD2 = 1 if SSBs are adjacent in FR2 ; 0 otherwise

NOTE:	Since this clause refers to target TCI states with the same PCI, it is assumed that TSSB1 =TSSB2.

Among the dual target TCI states indicated for switch, if one of the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot THARQ + 3Nslotsubframe,µ + TL1-RSRP1 + max {TOuk1*(Tfirst-SSB1+ AD1*TSSB1 + TSSB-proc), TOk2*(Tfirst-SSB2 + TSSB-proc)} / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .

-	AD1 = 1 if SSBs are adjacent in FR2; 0 otherwise

If the both of the dual target TCI states are unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + max{TL1-RSRP1 +TOuk1*(Tfirst-SSB1+ TSSB-proc), T L1-RSRP2 + TOuk2*(Tfirst-SSB2+ TSSB-proc)} / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .

Where

-	T L1-RSRP1 = 0 for the first TCI state and T L1-RSRP2 = 0 for the second TCI state in FR1 or when the TCI state switching not involving QCL-TypeD in FR2. Otherwise,

-	T L1-RSRP1 for the first TCI state and T L1-RSRP2 for the second TCI state are the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

-	TOuk1 for the first TCI state and TOuk2 for the second TCI state,  TOuk1 = 1 and TOuk2 = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement when TCI state switching involves QCL-TypeD

-	TOuk1 = 1 and TOuk2 = 1when TCI state switching involves other QCL types only

-	Tfirst-SSB1 is time to the first SSB transmission of the first TCI state of the pair of TCI states after L1-RSRP measurement when TCI state switching involves QCL-TypeD; Tfirst-SSB2 is time to the first SSB transmission of the second TCI state of the pair of TCI states after L1-RSRP measurement when TCI state switching involves QCL-TypeD;

-	Tfirst-SSB1 is time to the first SSB transmission of the first TCI state of the pair of TCI states after MAC CE command is decoded by the UE for other QCL types; Tfirst-SSB2 is time to the first SSB transmission of the second TCI state of the pair of TCI states after MAC CE command is decoded by the UE for other QCL types;

-	The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

Editor Note: for FR2 both of the dual target TCI state are unknown when SSB are adjacent, Longer delay is expected or one SSB period extension is needed.

### 8.21.4	DCI based downlink TCI state switch delay

When a UE is configured with the higher layer parameter with dl-OrJointTCI-StateList orTCI-UL-State, activated with TCI states for downlink transmission by MAC CE indication of more than one codepoints, and receives DCI format 1_1/1_2 with or without DL assignment providing indicated two TCI-States or TCI state pair in the active TCI list for a CC, the UE transmits a PUCCH with HARQ-ACK information corresponding to the DCI carrying the TCI-State indication. Requirements in clause 8.15.4 apply.

### 8.21.5	Active Downlink TCI state list update delay

The requirements specified in this clause are applicable if

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, and a MAC CE activates more than one target separate TCIs, and at least one DL TCI is included, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, and a MAC CE activates more than one target joint TCI.

If all the target TCI states in the active TCI state list are known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive  PDCCH or PDSCH with the new target TCI states at the first slot that is after

n + THARQ +3Nslotsubframe,µ +  TOk*(Tfirst-SSB_List + TSSB-proc) / NR slot length.

If a subset of the target TCI states in the active TCI state list are unknown, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive UE-dedicated PDCCH or PDSCH with the new target TCI states at the first slot that is after

n+ THARQ +3Nslotsubframe,µ + (TL1-RSRP_list +TOuk*(Tfirst-SSB_List+ TSSB-proc)) / NR slot length.

If all target TCI states in the active TCI state list are unknown, the requirements specified in this clause are not applicable.

Where

-	TL1-RSRP_List is the longest L1 measurement time (TL1-RSRP) of the source RS among the unknown target TCI states, where TL1-RSRP is specified in clause 8.21.3

-	Tfirst-SSB_List is the Tfirst-SSB from serving cell

-	THARQ, Tfirst-SSB, TSSB-proc , TOk, TOuk are defined in clause 8.21.3. TSSB is the SSB periodicity.

When UE receives PDSCH carrying MAC-CE for active TCI state list update, and

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, while the target TCI list comprises at least one DL TCIs and at least one UL TCIs,

UE is not expected to receive on DL based on the target TCI state before UE completes the DL and UL TCI state list update.

## 8.22	Active downlink TCI state switching delay for unified TCI for multi-DCI mTRP

### 8.22.1	Introduction

The requirements in this clause apply for a UE configured with two different values of CORESETPoolIndex in ControlResourceSet with dl-OrJointTCI-StateList configurations for DL channels on a serving cell. Further the requirements also apply for all the list of serving cells in simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 in MR-DC or standalone NR provided all serving cells in the list are configured with the same serving cell and BWP in unifiedTCI-StateRef-r17. UE shall complete the switch of active downlink TCI state within the delay defined in this clause. The requirements apply when UE is not expected to receive simultaneously from two different QCL-D sources in FR2. The requirements are applicable to each TRP independently.

When the target DL TCI state refers to an additional PCI different from the serving cell PCI in which this DL TCI-State is configured, the requirements in this clause are applicable provided that following conditions are met:

-	Active BWP of the serving cell and a cell with the additional PCI are the same

-	Center frequency, SCS and SFN offset of a cell with the additional PCI are as the same as serving cell - 	The cell with the additional PCI is known to the UE.

A cell with the additional PCI is known if the following conditions are met:

-	During the last 5s before L1-RSRP measurement is configured, the UE has sent a valid L3 measurement report for the cell with the additional PCIY

-	Timing offset between serving cell and the cell with the additional PCI is within CP of the corresponding SCS, except when UE supports RTD>CP with 2 TAs

Otherwise, the cell with the additional PCI is unknown.

### 8.22.2	Known conditions for downlink TCI state

The downlink TCI state is known if the conditions in section 8.15.2 are met.

### 8.22.3	MAC-CE based downlink TCI state switch delay

If only one TCI state is switched when the UE is operating in mDCI mode, requirements specified in clause 8.15.3 are applicable.

The requirements in this clause shall apply for DL TCI state switch using separate DL TCI state or joint TCI state of unified TCI state switch framework.

In case of joint TCI state switch, if the target PL-RS is not maintained, UE is not expected to receive on DL based on the target TCI state before UE completes the DL and UL TCI state switch.

If the target TCI state is known,

- 	If the UE is not configured with 2 TAs, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc+ OL*TSSB) / NR slot length.

-	If the UE is configured with 2 TAs in FR1 or configured with 2 TAs in FR2 and doesn’t support RTD>CP, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc+ OL*TSSB) / NR slot length.

-	The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ where THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3];

-	Tfirst-SSB is the time to the first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	TSSB-proc = 2 ms;

-	TOk  = 0 if the target TCI state have QCL relationship with a RS of a TCI state in the active list of TCI states or if the target TCI state is in the active list of TCI states, otherwise TOk = 1;

-	OL =1 if the SSB overlaps or is adjacent to the SSB from the other TRP in FR2 and the SSB periodicity is less than that of the other TRP; OL=1 if the SSB overlaps or is adjacent to the SSB from the other TRP in FR2 and the SSB periodicity is the same than that of the other TRP and the SSB is associated to the TRP with the lowest coresetPoolIndex; OL = 0 otherwise

If the target TCI state is unknown,

- If the UE is not configured with 2 TAs, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + (TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc+ OL*TSSB)) / NR slot length.

- If the UE is configured with 2 TAs in FR1 or configured with 2 TAs in FR2 and doesn’t support RTD>CP, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + (TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc+ OL*TSSB)) / NR slot length.

- The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .

Where

-	T L1-RSRP = 0 in FR1 or when the TCI state switching not involving QCL-TypeD in FR2. Otherwise,

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1, when receive timing difference is within CP,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2, when receive timing difference is within CP

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

Editor’s note: FFS the L1-RSRP measurement requirement when RTD>CP.

-	TOuk = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement when TCI state switching involves QCL-TypeD

-	TOuk = 1 when TCI state switching involves other QCL types only

-	Tfirst-SSB is time to first SSB transmission after L1-RSRP measurement when TCI state switching involves QCL-TypeD;

-	Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE for other QCL types;

-	The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

### 8.22.4	DCI based downlink TCI state switch delay

For DCI based downlink TCI state switch delay in m-DCI mode of operation, the requirements in section 8.15.4 apply to TCI state associated with each CORESETpoolIndex.

### 8.22.5	Active Downlink TCI state list update delay

The requirements specified in this clause are applicable if

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, and a MAC CE activates more than one target separate TCIs, and at least one DL TCI is included, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, and a MAC CE activates more than one target joint TCI.

If all the target TCI states in the active TCI state list are known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive PDCCH or PDSCH with the new target TCI states at the first slot that is after

n + THARQ +3Nslotsubframe,µ +  TOk*(Tfirst-SSB_List + TSSB-proc) / NR slot length.

If a subset of the target TCI states in the active TCI state list are unknown, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive UE-dedicated PDCCH or PDSCH with the new target TCI states at the first slot that is after

n+ THARQ +3Nslotsubframe,µ + (TL1-RSRP_list +TOuk*(Tfirst-SSB + TSSB-proc)) / NR slot length.

If all target TCI states in the active TCI state list are unknown, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive UE-dedicated PDCCH or PDSCH with the new target TCI states at the first slot that is after

n+ THARQ +3Nslotsubframe,µ + (TL1-RSRP_list +TOuk*(Tfirst-SSB + TSSB-proc)) / NR slot length.

Editor’s note: FFS how to take into account overlapping MAC-CE based switches from two TRPs where the SSB associated to each TCI state is overlapping or adjacent to each other.

Where

-	TL1-RSRP_List is the longest L1 measurement time (TL1-RSRP) of the source RS among the unknown target TCI states, where TL1-RSRP is specified in clause 8.22.3

Editor’s note: FFS the L1-RSRP measurement requirement when RTD>CP.

-	THARQ, Tfirst-SSB, TSSB-proc, TOk, TOuk are defined in clause 8.22.3. TSSB is the SSB periodicity.

When UE receives PDSCH carrying MAC-CE for active TCI state list update, and

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, while the target TCI list comprises at least one DL TCIs and at least one UL TCIs,

UE is not expected to receive on DL based on the target TCI state before UE completes the DL and UL TCI state list update.

## 8.23	Active uplink TCI state switching delay for unified TCI for single-DCI mTRP

### 8.23.1	Introduction

The requirements in this clause apply for a UE configured with dl-OrJointTCI-StateList (if unifiedTCI-StateType is indicated as Joint) or TCI-UL-State configurations for UL channels/signals on a serving cell. Further the requirements also apply for all the list of serving cells in simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 in MR-DC or standalone NR. There is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS. UE shall complete the switch of active uplink TCI state within the delay defined in this clause when the UE is requested to switch to a TCI state with the higher layer parameter dl-OrJointTCI-StateList or TCI-UL-State associated to a DL RS.

PL-RS may be associated with or included in UL TCI state or joint TCI state. The requirements in this clause shall apply if the following conditions are met:

-	PL-RS is identical to source RS in UL TCI state or joint TCI state

-	PL-RS and source RS in UL TCI state or joint TCI state are QCL-Type D

### 8.23.2	Known conditions for uplink TCI state

The uplink TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target uplink TCI state to the completion of active uplink TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target uplink TCI state or QCLed to the target uplink TCI state

-	Uplink TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target uplink TCI state before the uplink TCI state switch command

-	The RS configured in target uplink TCI state remains detectable during the uplink TCI state switching period

-	SNR of the RS configured in target uplink TCI state ≥ -3dB

-	The target uplink TCI state remains detectable during the uplink TCI state switching period

-	The SSB associated with the uplink TCI state remain detectable during the uplink TCI switching period

-	SNR of the uplink TCI state ≥ -3dB

Otherwise, the uplink TCI state is unknown.

### 8.23.3	MAC-CE based uplink TCI state switch delay

The requirements in this clause shall apply for UL TCI state switch using separate UL TCI state or joint TCI state of unified TCI state switch framework and having two indicated TCI-States.

In case of joint TCI state switch, UE is not expected to transmit on UL before UE completes the DL and UL TCI state switch.

For separate UL TCI state switch or joint TCI state switch for PUCCH or PUSCH, or semi-persistent/aperiodic/periodic SRS, when beamCorrespondenceWithoutUL-BeamSweeping is set to 1, upon receiving PDSCH carrying MAC-CE activation command in slot n on serving cell,

If both target TCI states are known,

-	The UE shall be able to transmit uplink signal with the target TCI state(s) in the slot n+THARQ + 3Nslotsubframe,µ + max{NM1* (Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2ms), NM2* (Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS 2+ 2ms) }/ NR slot length.

If both target TCI states are unknown,

-	The UE shall be able to transmit uplink signal with the target TCI state(s) in the slot n+THARQ + 3Nslotsubframe,µ + max{TL1-RSRP1 + Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2ms, TL1-RSRP2 + Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS 2+ 2ms }/ NR slot length.

If one of the target TCI states is unknown and the other TCI state is known,

-	The UE shall be able to transmit uplink signal with the target TCI state(s) in the slot n+THARQ + 3Nslotsubframe,µ + max{ TL1-RSRP1 + Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2ms, NM2* (Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS 2+ 2ms) }/ NR slot length.

Where,

-	THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	NM1 = 1, if the target PL-RS of the first TCI state is not maintained by the UE, 0 otherwise.

-	NM2 = 1, if the target PL-RS of the second first TCI is not maintained by the UE, 0 otherwise.

In FR2, in case that the target PL-RS associated with or included in the target UL or joint TCI state is SSB, the requirements in this clause shall apply when this target PL-RS is maintained by the UE.

-	PL-RS is maintained provided:

-		the target PL-RS is associated with or included in the UL or joint TCI states in the active TCI list for PUSCH/PUCCH/SRS transmissions

-	There are no more than 4 different RS activated as PL-RS per serving cell among all active UL TCI states (UL or joint TCI state) for PUSCH/PUCCH/SRS transmissions

-	The target pathloss reference signal remains detectable during TCI state switching period

-	SNR of the target pathloss reference signal≥-3dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the TCI state switching period.

-	SNR of the associated SSB ≥-3dB

-	Tfirst_target-PL-RS1 is time to first pathloss RS transmission of the first TCI state after L1-RSRP measurement when first target TCI state is unknown.

-	Tfirst_target-PL-RS2 is time to first pathloss RS transmission of the second TCI state after L1-RSRP measurement when second target TCI state is unknown.

-	Tfirst_target-PL-RS1 is time to first pathloss RS transmission of the first TCI state after MAC CE command is decoded by the UE when the first target TCI state is known.

-	Tfirst_target-PL-RS2 is time to first pathloss RS transmission of the second TCI state after MAC CE command is decoded by the UE when the first seond TCI state is known.

-	Ttarget_PL-RS1 is the periodicity of the target pathloss reference signal of the first TCI state which would SSB or NZP CSI-RS when PL-RS is associated with serving cell.

-	Ttarget_PL-RS2 is the periodicity of the target pathloss reference signal of the second TCI state which would SSB or NZP CSI-RS when PL-RS is associated with serving cell.

-	T L1-RSRP1 and T L1-RSRP2 are the time for Rx beam refinement in FR2 for the first and second TCI state, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

The requirements specified in this clause are applicable if no more than 4 different RSs are activated as PL-RS per serving cell among all active UL (or joint) TCI states.

### 8.23.4	DCI based uplink TCI state switch delay

When a UE is configured with the higher layer parameter with dl-OrJointTCI-StateList or TCI-UL-State, activated with TCI states for uplink transmission by MAC CE indication of more than one codepoints, and receives DCI format 1_1/1_2 with or without DL assignment providing indicated two TCI-States in the active TCI list for a CC, the UE transmits a PUCCH with HARQ-ACK information corresponding to the DCI carrying the TCI-State indication. Requirements in 8.16.4 apply.

### 8.23.5	Active uplink TCI state list update delay

The requirements specified in this clause are applicable if

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, and a MAC CE activates more than one target separate TCIs, and at least one DL TCI is included, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, and a MAC CE activates more than one target joint TCI.

If all the target TCI states in the active TCI state list are known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to be able to transmit PUCCH, PUSCH or SRS with the new target TCI states at the first slot that is after

n + THARQ + 3Nslotsubframe,µ + NM * (Tfirst_target-PL-RS + 4 * Ttarget_PL-RS + 2ms) / NR slot length.

If a subset of the target TCI states in the active TCI state list are unknown, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to transmit PUCCH, PUSCH or SRS with the new target TCI states at the first slot that is after

n+THARQ + 3Nslotsubframe,µ+ (TL1-RSRP_List + Tfirst_target-PL-RS + 4*Ttarget_PL-RS+ 2ms) / NR slot length.

If all target TCI states in the active TCI state list are unknown, the requirements specified in this clause are not applicable.

Where

-	TL1-RSRP_List is the longest L1 measurement time (TL1-RSRP) of the source RS among the unknown target TCI states, where TL1-RSRP is specified in clause 8.23.2

-	THARQ, Tfirst_target-PL-RS, Ttarget-PL-RS are defined in clause 8.23.3.

When UE receives PDSCH carrying MAC-CE for active TCI state list update, and

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, while the target TCI list comprises at least one DL TCIs and at least one UL TCIs,

UE is not expected to transmit on UL based on the target TCI before UE completes the DL and UL TCI list update.

The requirements in this clause are applicable when the source RS of the active UL TCI state is a DL-RS and this DL-RS is included as one of the source RSs in the DL active TCI list.

## 8.24	Active uplink TCI state switching delay for unified TCI for multi-DCI mTRP

### 8.24.1	Introduction

The requirements in this clause apply for a UE configured with dl-OrJointTCI-StateList (if unifiedTCI-StateType is indicated as Joint) or TCI-UL-State configurations for UL channels/signals on a serving cell and when UE is configured two different values of CORESETPoolIndex in ControlResourceSet. Further the requirements also apply for all the list of serving cells in simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 in MR-DC or standalone NR provided all serving cells in the list are configured with the same serving cell and BWP in unifiedTCI-StateRef-r17. There is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS. UE shall complete the switch of active uplink TCI state within the delay defined in this clause when the UE is requested to switch to a TCI state with the higher layer parameter dl-OrJointTCI-StateList or TCI-UL-State associated to a DL RS. The requirements apply when UE is not expected to receive simultaneously with multi-RX in FR2. The requirements are applicable to each TRP independently.

PL-RS may be associated with or included in UL TCI state or joint TCI state. The requirements in this clause shall apply if either of the following conditions are met:

-	PL-RS is identical to source RS in UL TCI state or joint TCI state

-	PL-RS and source RS in UL TCI state or joint TCI state are QCL-Type D

When the target UL TCI state refers to an additional PCI different from serving cell PCI in which this UL TCI state is configured, the requirements in this clause are applicable provided that the following conditions are met:

-	Active BWP of the serving cell and a cell with the additional PCI are the same

-	Center frequency, SCS and SFN offset of a cell with the additional PCI are as the same as serving cell

-	The cell with the additional PCI is known to the UE.

A cell with the additional PCI is known if the following conditions are met:

-	During the last 5s before L1-RSRP measurement is configured, the UE has sent a valid L3 measurement report for the cell with the additional PCI

-	Timing offset between serving cell and the cell with the additional PCI is within CP of the corresponding SCS, except when UE supports RTD>CP with 2 TAs

-	The SSB from the cell with the additional PCI remains detectable according to the cell identification requirements specified in clause 9.2

Otherwise, the cell with the additional PCI is unknown.

### 8.24.2	Known conditions for uplink TCI state

The uplink TCI state is known if the conditions in section 8.16.2 are met.

### 8.24.3	MAC-CE based uplink TCI state switch delay

### If only one TCI state is switched when the UE is operating in mDCI mode, requirements specified in clause 8.16.3 are applicable.

The requirements in this clause shall apply for UL TCI state switch using separate UL TCI state or joint TCI state of unified TCI state switch framework.

In case that source RS in UL TCI state or joint TCI state is associated with a PCI different from that of the serving cell, the requirements in this clause shall apply if the cell with different PCI satisfies the known cell condition defined in 8.24.1. If the known cell condition is not met, longer delay may be expected.

In case of joint TCI state switch, UE is not expected to transmit on UL based on the target TCI state before UE completes the DL and UL TCI state switch.

For separate UL TCI state switch or joint TCI state switch for PUCCH or PUSCH, or semi-persistent/aperiodic/periodic SRS, when beamCorrespondenceWithoutUL-BeamSweeping is set to 1, upon receiving PDSCH carrying MAC-CE activation command in slot n on serving cell,

-	If target TCI state is known,

-	If UE is not configured with 2 TAs, the UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3Nslotsubframe,µ + NM* (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) / NR slot length.

-	If UE is configured with 2 TAs in FR1 or configured with 2 TAs in FR2 and doesn’t support RTD>CP, the UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3Nslotsubframe,µ + NM* (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) / NR slot length.

-	FFS on additional time tracking of DL Ref RS for 2TA TOk-ref *(Tfirst-SSB-DLRef + 2ms)

-	If target TCI state is unknown,

-	If UE is not configured with 2 TAs, the UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3Nslotsubframe,µ + (TL1-RSRP + Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) / NR slot length.

-	If UE is configured with 2 TAs in FR1 or configured with 2 TAs in FR2 and doesn’t support RTD>CP, the UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3Nslotsubframe,µ + (TL1-RSRP +  Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) / NR slot length.

-	The UE shall be able to transmit with the old UL TCI state until slot n+ THARQ + 3Nslotsubframe,µ.

-	FFS on additional time tracking of DL Ref RS for 2TA TOuk-ref (Tfirst-SSB-DLRef + 2ms)

Where,

-	THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise.

In FR2, in case that the target PL-RS associated with or included in the target UL or joint TCI state is SSB, the requirements in this clause shall apply when this target PL-RS is maintained by the UE.

-	PL-RS is maintained provided:

-	the target PL-RS is associated with or included in the UL or joint TCI states in the active TCI list for PUSCH/PUCCH/SRS transmissions

-	There are no more than 4 different RS activated as PL-RS per serving cell among all active UL TCI states (UL or joint TCI state) for PUSCH/PUCCH/SRS transmissions

-	The target pathloss reference signal remains detectable during TCI state switching period

-	SNR of the target pathloss reference signal≥-3dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the TCI state switching period.

-	SNR of the associated SSB ≥-3dB

-	Tfirst_target-PL-RS is time to first pathloss RS transmission after L1-RSRP measurement when target TCI state is unknown.

-	Tfirst_target-PL-RS is time to first pathloss RS transmission after MAC CE command is decoded by the UE for known TCI State.

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB or NZP CSI-RS when PL-RS is associated with serving cell

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB when PL-RS is associated with PCI different from serving cell

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1 or 9.13.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

-	TOk-ref = 1, if the target TCI state is known, and the RS of DL timing reference is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise.

-	Tfirst-SSB-DLRef is the time to first SSB after MAC CE command is decoded when target TCI state is known.

-	Tfirst-SSB-DLRef is the time to first SSB after L1-RSRP measurement when target TCI state is unknown.

-	TOuk-ref = 1, for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement

The requirements specified in this clause are applicable if no more than 4 different RSs are activated as PL-RS per serving cell among all active UL (or joint) TCI states.

### 8.24.4	DCI based uplink TCI state switch delay

When a UE is configured with the higher layer parameter with dl-OrJointTCI-StateList or TCI-UL-State, activated with TCI states for uplink transmission by MAC CE indication of more than one codepoints,  and receives DCI format 1_1/1_2 with or without DL assignment providing indicated TCI-State or TCI state pair in the active TCI list  for a CC, the UE transmits a PUCCH with HARQ-ACK information corresponding to the DCI carrying the TCI-State indication.

The requirements in this clause are applicable only if the DCI format indicating UL TCI state or joint TCI state switch is received by UE when,

-	target TCI state is known, and

-	target TCI state is in active TCI state list, and

-	target PL-RS is maintained as defined in clause 8.24.3

If the target TCI state is known, the uplink TCI switching to the indicated UL TCI state or joint TCI state in the DCI format shall be completed starting from the first slot that is at least beamAppTime-r17 symbols after the last symbol of the PUCCH carrying HARQ-ACK in response to the DCI triggering TCI state activation. The first slot and the beamAppTime-r17symbols are both determined on the carrier with the smallest SCS among the carrier(s) applying the beam indication. The value of beamAppTime-r17is defined in TS 38.331 [2]. The known condition for TCI state defined in clause 8.24.2 is applied.

If a PL-RS is associated with or included in UL TCI state or joint TCI state, the UL TCI switching and PL-RS switching shall be completed at the same time.

### 8.24.5	Active Uplink TCI state list update delay

The requirements specified in this clause are applicable if

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, and a MAC CE activates more than one target separate TCIs, and at least one UL TCI is included, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, and a MAC CE activates more than one target joint TCI.

If all the target TCI states in the active TCI state list are known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to transmit PUCCH, PUSCH or SRS with the new target TCI states at the first slot that is after

n + THARQ + 3Nslotsubframe,µ + NM * (Tfirst_target-PL-RS_List + 4 * Ttarget_PL-RS_List + 2ms) / NR slot length,

FFS on additional time tracking of DL Ref RS for 2TA

If a subset of target TCI states in the active TCI state list are unknown, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to transmit PUCCH, PUSCH or SRS with the new target TCI states at the first slot that is after

n+THARQ + 3Nslotsubframe,µ+ (TL1-RSRP_List + Tfirst_target-PL-RS_List + 4*Ttarget_PL-RS_List + 2ms) / NR slot length

FFS on additional time tracking of DL Ref RS for 2TA

If all target TCI states in the active TCI state list are unknown, the requirements specified in this clause are not applicable.

Where

-	If all TCIs are known,

-	if the target PL-RS associated with or included in any UL TCI is not maintained, NM = 1;  Where maintained PL-RS is defined in clause 8.16.3

-	if a subset of target TCI states in the active TCI state list are unknown,

-	TL1-RSRP_List = is the longest L1 measurement time (TL1-RSRP) of the source RS among the unknown target TCI states , where TL1-RSRP is specified in clause 8.16.3

-	if the number of cells associated with the target TCI states in the active TCI list is 2, and time to first PL-RS associated to the TCIs are overlapped in FR2,

-	Tfirst_target-PL-RS_List = Tfirst_target-PL-RS_SC + min(Ttarget-PL-RS_SC , Ttarget-PL-RS_CDP) ;

-	Ttarget-PL-RS_List = 2 * Ttarget-PL-RS_SC, if Ttarget-PL-RS_SC = Ttarget-PL-RS_CDP

-	Ttarget-PL-RS_List = max(Ttarget-PL-RS_SC , Ttarget-PL-RS_CDP), if  Ttarget-PL-RS_SC  ≠ Ttarget-PL-RS_CDP

-	if the number of cells associated with the target TCI states in the active TCI list is 2, and time to first PL-RS associated to the TCIs are not overlapped in FR2,

-	Tfirst_target-PL-RS_List = max (Tfirst_target-PL-RS_SC, Tfirst_target-PL-RS_SC);

-	Ttarget-PL-RS_List = max(Ttarget-PL-RS_SC , Ttarget-PL-RS_CDP)

-	Otherwise,

-	Tfirst_target-PL-RS_List = Tfirst_target-PL-RS_SC

-	Ttarget-PL-RS_List = Ttarget-PL-RS_SC.

-	Tfirst_target-PL-RS_SC is Tfirst_target-PL-RS for serving cell

-	Tfirst_target-PL-RS_CDP is Tfirst_target-PL-RS for cell with different PCI from serving cell

-	Ttarget-PL-RS_SC is Ttarget-PL-RS for serving cell

-	Ttarget-PL-RS_CDP is Ttarget-PL-RS for cell with different PCI from serving cell-	THARQ, Tfirst_target-PL-RS, Ttarget-PL-RS are defined in clause 8.16.3.

The requirements specified in this clause do not apply if more than 4 different RSs are activated as PL-RS per serving cell among all active UL (or joint) TCI states.

When UE receives PDSCH carrying MAC-CE for active TCI state list update, and

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, while the target TCI list comprises at least one DL TCIs and at least one UL TCIs,

UE is not expected to transmit on UL based on the target TCI before UE completes the DL and UL TCI list update.

The requirements in this clause are applicable when the source RS of the active UL TCI state is a DL-RS and this DL-RS is included as one of the source RSs in the DL active TCI list.

## <End of Change #1>