# 3GPP TSG-RAN WG4 Meeting # 110                                   R4-2402039

# Athens, Greece, 26 February – 1 March, 2024

# Title: 	Simulation results for BS demodulation performance for less than 5MHz

# Source: 	ZTE Corporation

# Agenda item:	8.10.6.2

# Document for:	Discussion

# 1 Introduction

This contribution contains initial simulation results for the <5MHz WID [1]. The results are provided to aid in determining where new requirements can/should be introduced.

# 2 Discussion

## 2.1 PUSCH requirements

Based on previous meeting’s WF [2], we evaluated the PUSCH demodulation performance between 15PRBs and 12PRBs in following table. In our contribution[3], we evaluated the PUSCH demodulation requirements for 25PRBs with 5MHz channel bandwidth.

Table 1. PUSCH simulation results for 15PRBs and 12PRBs

| Test num. | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  |  |  |  |  | 15PRBs | 12PRBs |
| 1 | MCS2 | TDLB100-400 | 1x2, ULA Low | 70 | -5.3 | -5.1 |
| 2 | MCS16 | TDLC300-100 | 1x2, ULA Low | 70 | 7.0 | 7.2 |
| 3 | MCS20 | TDLA30-10 | 1x2, ULA Low | 70 | 9.1 | 9.2 |

Observation 1. The simulation results indicate no significant difference between 15PRBs and 12PRBs. Additionally, there was no significant difference in performance observed between 3MHz and 5MHz or 10MHz.

## 2.2 PUCCH requirements

Based on the previous meeting’s WF [2], we evaluated the PUCCH format 2 demodulation performance under the following parameters.

Table 2. PUCCH format 2 simulation assumptions

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 3. PUCCH format 2 simulation results between 5MHz and 3MHz

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5MHz | 3MHz |
| 1 | 2 | Normal | TDLC-300-100 Low | -1.1 | -0.4 |

Figure 1.  PUCCH format 2 1T2R TDLC300-100 Low

# 3 Conclusion

In this contribution, we give some simulation results on demodulation performance requirements for less than 5MHz demodulation requirements, The conclusions are:

Observation 1. The simulation results indicate no significant difference between 15PRBs and 12PRBs. Additionally, there was no significant difference in performance observed between 3MHz and 5MHz or 10MHz.

# 4 References

- RP-222645, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, RAN#95e.

- R4-2321064, WF on NR_FR1_lessthan_5MHz_BW_demod, Nokia, Nokia Shanghai Bell.

- R4-2319544, Simulation results for BS demodulation performance for less than 5MHz. ZTE Corporation.