# 3GPP TSG-RAN WG4 Meeting # 110                                   R4-2402042

# Athens, Greece, 26 February – 1 March, 2024

# Title: 	Simulation results for MU-MIMO with R-ML receiver

# Source: 	ZTE Corporation

# Agenda item:	8.13.2.2

# Document for:	Approval

# 1 Introduction

- In RAN4#109 meeting, some conclusions has been reached for advanced receiver and a WF was agreed[1]. In this paper, we will provide our simulation results for MU-MIMO.

# 2 Discussion

# 2.1 Simulation assumptions

- The simulation assumptions are summarized in Table 2-1:

Table 2-1: Simulation assumptions

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 |  |
| Duplex mode |  |  | FDD |  |
| MCS |  |  | 4,13,17 | Rank 1+1: QPSK Rank 2+2: QPSK, 16QAM and 64QAM |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | Scrambling ID |  | Same scrambling ID for both UEs |  |
| MIMO configuration |  |  | For Rank 1+1: 2T2R ULA Medium For Rank 2+2: 4T4R ULA Low |  |
| Number of CDM groups without data |  |  | 1 for paired UE allocated in same CDM groups and 2 for paired UE allocated in different CDM groups |  |
| HARQ process number |  |  | 4 |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - Option 1: Select the precoding matrix to ensure orthogonality with target UE - Option 2: Select the precoding matrix randomly ensuring the selected precoding matrix shall not be identical to the precoding matrix of target UE |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | TDLC300-100,TDLA30-10 |  |
| Receiver type |  |  | MMSE-IRC,E-MMSE-IRC, R-ML | N/A |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |

  - 2.2 Simulation results

## 2.2.1 Without Blind detection

In this section, we provide the summary of link-level analysis of the co-schedule UE(s) without modulation order blind detection performance and advanced receiver throughput performance r in case of all information of co-schedule UE is known to target UE . The simulation results for the normalized throughput are illustrated below:

### 2.2.1.1 FDD

Table 2-2. Summary of simulation results

| Rank allocation | Modulation Order |  | Propagation conditions | PMI selection | Antenna configuration | R-ML |
| --- | --- | --- | --- | --- | --- | --- |
|  | Target UE | Co-schedule UE(s) |  |  |  | SNR (dB) |
| Rank1+1 | MCS 13 | QPSK | TDLC300-100 ULA Low | Random | 2T2R | 9.6 |
| Rank1+1 | MCS 13 | QPSK | TDLC300-100 ULA medium | Random | 2T2R | 12.7 |
| Rank2+2 | MCS 13 | QPSK | TDLA30-10 ULA Low | Orthogonal | 4T4R | 10.3 |
| Rank2+2 | MCS 17 | 16QAM | TDLA30-10 ULA Low | Orthogonal | 4T4R | 16.3 |

### 2.2.1.2 TDD

Table 2-3. Summary of simulation results

| Rank allocation | Modulation Order |  | Propagation conditions | PMI selection | Antenna configuration | R-ML |
| --- | --- | --- | --- | --- | --- | --- |
|  | Target UE | Co-schedule UE(s) |  |  |  | SNR (dB) |
| Rank1+1 | MCS 13 | QPSK | TDLC300-100 ULA Low | Random | 2T2R | 10.1 |
| Rank1+1 | MCS 13 | QPSK | TDLC300-100 ULA medium | Random | 2T2R | 13.6 |
| Rank2+2 | MCS 13 | QPSK | TDLA30-10 ULA Low | Orthogonal | 4T4R | 11.0 |
| Rank2+2 | MCS 17 | 16QAM | TDLA30-10 ULA Low | Orthogonal | 4T4R | 17.1 |

## 2.2.2 Blind detection

In this section, we provide the summary of link-level analysis of the co-schedule UE(s) modulation order blind detection performance and advanced receiver throughput performance. The simulation results for the normalized throughput are illustrated below:

### 2.2.1.1 FDD

Table 2-4. Summary of simulation results

| Rank allocation | Modulation Order |  | Propagation conditions | PMI selection | Antenna configuration | R-ML |
| --- | --- | --- | --- | --- | --- | --- |
|  | Target UE | Co-schedule UE(s) |  |  |  | SNR (dB) |
| Rank1+1 | MCS 13 | QPSK | TDLC300-100 ULA medium | Random | 2T2R | 12.7 |
| Rank1+1 | MCS 13 | QPSK | TDLC300-100 ULA Low | Random | 2T2R | 9.7 |

### 2.2.1.2 TDD

Table 2-5. Summary of simulation results

| Rank allocation | Modulation Order |  | Propagation conditions | PMI selection | Antenna configuration | R-ML |
| --- | --- | --- | --- | --- | --- | --- |
|  | Target UE | Co-schedule UE(s) |  |  |  | SNR (dB) |
| Rank1+1 | MCS 13 | QPSK | TDLC300-100 ULA medium | Random | 2T2R | 13.6 |
| Rank1+1 | MCS 13 | QPSK | TDLC300-100 ULA Low | Random | 2T2R | 10.9 |

# 3 Conclusion

In this contribution, we give some simulation results on demodulation performance requirements for MU-MIMO.

# 4 References

- R4-2321114, WF on advanced receiver for MU-MIMO scenario. China Telecom