3GPP TSG-RAN WG4 Meeting #110                                R4-2402269

Athens, Greece, 26 Feb – 01 Mar, 2024

Source:	Samsung, KDDI, Qualcomm

Title:	TP for TR 38.898: DC_1-41_n77

Agenda item:			7.16.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.898 to include DC_1-41_n77 according to the request in [1].

The corresponding 3-band PC3 combo has already been specified.

The 2-band PC2 DC_1_n77 and DC_41_n77 have already been specified.

Note that PC3 MSD for these 3-band combos had been introduced in Rel-15 while modified to N/A in Rel-16 which is likely a mistake, Qualcomm would bring a correction CR for PC3 MSD correction since Rel-16 in this meeting (Tdoc number may could not be provided timely before this TP submission) to pave the path for the PC2 introduction in this TP.

2. Reference

[1] RP-233977 Revised WID on High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL)

# 3. Text Proposal

<Start of Text Proposal>

### 5.x   DC_1-41_n77

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-41A_n77A14 DC_1A-41C_n77A14 | DC_1A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration. |  |

#### 5.x.2	Maximum output power for DC

Since the maximum output power requirements for DC_1_n77 have already been specified, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n77, own Rx impact of the 3rd band is the followings.

- 4th and 5th order IMD generated by dual uplink of band 1 and band n77 may also impact the own Rx of band 41.

The new MSDs are specified in below table.

Table 5.x.3-1: MSD test points for Scell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-41A_n77A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
| DC_1A-41C_n77A | 41 | N/A | 5 | N/A | 2510 | 22.5 | IMD4 |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 41 | N/A | 5 | N/A | 2510 | 15.6 | IMD5 |
|  | n77 | 4150 | 10 | 50 | 4150 | N/A | N/A |

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC.

<End of Text Proposal>