3GPP TSG-RAN WG4 Meeting #110                                R4-2402270

Athens, Greece, 26 Feb – 01 Mar, 2024

Source:	Samsung, KDDI, Qualcomm, LGE

Title:	TP for TR 38.898: DC_1A_n41A-n77A

Agenda item:			7.16.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.898 to include DC_1A_n41A-n77A according to the request in [1].

The corresponding 3-band PC3 combo has already been specified.

The 2-band PC2 DC_1A_n77A and DC_1A_n41A have already been specified.

Note that some MSD exception notes are missing for PC3 DC_1A_n41A-n77A, which is corrected in this meeting by formal correction CR R4-2402272 and R4-2402273.

2. Reference

[1] RP-233977 Revised WID on High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL)

# 3. Text Proposal

<Start of Text Proposal>

### 5.x   DC_1_n41-n77

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n41A-n77A14 | DC_1A_n41A14 DC_1A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration. |  |

#### 5.x.2	Maximum output power for DC

Since the maximum output power requirement for PC2 DC_1_n41 and DC_1_n77 have already been specified, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n41 and DC_1_n77, own Rx impact of the 3rd band is the followings.

- the 3rd, 4th, and 5th order IMD generated by dual uplink of band 1 and band n41 may also impact the own Rx of band n77.

- the 4th and 5th order IMD generated by dual uplink of band 1 and band n77 may also impact the own Rx of band n41.

The new MSDs are specified in below table.

Table 5.x.3-1: MSD test points for Scell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n41A-n77A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n41 | N/A | 10 | N/A | 2515 | 22.0 | IMD41 |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n41 | 2650 | 10 | 25 | 2650 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3330 | 28.2 | IMD31,5 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 5: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC.

<End of Text Proposal>