3GPP TSG RAN WG4 Meeting #110		 R4-2402312

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda Item:	8.15.1.1

Source: 	MediaTek Inc.

Title: 	(NR_MC_enh-Core)Discussion on the UE feature list for MC_enh

Document for:	Discussion

1. Introduction

In last RAN4#108-bis meeting, the LS for advanced UE capability for parallel Tx switching on four different bands  and draft CR were agreed[1][2]. The whole FG did not reach consensus in the working group. This contribution is re-submission of the MC_enh FG for parallel switching on four-band Tx switching.

2. Discussion

- NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-x | Preferred switching band pairs | Support the indication of UE’s preferred (switched-from, switched-to) band pairs for parallel UL transmission switching for a band combination consisting of four different bands | 38-1 [Rel-18 Tx switching] | Yes | No | Network can only assume the maximum switching period | Per BC | No | FR1 only | N.A |  | Optional with capability signalling |

To complete the Rel-18 work on multi-carrier enhancements for TX switching, the advanced UE capability for TX switching for a band combination consisting of four different bands is proposed.

3. Conclusion

Proposal 1:

- NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-x | Preferred switching band pairs | Support the indication of UE’s preferred (switched-from, switched-to) band pairs for parallel UL transmission switching for a band combination consisting of four different bands | 38-1 [Rel-18 Tx switching] | Yes | No | Network can only assume the maximum switching period | Per BC | No | FR1 only | N.A |  | Optional with capability signalling |

Reference:

- R4-2317608	Draft CR for 38.101-1: Time mask for switching across three or four uplink bands, RAN4#108-bis

- R4-2317609	LS on Rel-18 UL Tx switching for parallel switching on four bands, RAN4#108-bis