3GPP TSG-RAN WG4 Meeting #110														R4-2402326

Athens, Greece, 26 February – 1 March, 2024

Source:				Ericsson

Title:					LS Reply to RAN2 on UAV

Agenda item:		6.2.6

Document for:		Approval

# Introduction

RAN4 concluded the UAV specification for LTE ([1]) and NR ([2]) in last RAN4#109 meeting.

In the meantime, RAN2 sent a LS to RAN4 requesting clarification on the need of additionalPmax-r18 in NR-NS-PmaxValueAerial-r18

This contribution is discussing this aspect, proposing a reply to RAN2 LS in annex.

# Discussion

## Need for additionalPmax-r18

The additionalPmax in NR-NS-Pmaxvalue IE is used to overrule the cell-specific p-Max, providing additional flexibility to set the UE maximum output power.

We should keep the same flexibility with UAV UE and request than RAN2 to keep the additionalPmax-r18.

## Clarification

As discussed in past RAN4 meetings, RAN4 will still rely on the existing NS(s) specified for a band if the additional requirements in that band are the same for ”legacy” and UAV UEs.

RAN4 will then only specify new UAV NS(s) for a band:

- If additional requirement(s) exists for UAV and not for legacy UE.

- If the additional requirement(s) for UAV differs from the additional requirement(s) for legacy UE.

The consequence on the expected UAV UE behavior is then the following. When a UAV UE connects to a cell, it shall first:

- Check if there is any UAV NS broadcasted by the BS for the considered band.

- If there is at least one UAV NS:

  - The UAV UE shall support it.

  - If the UAV UE doesn’t support none of the UAV NS, it shall not connect.

- If there is no UAV NS, and only if there is no UAV NS:

  - The UAV UE shall still check if there is any ”legacy” NS for that same band.

While replying to RAN2 LS, it would be relevant to take that opportunity to clarify this expected UAV UE behavior.

# Conclusion

In this contribution, we discussed the RAN2 LS and proposed a reply in Annex.

Proposal: Approve the LS reply to RAN2 in Annex.

# Reference

- R4-2322001, Big CR for introduction of aerial UE for LTE, Nokia, Huawei, Ericsson, Qualcomm, Murata

- R4-2322011, Running CR to TS 38.101-1 - Introduction of Aerial UEs support, Ericsson, Nokia, Huawei, Murata, Qualcomm

# Annex : LS reply to RAN2 on UAV capabilities

3GPP TSG-RAN WG4 Meeting #110	R4-240xxxx

Athens, Greece, 26 February – 1 March 2024

Title:	LS Reply to RAN2 on UAV UE capabilities and NS values

Response to:	R2-2313949

Release:	Release 18

Work Item:	NR_UAV-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

#### Name:		Dominique Everaere

- E-mail Address:	dominique.everaere@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN2 for their LS R2-2313949.

RAN4 confirms that it has considered RAN2 decision on introducing an UAV capability to indicate that the UE supports Rel-18 UAV enhancements will be introduced.

Answering RAN2 question, RAN4 would like to clarify that the additionalPmax-r18 should be included in NR-NS-PmaxValueAerial-r18, this to provide the same functionality for UAV NR UE comparing to legacy NR UE.

RAN4 wishes to take this opportunity to share with RAN2 the following expected UAV UE behavior related to NS handling. The UAV UE shall first check if there is any UAV NS in the NR-NS-PmaxValueAerial-r18 for the considered band and, if (and only if) there is no NS, it shall then check if there is any NS in NR-NS-PmaxValue IE for that same band. RAN4 will indeed specify a new UAV NS only if there is additional requirement specific to UAV UE for a considered band.

Note that if there is an UAV NS in the NR-NS-PmaxValueAerial-r18, the UAV UE shall not check anymore if there is a NS in NR-NS-PmaxValue IE, even if it doesn’t support the UAV NS. In that later case (UAV NS not supported), the UAV UE shall not connect to the cell.

2. Actions:

To RAN WG2.

ACTION: 	RAN4 respectfully requests RAN2 to take into consideration the above information.

3. Date of Next TSG-RAN WG2 Meeting:

RAN4#110bis	from 2024-04-15	to 2024-04-19		Changsha, China

RAN4#111	from 2024-05-20	to 2024-05-25		Fukuoka, Japan