3GPP TSG-RAN WG4 Meeting #110                                 R4-2402356

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

Source:	Samsung, TELUS, Bell Mobility

Title:	TP for TR 38.899 to include HPUE CA_n71-n77 with UL CA_n77(2A)

Agenda item:			7.18.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TP for TR 38.899 to include HPUE CA_n71-n77. All fallbacks have been specified.

# 2. Reference

[1] RP-233561_Revised WID: Rel-18 High power UE (power class 1,5 and 2) for a single FR1 NR TDD band in UL of NR inter-band CA/DC combinations with/without NR SUL (supplementary uplink) with y bands downlink (y=2,3,4,5,6) and x bands uplink (x=1,2)

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	CA_n71-n77

### 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n71A-n77(2A) | CA_n77(2A)8 | n71 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n71A-n77(3A) | CA_n77(2A)8 | n71 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

### 5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY or (CA_nC) | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n77(2A) | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.x.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.x.3.1	Power class 2 case a, b

Based on calculation, IMD4/6 of dual UL CA_n77(2A) falls into n71 DL, the MSD values are defined as below.

Table 5.x.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n71-n77 | n71 | N/A | 5 | N/A | 640 | 18.5 | FDD | IMD414 |
|  | n7712 | 3480 | 10 | 1 (RBSTART=25) | 3480 | N/A | TDD | N/A |
|  |  | 3800 | 10 | 1 (RBSTART=25) | 3800 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 14: This band is subject to IMD6 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

<End of Text Proposal>