3GPP TSG-RAN4 Meeting #110	R4-2402360

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR | 0160 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | ❌ | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_CA_R17_Intra-Core) Rel-17 Cat F CR for 38.307 Correct and update the requirements for EN-DC and NR-CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia, ZTE Corporation, Huawei, HiSilicon, CHTTL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R16_1BLTE_1BNR_2DL2UL-Core, DC_R16_2BLTE_1BNR_3DL2UL-Core, DC_R16_3BLTE_1BNR_4DL2UL-Core, DC_R16_4BLTE_1BNR_5DL2UL-Core, DC_R16_5BLTE_1BNR_6DL2UL-Core, DC_R16_xBLTE_2BNR_yDL2UL-Core, NR_CA_R16_intra-Core, NR_CADC_R16_2BDL_xBUL-Core, NR_CA_R16_3BDL_1BUL-Core, NR_CADC_R16_3BDL_2BUL-Core, NR_CA_R16_4BDL_1BUL-Core, DC_R16_LTE_NR_3DL3UL-Core, DC_R16_xBLTE_2BNR_yDL3UL-Core, DC_R17_1BLTE_1BNR_2DL2UL-Core, DC_R17_2BLTE_1BNR_3DL2UL-Core, DC_R17_3BLTE_1BNR_4DL2UL-Core, DC_R17_4BLTE_1BNR_5DL2UL-Core, DC_R17_xBLTE_2BNR_yDL2UL-Core, DC_R17_xBLTE_yBNR_3DL3UL-Core, DC_R17_xBLTE_3BNR_yDL2UL-Core, NR_CA_R17_Intra-Core, NR_CADC_R17_2BDL_xBUL-Core, NR_CA_R17_3BDL_1BUL-Core, NR_CADC_R17_3BDL_2BUL-Core, NR_CA_R17_4BDL_1BUL-Core, NR_CADC_R17_4BDL_2BUL-Core, NR_CADC_R17_5BDL_xBUL-Core, DC_R17_5BLTE_1BNR_6DL2UL-Core, DC_R17_xBLTE_2BNR_yDL3UL-Core, DC_R17_xBLTE_4BNR_yDL2UL-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The requirements for band combinations in 38.307 need to be corrected and updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct and update the requirements for band combinations in 38.307. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for band combinations in 38.307 remain not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.7, 6.2, 7.1, 8.1, 8.2, B.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### << Start of changes >>

Table 5.2.1-1: NR intra-band contiguous CA within FR1

| Feature | DL/UL | CA BW Class | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- |
| Intra-band contiguous CA configurations within FR1 | DL | B, C, D, E, M, N, O | FDD,TDD | Rel-15 | Table B.4.2-1 |
|  | UL | A,B,C | FDD,TDD | Rel-15 |  |
| Intra-band contiguous CA configurations with UL MIMO within FR1 | UL | C | TDD | Rel-15 | Table B.4.2-2 |

### << Next changes >>

Table 5.2.1-2: NR intra-band non-contiguous CA within FR1

| Feature | DL/UL | number of sub-blocks | maximum number of CCs within a sub-block | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- |
| Intra-band non-contiguous CA configurations within FR1 | DL | 2 | 1 | FDD, TDD | Rel-15 | Table B.4.2-1 |
|  |  | 3 | 1 | FDD, TDD | Rel-15 |  |
|  |  | 4 | 1 | TDD | Rel-15 |  |
|  | UL | 1 | 1 | FDD, TDD | Rel-15 |  |
|  |  | 2 | 1 | TDD | Rel-15 |  |

### << Next changes >>

Table 5.2.2-1: NR inter-band CA within FR1

| Feature | DL/UL | Maximum number of bands | Maximum number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band CA configurations within NR FR1 | DL | 5 | 8 | A, B, C | TDD, FDD, SDL and FDD, SDL and TDD, FDD and TDD, FDD and SDL and TDD | Rel-15 | Table B.4.2-1 |
|  | UL | 2 | 2 | A | TDD, FDD, FDD and TDD | Rel-15 |  |
|  |  | 1 | 2 | B, C, 2A |  |  |  |
|  |  | 2 | 3 | A, B, C |  |  |  |

### << Next changes >>

Table 5.7-1: NR CA, EN-DC, and NR DC configurations involving shared spectrum access

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see 38.307 of the REL in which the NR CA, EN-DC, or NR DC configuration was introduced) |
| --- | --- | --- | --- |
| Intra-band and Inter-band NR CA configurations involving shared spectrum access | FDD and TDD, TDD | Rel-16 | Table B.3.1-1, Table B.4.2-1, Table B.4.8-1 |
| Inter-band EN-DC configurations involving shared spectrum access | FDD and TDD, TDD | Rel-16 | Table B.4.6-1 |
| Inter-band NR DC configurations involving shared spectrum access | FDD and TDD, TDD | Rel-16 | Table B.4.5-1, Table B.4.5a-1 |

### << Next changes >>

Table 6.2.1-1: NR intra-band contiguous CA within FR2

| Feature | DL/UL | CA BW Class | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- |
| Intra-band contiguous CA configurations within FR2 | DL | B, C, D, E, F, G, H, I, J, K, L, M, O, P, Q | TDD | Rel-15 | Table B.4.2-1 |
|  | UL | A, B, D, E, F, G, H, I, J, K, L, M, O, P, Q | TDD | Rel-15 |  |

### << Next changes >>

Table 6.2.1-2: NR non-contiguous intra-band CA within FR2

| Feature | DL/UL | number of sub-blocks | maximum number of CCs within a sub-block | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- |
| Intra-band non- contiguous CA configurations within | DL | 2 | 7 | TDD | Rel-15 | Table B.4.2-1 |
| FR2 |  | 3 | 4 | TDD | Rel-15 |  |
|  |  | 4 | 4 | TDD | Rel-15 |  |
|  |  | 5 | 4 | TDD | Rel-15 |  |
|  |  | 6 | 4 | TDD | Rel-15 |  |
|  |  | 7 | 3 | TDD | Rel-15 |  |
|  |  | 8 | 3 | TDD | Rel-15 |  |
|  |  | 9 | 2 | TDD | Rel-15 |  |
|  |  | 10 | 2 | TDD | Rel-15 |  |
|  |  | UL |  | 1 |  | 4 |  | TDD |  | Rel-15 |  |  |  |
|  |  |  |  | 2 |  | 1 |  | TDD |  | Rel-15 |  |  |  |
|  |  | 3 | 1 | TDD | Rel-15 |  |

### << Next changes >>

Table 6.2.1-3: NR inter-band CA within FR2

| Feature | DL/UL | Maximum number of bands | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- |
| Inter-band CA configurations within NR FR2 | DL | 2 | A, G, H, I, J, K, L, M | TDD | Rel-16 | Table B.4.2-1 |
|  | UL | 2 | A, G, H, I, J, K, L, M | TDD | Rel-16 |  |

### << Next changes >>

Table 7.1-1: NR inter-band CA between FR1 and FR2

| Feature | DL/UL | maximum number of bands | maximum number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band CA configurations for NR interworking between FR1 and FR2 | DL FR1 | 4 | 5 | A, B, C | FDD, TDD, FDD and TDD | Rel-15 | Table B.4.4-1 |
|  | DL FR2 | 1 | 8 | A, B, C, D, E, F, G, H, I, J, K, L, M, O, P, Q | TDD | Rel-15 |  |
|  | UL FR1 | 2 | 3 | A, B, C | FDD, TDD, FDD and TDD | Rel-15 |  |
|  | UL FR2 | 1 | 8 | A, B, C, D, E, F, G, H, I, J, K, L, M | TDD | Rel-15 |  |

### << Next changes >>

Table 8.1.2.1-1: EN-DC inter-band configurations without SUL within FR1

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band EN-DC | DL | 5 | 6 | 4 | 4 | FDD, TDD, FDD and TDD, FDD and SDL, FDD and SDL and TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 2 contiguous | 1 | 2 contiguous | FDD, TDD, FDD and TDD | Rel-15 |  |

### << Next changes >>

Table 8.1.2.2-1: EN-DC inter-band configurations including FR2

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band EN-DC | DL | 4 | 6 | 1 | 15 | TDD, FDD and TDD, FDD and SDL and TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 4 | 1 | 8 | TDD, FDD and TDD | Rel-15 |  |

### << Next changes >>

Table 8.1.2.3-1: EN-DC inter-band configurations including FR1 and FR2

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band EN-DC | DL FR1 | 4 | 6 | 2 | 2 | TDD, FDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
|  | DL FR2 |  |  | 1 | 8 | TDD | Rel-15 |  |
|  | UL FR1 | 1 | 2 | 1 | 1 | FDD, TDD, FDD and TDD | Rel-15 |  |
|  | UL FR2 |  |  | 1 | 8 | TDD | Rel-15 |  |

### << Next changes >>

Table 8.2.1.1-1: NE-DC interband configurations without SUL within FR1

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interband NE-DC | DL | 4 | 5 | 1 | 2 | FDD, TDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 1 | 1 | 1 | FDD, TDD, FDD and TDD | Rel-15 |  |

### << Next changes >>

#### 8.2.1.2	Interband NE-DC including frequency range 2

Requirements for a Rel-17 UE for additional NE-DC interband configurations including FR2 compared to TS 38.101-3 of Rel-17 [4] are introduced via this clause.

Table 8.2.1.2-1: NE-DC interband configurations including FR2

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interband NE-DC | DL | 4 | 5 | 1 | 8 | FDD and TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 2 | 1 | 8 | FDD and TDD | Rel-15 |  |

### << Next changes >>

Table B.4.2-1: Common UE RF requirements for a release independent CA configurations within NR frequency range 1 or NR frequency range 2

| Clause | Description |
| --- | --- |
| 5.2A | Operating bands for CA |
| 5.3A | UE channel bandwidth for CA |
| 5.4A | Channel arrangement for CA |
| 5.5A | Configurations for CA |
| 6.2A | Transmitter power for CA |
| 6.3A | Output power dynamics for CA |
| 6.4A | Transmit signal quality for CA |
| 6.5A | Output RF spectrum emissions for CA |
| 6.6A of [3] | Beam correspondence for CA |
| 7.3A | Reference sensitivity for CA |
| 7.4A | Maximum input level for CA |
| 7.5A | Adjacent Channel Selectivity for CA |
| 7.6A | Blocking characteristics for CA |
| 7.7A of [2] | Spurious response for CA |
| 7.8A of [2] | Intermodulation characteristics for CA |
| 7.9A of [2] | Spurious emissions for CA |

### << Next changes >>

Table B.4.4-1: Common UE RF requirements for a release independent interband CA configurations between NR frequency range 1 and NR frequency range 2

| Clause | Description |
| --- | --- |
| 5.2A | Operating bands for CA |
| 5.3A | UE channel bandwidth for CA |
| 5.4A | Channel arrangement for CA |
| 5.5A | Configuration for CA |
| 6.2A | Transmitter power for CA |
| 6.3A | Output power dynamics for CA |
| 6.4A | Transmit signal quality for CA |
| 6.5A | Output RF spectrum emissions for CA |
| 7.3A | Reference sensitivity for CA |
| 7.4A | Maximum input level for CA |
| 7.5A | Adjacent Channel Selectivity for CA |
| 7.6A | Blocking characteristics for CA |
| 7.7A | Spurious response for CA |
| 7.8A | Intermodulation characteristics for CA |
| 7.9A | Spurious emissions for CA |

### << Next changes >>

Table B.4.5-1: Common UE RF requirements for a release independent Inter-band NR-DC configurations between frequency range 1 and frequency range 2

| Clause | Description |
| --- | --- |
| 4.2 | Applicability of minimum requirements |
| 5.2B | Operating bands for DC |
| 5.5B | Configuration for DC |
| 6.2B.5 | Configured output power for NR-DC |

### << Next changes >>

Table B.4.6-1: Common UE RF requirements for a release independent NR interworking between NR and E-UTRA

| Clause | Description |
| --- | --- |
| 4.2 | Applicability of minimum requirements |
| 5.2B | Operating bands for DC |
| 5.3B | UE channel bandwidth for DC |
| 5.4B | Channel arrangement for DC |
| 5.5B | Configuration for DC |
| 6.2B | Transmitter power for DC |
| 6.3B | Output power dynamics for DC |
| 6.4B | Transmit signal quality for DC |
| 6.5B | Output RF spectrum emissions for DC |
| 6.6B | Beam correspondence for DC |
| 7.3B | Reference sensitivity level for DC |
| 7.4B | Maximum input level for DC in FR1 |
| 7.5B | Adjacent Channel Selectivity for DC in FR1 |
| 7.6B | Blocking characteristics for DC in FR1 |
| 7.7B | Spurious response for DC in FR1 |
| 7.8B | Intermodulation characteristics for DC in FR1 |
| 7.9A | Spurious emissions for CA in FR1 |

### << Next changes >>

Table B.4.8-1: Common UE RF requirements for shared spectrum access

| Clause | Description |
| --- | --- |
| 5.2A | Operating bands for CA |
| 5.3A | UE channel bandwidth for CA |
| 5.4A | Channel arrangement for CA |
| 5.5A | Configurations for CA |
| 6.2F | Transmitter power for shared spectrum channel access |
| 6.3F | Output power dynamics for shared spectrum channel access |
| 6.4F | Transmit signal quality for shared spectrum channel access |
| 6.5F | Output RF spectrum emissions |
| 7.3F | Reference sensitivity for shared spectrum channel access |
| 7.4 | Maximum input level |
| 7.5F | Adjacent channel selectivity |
| 7.6F | Blocking characteristics |
| 7.7F | Spurious response for shared spectrum channel access |
| 7.8F | Intermodulation characteristics for shared spectrum channel access |
| 7.9 | Spurious emissions |

### << End of changes >>