3GPP TSG-RAN WG4 Meeting # 110	R4-2402378

# Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0728 | rev | - | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Core) CR for Rel-17 TS 38.101-2 on correction of MPR requirement for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2024-01-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MPR expression for UL CA with contiguous allocations within the cumulative aggregated bandwidth (CABW) for (1) ≥ 400 MHz and < 800 MHz and (2) ≥ 800 MHz and ≤ 1400 MHz in Table 6.2A.2.2-1 should be aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing sign of “≤” to the MPR values for UL CA with contiguous allocations within the CABW for (1) ≥ 400 MHz and < 800 MHz and (2) ≥ 800 MHz and ≤ 1400 MHz in Table 6.2A.2.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-consistencies exist in the core specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes >

6.2A.2.2	Maximum output power reduction for power class 1

6.2A.2.2.1	Maximum output power reduction for power class 1 intra-band contiguous UL CA

For power class 1, MPR for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth is defined as:

MPRC_CA = max(MPRWT_C_CA, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB is less than or equal to 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT_C_CA is the maximum power reduction due to modulation orders, transmit bandwidth configurations, and waveform types. MPRWT_C_CA is defined in Tables 6.2A.2.2-1 and 6.2A.2.2-2.

Table 6.2A.2.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-1

| Waveform Type |  | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.51 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6.51 | ≤ 8.7 | ≤ 9.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| NOTE 1: (Void) |  |  |  |  |  |

Table 6.2A.2.2-2: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-2

| Waveform Type | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2000 MHz |
| Pi/2 BPSK | ≤ 7.0 | ≤ 5.0 | ≤ 2.0 | ≤ 2.0 |
| QPSK | ≤ 8.0 | ≤ 6.0 | ≤ 3.0 | ≤ 3.0 |
| 16 QAM | ≤ 8.0 | ≤ 6.0 | ≤ 4.0 | ≤ 4.0 |
| 64 QAM | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW  400 MHz, MPRWT_C_CA shall be derived instead as MAX(MPR1, MPR2), where:

MPR1 shall be determined from Table 6.2.2.1-1 if CABW  200 MHz, from Table 6.2.2.1-2 if CABW > 200 MHz.

MPR2 shall be determined from Table 6.2.2.1-1 if UL BWchannel_CA  200 MHz, from Table 6.2.2.1-2 if UL BWchannel_CA > 200 MHz.

and assume all UL CCs use the same SCS for the purpose of determination of inner and outer RB allocations in Table 6.2.2.1-1 and Table 6.2.2.1-2:

NRB shall be chosen as the sum of NRB of all constituent UL CCs in the CA configuration.

LCRB shall be chosen as BWalloc,RB

RBstart shall be derived as: RBstart_allocatedCC+NRB_unallocatedCC_low

RBstart_allocatedCC is the index of the first allocated RB in the CC with allocation

NRB_unallocatedCC_low is the sum of NRB in all UL CCs lower in frequency compared to the CC with allocation

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRC_CA.

For intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +  14.4)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs

<End of Changes >