3GPP TSG-RAN WG4 Meeting #110	R4-2402468

# Athens, Greece, February 26 – March 1, 2024

Source:	T-Mobile USA

Title:	TP for TR38.850: FDD PC2 for n25 with DL CA_n25A-n77A

Agenda item:	7.19.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 n25 for DL CA_n25A-n77A.

In other PC2 and PC1.5 MSD analysis, T-Mobile USA has been using calculations that increase the Interference by 3dB, but not the noise. This contribution proposes new MSD values based on this approach for single Tx.

- Reference

[1]

- Text Proposal

<Start of text proposal>

## 5.x	CA_ n25A-n77A

### 5.x.1	UE maximum output power

Table 5.5A.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n77A |  | n258 n778,9 CA_n25A-n77A8,14 | n25 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |

NOTE 8:	Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 10: 	Only single uplink carriers with power class other than PC3 are listed.

### 5.x.2	Reference sensitivity requirements

#### 5.x.2.0	General

For PC2, CA_n25-n77 has harmonic MSD for UL n25. This section will examine the existing PC3 MSD and propose MSD for PC2 FDD.

#### 5.x.2.1	Reference sensitivity requirements with PC2 on n71 without TxD

For CA_n25-n71, this is the configuration and MSD for UL n71 with PC3

Table 7.3A.4-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n25 |  | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n25 |  | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n25 |  | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |

For PC3 MSD we have N+IPC3. For PC2 MSD we have N+IPC2. So, for PC2 compared to PC3, I increases by 3 dB. For our other PC2 and PC1.5 MSD analysis we have been using the following approach:

MSD due to interference power I is given by:

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have:

If I is increased by X dB, then MSD(X) is given by

Using that approach, the following is proposed as a the PC2 MSD:

Table 7.3A.4-2a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1 for UE not supporting Tx Diversity

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 26.9 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 16.7 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 2.0 | NOTE 6 | UL2/DL1 near-miss |

#### 5.x.2.2	Reference sensitivity requirements with PC2 on n71 with TxD

[TBD]