3GPP TSG-RAN WG4 Meeting#110                             R4-2402522

Athens, Greece, Feb 26 – Mar 1, 2024

Agenda item:	8.18.5.2

Source: 	ZTE Corporation

Title: 	Further discussion on UE RF Rx requirements for NTN in Ka-band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. In the previous RAN4 meetings, there were lots of discussions on VSAT RF requirements, however there are still lots of open issues left for further discussions. In the contribution, we provided some further detailed analysis for the remaining issues.

Discussion

- 2.1. The minimum EIS

Basic principle:

The minimum EIS of NTN VSAT will support the NTN VSAT with capability of decoding low modulation order at the lowest supported elevation angle (e.g. 30 degree for GSO and FFS for LEO). Low MCS in the following calculation is based on QPSK with coding rate as 1/3, then corresponding SNR for 95% throughput should be around -1dB;

EISmin=SANTxPower−PathlossFreeSpace−Atmosphereloss

Where:

- SAN_TxPower=X+30+10*log10(BW)-[3/0]dB;  X is assumed as 40 dBW/MHz for GEO and 10 dBW/MHz for LEO1200KM and 4 dBW/MHz for LEO600MHz;  3dB margin is considered for VSAT is located at the beamprint edge which is corresponding to 3dB beamwidth of radiation beam. If VSAT is located at the center of beamprint, 0dB margin could be considered

- Pathloss is the same as previous calculation for minimum peak EIRP and minimum output power; Note: if consider the pathloss captured in TR 38.811 clause 6.6, then LOS probability and shadow fading varying with elevation angle should be considered as well especially for the low elevation angle.

- Atmosphere loss is also the same as before.

Similar as section 2.1, in order to better visualize the required receiver antenna size of NTN UE, we made the following estimation for it.

Antennasizemini=10−174dBm/Hz+10log10BW∗106+NF+SNR+IM−Gainelement−EIS_min10

Where

- EIRP_mini is derived as above;

- G_element is assumed as 5dB;

- NF is assumed as 2.5db for NTN UE except for type 3.  For NTN UE type 3, it is assumed as 6dB;

- BW is assumed as 100MHz;

- SNR is assumed as -1dB for the same MCS as specified in the 38.101-2.

- Gain_element is assumed as 5dB with 90 degree as 3dB beamwidth;

- 2.2. The maximum input power

Basic principle:

The maximum input power of NTN VSAT will support the NTN VSAT with capability of decoding highest modulation order at the supported elevation angle 90 closest to satellite, then corresponding SNR of DL 64QAM with 948/1024 coding rate for 95% throughput should be around 20dB;

EISmin=SANTxPower−PathlossFreeSpace−Atmosphereloss

Where:

- SAN_TxPower=X+30+10*log10(BW)-0dB;  X is assumed as 40 dBW/MHz for GEO and 10 dBW/MHz for LEO1200KM and 4 dBW/MHz for LEO600MHz;  3dB margin is considered for VSAT is located at the beamprint edge which is corresponding to 3dB beamwidth of radiation beam. If VSAT is located at the center of beamprint, 0dB margin could be considered

- Pathloss is the same as previous calculation for minimum peak EIRP and minimum output power; Note: if consider the pathloss captured in TR 38.811 clause 6.6, then LOS probability and shadow fading varying with elevation angle should be considered as well especially for the low elevation angle.

- Atmosphere loss is also the same as before.

Similar as section 2.2, in order to better visualize the required receiver antenna size of NTN UE, we made the following estimation for it.

Antennasizemini=10−174dBm/Hz+10log10BW∗106+NF+SNR+IM−Gainelement−EIS_min10

Where

- EIRP_mini is derived as above;

- G_element is assumed as 5dB;

- NF is assumed as 2.5db for NTN UE except for type 3.  For NTN UE type 3, it is assumed as 6dB;

- BW is assumed as 100MHz;

- SNR is assumed as 20dB.

- Gain_element is assumed as 5dB with 90 degree as 3dB beamwidth;

## 2.3. ACS requirement for NTN VSAT

| ## Scenario [Fixed VSAT] | ## ACIR requirement [Case 5 for Fixed VSAT ACS] |  |
| --- | --- | --- |
|  | ## Elevation angle =90 | ## Elevation angle =25 |
| ## GEO | ## 32 | ## 36 |
| ## LEO-1200KM [2.5dB NF] | ## 34 | ## >40 |
| ## LEO-1200KM [6dB NF] | ## 30 | ## 40 |
| ## LEO-600KM [2.5dB NF] | ## 36 | ## >40 |
| ## LEO-600KM [6dB NF] | ## 32 | ## 38 |

| ## Scenario [Fixed VSAT] | ## ACIR requirement [Case 8 for Fixed VSAT ACS] |  |
| --- | --- | --- |
|  | ## Elevation angle =90 | ## Elevation angle =25 |
| ## GEO | ## 0 | ## 0 |
| ## LEO-1200KM [2.5dB NF] | ## 0 | ## 0 |
| ## LEO-1200KM [6dB NF] | ## >0dB | ## 0 |
| ## LEO-600KM [2.5dB NF] | ## 0 | ## 0 |
| ## LEO-600KM [6dB NF] | ## 4 | ## 0 |

Observation 5: for Fixed VSAT, more than 40dB ACIR requirement from receiver side is needed for some scenarios.

| ## Scenario [L- ESIM] | ## ACIR requirement [Case 5 for Fixed VSAT ACS] |  |
| --- | --- | --- |
|  | ## Elevation angle =90 | ## Elevation angle =25 |
| ## GEO | ## 28 | ## 32 |

| ## Scenario [Fixed VSAT] | ## ACIR requirement [Case 8 for Fixed VSAT ACS] |  |
| --- | --- | --- |
|  | ## Elevation angle =90 | ## Elevation angle =25 |
| ## GEO | ## 0 | ## 0 |

Observation 1: for L-ESIM, the 32dB ACIR requirement from transmitter perspective is enough.

Conclusions

In this contribution, we want to share some further views on VSAT RF requirements for NTN in Ka-band.and proposals are made as following:

Proposal 1: to consider the basic principle as described in section 2.1 for minimum EIS requirements for VSAT.

Proposal 2: to consider the basic principle as described in section 2.2 for maximum input power for VSAT.

Observation 1: for L-ESIM, the 32dB ACIR requirement from transmitter perspective is enough.

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828

- https://www.kingsat-tech.com/product/enhanced-vsat-ku-band-3-axis-stabilized-and-4-axis-tracking114.html

- https://www.mitsubishielectric.com/news/2020/0206.html

- R4-2217792, Email discussion summary for [104-bis-e][140] NR_NTN_enh_UERF, ZTE, Noted

- R4-2217743, WF on NTN UE in Ka-band, ZTE, Approved

- http://satstar.net/satellites/space2.html

- https://www.satbeams.com/satellites

- http://satstar.net/beams/inmar5f3_sb11.html

- R4-2216515,NTN enhancements – Regulatory aspects, Ericsson, Noted

- R4-2220573	WF on NTN UE RF requirements, ZTE, approved.

- R4-2220119, Topic summary for [105][139] NR_NTN_enh_UERF, ZTE

- https://www.etsi.org/deliver/etsi_en/301400_301499/301459/02.01.01_60/en_301459v020101p.pdf

- RF Spectrum FCC 47CFR25.138, 47CFR25.202,

- https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-25/subpart-C/section-25.202

- https://www.digisat.org/enterprise-75cm-professional-ka-band-terminal

- https://www.satsig.net/eut2beac.htm

- R4-2303533,WF on NTN UE RF requirements for Ka band, ZTE, Approved.

- R4-2306624, WF on NTN UE RF requirements in Ka-band. ZTE, approved.

- https://www.eutelsat.pl/files/contributed/satellites/pdf/esog120.pdf