3GPP TSG-RAN WG4 Meeting #110	R4-2402534

# Athens, 26th Feb - 1st Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-1 | CR | 0029 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BigCR for introduction of performance requirements for NCR-MT in TS 38.115-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-03-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce conducted performance requirements for NCR-MT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For introducing conducted performance requirements for NCR-MT, update clause 2, F, G, add new clause 7, D.9, D.10, D.11, D.12, change original clause F to H. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There may be inconsist between specification and RAN4 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7 (New clause), D.9 (New clause), D.10 (New clause), D.11 (New clause), D.12 (New clause), F, G, F (original) -> H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | New clause: 7, D.9, D.10, D.11, D.12 Change original clause F to H. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF THE CHANGE 1>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.106: "NR; Repeater Radio Transmission and Reception"

[3]	3GPP TS 38.115-2: "NR; Repeater conformance testing, Part 2: Radiated conformance testing"

[4]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain"

[5]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

[6]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception"

[7]	3GPP TS 38.141-1: "NR; Base Station (BS) conformance testing, Part 1: Conducted conformance testing"

[8]	3GPP TS 38.211: "NR; Physical channels and modulation"

[9]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

[11]	ITU-R Recommendation M.1545: “Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications – 2000”

[12]	ITU-T Recommendation O.150, "Equipment for the measurement of digital and analogue/digital parameters"

[13]	Federal Communications Commission: "Title 47 of the Code of Federal Regulations (CFR) "

[14]	ECC/DEC/(17)06: "The harmonised use of the frequency bands 1427-1452 MHz and 1492-1518 MHz for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL)"

[15]	3GPP TR 25.942: "RF system scenarios"

[16]	IEC 60 721-3-3: "Classification of environmental conditions - Part 3-3: Classification of groups of environmental parameters and their severities - Stationary use at weather protected locations"

[17]	IEC 60 721-3-4: "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Clause 4: Stationary use at non-weather protected locations"

[18]	IEC 60 721: "Classification of environmental conditions"

[19]	IEC 60 068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold"

[20]	IEC 60 068-2-2: (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat"

[21]	IEC 60 068-2-6: (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)"

[22]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[23]	3GPP TS 38.214: "NR; Physical layer procedures for data"

[24]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz"

<END OF THE CHANGE 1>

<START OF THE CHANGE 2>

# 7 	Conducted performance requirements

## 7.1	NCR-MT Performance requirements

### 7.1.1	General

#### 7.1.1.1	Scope and definitions

Conducted performance requirements specify the ability of the NCR-MT to correctly demodulate signals in various conditions and configurations. Conducted performance requirements are specified at the antenna connectors (NCR-MT type 1-C) or TAB connector(s) (NCR type 1-H).

Conducted performance requirements for the NCR-MT are specified for the fixed reference channels and the propagation conditions defined in TS 38.106 [2] annex F and annex G, respectively.

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in a slot on a single antenna connector or TAB connector.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a slot.

### 7.1.2	Demodulation performance requirements

#### 7.1.2.1	General

#### 7.1.2.2	Performance requirements for PDSCH

##### 7.1.2.2.1	Definition and applicability

The performance requirement of PDSCH is determined by two means; a minimum required throughput for a given SNR and a minimum SNR at which 1% BLER for the first transmission is achieved. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex F. The throughput performance requirements assume HARQ re-transmissions, whereas the first transmission BLER requirements do not consider HARQ.

##### 7.1.2.2.2	Minimum requirement

The minimum requirement is in TS 38.106 [2] clause 8.3.1.

##### 7.1.2.2.3	Test purpose

The test shall verify the receiver’s ability to achieve throughput  and 1st transmission BLER under multipath fading propagation conditions for a given SNR.

##### 7.1.2.2.4	Method of test

7.1.2.2.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

7.1.2.2.4.2	Procedure

1)	Connect the NCR tester generating the wanted signal, multipath fading simulators and AWGN generators to all NCR-MT TAB connectors or antenna connectors  for diversity reception via a combining network as shown in annex D.9 or D.11.

2)	Adjust the AWGN generator and adjust the AWGN power level to -77.2 dBm / 38.16MHz.

3)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex F and the test parameters in table 7.1.2.2.4.2-1.

Table 7.1.2.2.4.2-1: Test parameters for testing PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active BWP index |  |  | 1 |
| Default TDD UL-DL pattern (Note 1) |  |  | 7D1S2U, S=6D:4G:4U |
| PDSCH transmission scheme |  |  | Transmission scheme 1 |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) | RBs | 0 |
|  | Subcarrier spacing | kHz | 30 |
| DL BWP configuration #1 | Cyclic prefix |  | Normal |
|  | RB offset | RBs | 0 |
|  | Number of contiguous PRB | PRBs | 106 |
| PDSCH DMRS configuration | Antenna ports indexes |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests {1000-1002} for Rank 3 tests {1000-1003} for Rank 4 tests |
|  | Position of the first DMRS for PDSCH mapping type A |  | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for Rank 1 and Rank 2 tests 2 for Rank 3 and Rank 4 tests |
|  | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PT-RS configuration |  |  | PT-RS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  | 1 |
| Maximum number of HARQ transmission |  |  | 4 |
| HARQ ACK/NACK bundling |  |  | Multiplexed |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns. Note 2: Point A coincides with minimum guard band as specified in TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing. |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex F.

5)	Adjust the equipment so that required SNR specified in table 81.2.2.5.1-1 is achieved at the IAB-MT input.

6)	For each of the reference channels in table 7.1.2.2.5.1-1 measure the throughput and the 1st transmission BLER.

##### 7.1.2.2.5	Test requirement

The throughput measured according to clause 7.1.2.2.4.2 shall not be below the limits for the SNR levels specified in table 7.1.2.2.5-1.

Table 7.1.2.2.5-1: Minimum performance PDSCH

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test metric | SNR (dB) |
| 1 |  | 10 / 15 | 4 | TDLA30-10 | 2x2 | 70% TPUT | -0.1 |
| 2 |  | 10 / 15 | 4 | TDLA30-10 | 2x2 | 1% BLER | 5.7 |
| 3 |  | 10 / 15 | 4 | TDLA30-10 | 2x4 | 70% TPUT | -3.1 |
| 4 |  | 10 / 15 | 4 | TDLA30-10 | 2x4 | 1% BLER | 0.9 |
| 5 |  | 10 / 15 | 13 | TDLA30-10 | 2x2 | 70% TPUT | 7.6 |
| 6 |  | 10 / 15 | 13 | TDLA30-10 | 2x2 | 1% BLER | 13.7 |
| 7 |  | 10 / 15 | 13 | TDLA30-10 | 2x4 | 70% TPUT | 4.3 |
| 8 |  | 10 / 15 | 13 | TDLA30-10 | 2x4 | 1% BLER | 8.5 |
| 9 |  | 40 / 30 | 4 | TDLA30-10 | 2x2 | 70% TPUT | -0.2 |
| 10 |  | 40 / 30 | 4 | TDLA30-10 | 2x2 | 1% BLER | 5.1 |
| 11 |  | 40 / 30 | 4 | TDLA30-10 | 2x4 | 70% TPUT | -3.1 |
| 12 |  | 40 / 30 | 4 | TDLA30-10 | 2x4 | 1% BLER | 0.3 |
| 13 |  | 40 / 30 | 13 | TDLA30-10 | 2x2 | 70% TPUT | 7.6 |
| 14 |  | 40 / 30 | 13 | TDLA30-10 | 2x2 | 1% BLER | 12.7 |
| 15 |  | 40 / 30 | 13 | TDLA30-10 | 2x4 | 70% TPUT | 4.4 |
| 16 |  | 40 / 30 | 13 | TDLA30-10 | 2x4 | 1% BLER | 7.8 |

#### 7.1.2.3	Performance requirements for PDCCH

##### 7.1.2.3.1	Definition and applicability

The performance requirement of PDCCH is determined by a maximum allowed missed detection rate for a given SNR. The required missed detection rate is expressed for the FRCs listed in annex F.

##### 7.1.2.3.2	Minimum requirement

The minimum requirement is in TS 38.106 [2] clause 8.3.2.

##### 7.1.2.3.3	Test purpose

The test shall verify the receiver's ability to achieve missed detection rate under multipath fading propagation conditions for a given SNR.

##### 7.1.2.3.4	Method of test

7.1.2.3.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

7.1.2.3.4.2	Procedure

1)	Connect the NCR tester generating the wanted signal, multipath fading simulators and AWGN generators to all NCR-MT TAB connectors or antenna connectors for diversity reception via a combining network as shown in annex D.9 or D.11.

2)	Adjust the AWGN generator and adjust the AWGN power level to -77.2 dBm / 38.16MHz.

3)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex A and the test parameters in table 7.1.2.3.4.2-1.

Table 7.1.2.3.4.2-1: Test parameters for testing PDCCH

| Parameter | Unit | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- | --- |
| CCE to REG mapping type |  | interleaved |  | interleaved |  |
| Interleaver size |  | 3 |  |  |  |
| REG bundle size |  | 2 | 6 |  |  |
| Shift Index |  | 0 |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex F.

5)	Adjust the equipment so that required SNR specified in table 7.1.2.3.5.1-1is achieved at the NCR-MT input.

6)	For each of the reference channels in table 7.1.2.3.5.1-1, applicable for the NCR-MT, measure the missed detection.

##### 7.1.2.3.5	Test requirement

For the parameters specified in Table 7.1.2.3.4.2.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 7.1.2.3.5-1 for SCS supported by the NCR-MT.

Table 871.2.3.5-1: Minimum performance for PDCCH

| Test number | Bandwidth (MHz) |  | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 15 | 24 | 2 | 2 |  | TDLA30-10 | 1x2 | 1 | 8.7 |
| 2 | 10 | 15 | 24 | 2 | 2 |  | TDLA30-10 | 1x4 | 1 | 2.8 |
| 3 | 10 | 15 | 48 | 2 | 4 |  | TDLA30-10 | 1x2 | 1 | 6.1 |
| 4 | 10 | 15 | 48 | 2 | 4 |  | TDLA30-10 | 1x4 | 1 | 0.8 |
| 5 | 10 | 15 | 48 | 1 | 8 |  | TDLA30-10 | 2x2 | 1 | 0.6 |
| 6 | 10 | 15 | 48 | 1 | 8 |  | TDLA30-10 | 2x4 | 1 | -1.7 |
| 7 | 40 | 30 | 102 | 1 | 2 |  | TDLA30-10 | 1x2 | 1 | 7.6 |
| 8 | 40 | 30 | 102 | 1 | 2 |  | TDLA30-10 | 1x4 | 1 | 2.7 |
| 9 | 40 | 30 | 102 | 1 | 4 |  | TDLA30-10 | 1x2 | 1 | 5.5 |
| 10 | 40 | 30 | 102 | 1 | 4 |  | TDLA30-10 | 1x4 | 1 | 1.3 |
| 11 | 40 | 30 | 90 | 1 | 8 |  | TDLA30-10 | 2x2 | 1 | 0.1 |
| 12 | 40 | 30 | 90 | 1 | 8 |  | TDLA30-10 | 2x4 | 1 | -3.3 |

7.1.3	CSI reporting requirements

7.1.3.1	General

7.1.3.1.1	Applicability rule for NCR-MT

7.1.3.1.1.1	General

Unless otherwise stated, for an NCR-MT declared to support more than 4 antenna connectors (for NCR type 1-C), the performance requirement tests for 4 RX antennas shall apply, and the specific connectors used for testing is up to NCR-MT implementation.

7.1.3.1.1.2	Applicability of requirements for different subcarrier spacings

Unless otherwise stated, the tests shall apply only for each subcarrier spacing declared to be supported (see D.14 in table 4.6-1).

7.1.3.1.1.3	Applicability of requirements for TDD with different UL-DL patterns

Unless otherwise stated, for each subcarrier spacing declared to be supported, if NCR-MT supports multiple TDD UL-DL patterns, only one of the supported TDD UL-DL patterns shall be used for all tests.

7.1.3.1.1.4	Applicability of requirements for NCR-MT features

Unless otherwise stated, for NCR type 1-C, the CSI reporting tests shall apply only in case the number of NZP-CSI-RS ports in the test case satisfies maximum number of ports across all configured NZP-CSI-RS resources per CC declared to be supported (see D.201 in table 4.6-1, maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC).

Note:	Applicability information may be obtained based on vendor declaration (Section 4.6) or alternatively from reading capability signaling.

7.1.3.1.2	Common test parameters

Parameters specified in Table 7.1.3.1.2-1 are applied for all test cases in this clause unless otherwise stated.

Table 7.1.3.1.2-1: Test parameters for CSI test cases

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH transmission scheme |  |  | Transmission scheme 1 |
| Duplex mode |  |  | TDD |
| PTRS epre-Ratio |  |  | N/A |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 3) | RBs | 0 |
|  | Subcarrier spacing | kHz | 30 |
| DL BWP configuration #1 | Cyclic prefix |  | Normal |
|  | RB offset | RBs | 0 |
|  | Number of contiguous PRB | PRBs | 106 |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
|  | DMRS ports indexes |  | {1000} for Rank1 {1000,1001} for Rank2 {1000,1001,1002} for Rank3 {1000,1001,1002,1003} for Rank4 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 2 |
| PTRS configuration | Frequency density (KPT-RS) |  | N/A |
|  | Time density (LPT-RS) |  | N/A |
|  | Resource Element Offset |  | N/A |
| NZP CSI-RS for CSI acquisition | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| Note 1: PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL. NOTE 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [22] for tested channel bandwidth and subcarrier spacing. |  |  |  |

7.1.3.2	Reporting Channel Quality Indicator (CQI)

7.1.3.2.1	Definition and applicability

The reporting accuracy of the channel quality indicator (CQI) under frequency non-selective conditions is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI median. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [23]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

Which specific test(s) are applicable to NCR-MT is based on the test applicability rules defined in clause [7.1.3.1.1].

7.1.3.2.2	Minimum requirement

The minimum requirement is in TS 38.106 [2] clause [7.1.3.1].

7.1.3.2.3	Test purpose

The test shall verify the receiver's ability to report CQI values accordance with the CQI definition given in TS 38.214 [23].

7.1.3.2.4	Method of test

7.1.3.2.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

7.1.3.2.4.2	Test procedure

1)	Connect the NCR-MT tester generating the wanted signal and AWGN generators to all NCR-MT TAB connectors for diversity reception via a combining network as shown in annex D.5 and D.6.

2)	Adjust the AWGN generator, according to the channel bandwidth, defined in table 7.1.3.2.4.2-1.

Table 7.1.3.2.4.2-1: AWGN power level at the NCR-MT input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 kHz | 10 | -83.3 dBm / 38.16MHz |
| 30 kHz | 40 | -77.2 dBm / 38.16MHz |

3)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex A and the test parameters in table 7.1.3.2.4.2-2 to table 7.1.3.2.4.2-5.

Table 7.1.3.2.4.2-2: Test parameters for testing CQI reporting

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  | kHz | 15 |  |  |  |
| Default TDD UL-DL pattern (Note 1) |  |  | 7D1S2U, S=6D:4G:4U |  |  |  |
| SNR |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  | AWGN |  |  |  |
| Antenna configuration |  |  | 2x2 |  |  |  |
| Beamforming Model |  |  | As specified in Annex J.3 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  | 2 |  |  |  |
|  | CDM Type |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 |  |  |  |
| ReportConfigType |  |  | Periodic |  |  |  |
| CQI-table |  |  | Table 2 |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |
| cqi-FormatIndicator |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  | Wideband |  |  |  |
| Sub-band Size |  | RB | 8 |  |  |  |
| Csi-ReportingBand |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  | slot | 5/0 |  |  |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  | Codebook Mode |  | 1 |  |  |  |
|  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  | RI Restriction |  | N/A |  |  |  |
| CQI/RI/PMI delay |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  | 1 |  |  |  |
| Measurement channel |  |  | M-FR1-B.1.4-1 |  |  |  |
| Note 1: The same requirements are applicable for TDD with different UL-DL pattern. |  |  |  |  |  |  |

Table 7.1.3.2.4.2-3: Test parameters for testing CQI reporting

| Parameter |  | Unit |  | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz |  | 10 |  |  |  |
| Subcarrier spacing |  | kHz |  | 15 |  |  |  |
| Default TDD UL-DL pattern (Note 1) |  |  |  | 7D1S2U, S=6D:4G:4U |  |  |  |
| SNR |  | dB |  | 6 | 7 | 12 | 13 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2x4 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex J.3 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |  |  |
| Codebook configuration | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  | Codebook Mode |  |  | 1 |  |  |  |
|  | CodebookSubsetRestriction |  |  | 010000 |  |  |  |
|  | RI Restriction |  |  | N/A |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | M-FR1-B.1.4-1 |  |  |  |
| Note 1: The same requirements are applicable for TDD with different UL-DL pattern. |  |  |  |  |  |  |  |

Table 7.1.3.2.4.2-4: Test parameters for testing CQI reporting

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  | kHz | 30 |  |  |  |
| Default TDD UL-DL pattern (Note 1) |  |  | 7D1S2U, S=6D:4G:4U |  |  |  |
| SNR |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  | AWGN |  |  |  |
| Antenna configuration |  |  | 2x2 |  |  |  |
| Beamforming Model |  |  | As specified in Annex J.3 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  | 2 |  |  |  |
|  | CDM Type |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 |  |  |  |
| ReportConfigType |  |  | Periodic |  |  |  |
| CQI-table |  |  | Table 2 |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |
| cqi-FormatIndicator |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  | Wideband |  |  |  |
| Sub-band Size |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  | slot | 10/9 |  |  |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  | Codebook Mode |  | 1 |  |  |  |
|  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  | RI Restriction |  | N/A |  |  |  |
| CQI/RI/PMI delay |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  | 1 |  |  |  |
| Measurement channel |  |  | M-FR1-B.1.4-2 |  |  |  |
| Note 1: The same requirements are applicable for TDD with different UL-DL pattern. |  |  |  |  |  |  |

Table 7.1.3.2.4.2-5: Test parameters for testing CQI reporting

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  | kHz | 30 |  |  |  |
| Default TDD UL-DL pattern (Note 1) |  |  | 7D1S2U, S=6D:4G:4U |  |  |  |
| SNR |  | dB | 5 | 6 | 11 | 12 |
| Propagation channel |  |  | AWGN |  |  |  |
| Antenna configuration |  |  | 2x4 |  |  |  |
| Beamforming Model |  |  | As specified in Annex J.3 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  | 2 |  |  |  |
|  | CDM Type |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 |  |  |  |
| ReportConfigType |  |  | Periodic |  |  |  |
| CQI-table |  |  | Table 2 |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |
| cqi-FormatIndicator |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  | Wideband |  |  |  |
| Sub-band Size |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  | slot | 10/9 |  |  |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  | Codebook Mode |  | 1 |  |  |  |
|  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  | RI Restriction |  | N/A |  |  |  |
| CQI/RI/PMI delay |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  | 1 |  |  |  |
| Measurement channel |  |  | M-FR1-B.1.4-2 |  |  |  |
| Note 1: The same requirements are applicable for TDD with different UL-DL pattern. |  |  |  |  |  |  |

4)	Adjust the equipment so that required SNR specified in table 7.1.3.2.4.2-2 to 7.1.3.2.4.2-5 is achieved at the NCR-MT input.

5)	For each test specified in table 7.1.3.2.4.2-2 to 7.1.3.2.4.2-5 applicable for the NCR-MT, measure the median CQI and the BLER at median CQI and (median CQI+1 or median CQI-1) as per clause 7.1.3.2.5.

7.1.3.2.5	Test requirement

For the parameters specified in Table 7.1.3.2.4.2-2 to 7.1.3.2.4.2-5, and using the downlink physical channels specified in Annex A, the test requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

<END OF THE CHANGE 2>

<START OF THE CHANGE 3>

## D.9	NCR-MT type 1-C PDCCH and PDSCH demodulation testing

NCR-MT under test

Channel

Simulator

Channel

Simulator

Channel

Simulator

Channel

Simulator

NCR-MT tester

Tx 1

Tx 2

feedback

AWGN GeneratorAWGN Generator

AWGN GeneratorAWGN Generator

Synchronization source (if used see NOTE 2)

RX A

RX B

Figure D.9-1: Functional set-up for performance requirements for PDSCH and PDCCH transmission on two antenna ports in multipath fading conditions (2 Rx case shown)

NOTE 1:	The feedback could be done as an RF feedback, either using NR channels or using other means, or as a digital feedback. The HARQ Feedback should be error free.

NOTE 2:	In tests performed with signal generators, a synchronization signal may be provided between the NCR-MT node and the signal generator, or a common (e.g., GNSS) source may be provided to both NCR-MT node and the signal generator, to enable correct timing of the wanted signal. The method of synchronization with the TE is left to test implementation.

NOTE 3:	It is left up to implementation how L1/L2 is configured for testing.

<END OF THE CHANGE 3>

<START OF THE CHANGE 4>

## D.10 	NCR-MT type 1-C CQI testing

NCR-MT under test

AWGN Generator

AWGN GeneratorAWGN Generator

NCR tester

Synchronization source (if used see NOTE 1)

RX A

RX B

Figure D.10-1: Functional set-up for performance requirements for CQI in static conditions for NCR-MT with Rx diversity (2 Rx case shown)

NOTE 1:	In tests performed with signal generators, a synchronization signal may be provided between the NCR node and the signal generator, or a common (e.g., GNSS) source may be provided to both NCR node and the signal generator, to enable correct timing of the wanted signal. The method of synchronization with the TE is left to test implementation.

NOTE 2:	It is left up to implementation how L1/L2 is configured for testing.

<END OF THE CHANGE 4>

<START OF THE CHANGE 5>

## D.11	NCR-MT type 1-H PDCCH and PDSCH demodulation testing

transceiver unit array

#1

#2

#K

transceiver array boundary

Transceiver array boundary connector TAB(n)

Load

Channel

Simulator

Channel

Simulator

Channel

Simulator

Channel

Simulator

NCR-MT tester

Tx 1

Tx 2

feedback

AWGN GeneratorAWGN Generator

AWGN GeneratorAWGN Generator

Synchronization source (if used see NOTE 2)

Figure D.11-1: Functional set-up for performance requirements for PDSCH and PDCCH transmission on two antenna ports in multipath fading conditions (2 Rx case shown)

NOTE 1:	The feedback could be done as an RF feedback, either using NR channels or using other means, or as a digital feedback. The HARQ Feedback should be error free.

NOTE 2:	In tests performed with signal generators, a synchronization signal may be provided between the NCR-MT node and the signal generator, or a common (e.g., GNSS) source may be provided to both NCR-MT node and the signal generator, to enable correct timing of the wanted signal. The method of synchronization with the TE is left to test implementation.

NOTE 3:	It is left up to implementation how L1/L2 is configured for testing.

<END OF THE CHANGE 5>

<START OF THE CHANGE 6>

## D.12 	NCR-MT type 1-H CQI testing

transceiver unit array

#1

#2

#K

transceiver array boundary

Transceiver array boundary connector TAB(n)

Load

AWGN Generator

AWGN GeneratorAWGN Generator

NCR tester

Synchronization source (if used see NOTE 1)

Figure D.12-1: Functional set-up for performance requirements for CQI in static conditions for NCR-MT with Rx diversity (2 Rx case shown)

NOTE 1:	In tests performed with signal generators, a synchronization signal may be provided between the NCR node and the signal generator, or a common (e.g., GNSS) source may be provided to both NCR node and the signal generator, to enable correct timing of the wanted signal. The method of synchronization with the TE is left to test implementation.

NOTE 2:	It is left up to implementation how L1/L2 is configured for testing.

<END OF THE CHANGE 6>

<START OF THE CHANGE 7>

Annex F (normative):
NCR-MT Reference measurement channels

## F.7	NCR-MT Fixed Reference Channels

### F.7.1	Fixed Reference Channels for PDSCH performance requirements

#### F.7.1.1	FDD

The parameters for the reference measurement channels are specified in table F.7.1.1.1-1 and table F.7.1.1.2-1 for FR1 FDD 15kHz PDSCH performance requirements.

##### F.7.1.1.1	Fixed Reference Channels for PDSCH performance requirements (QPSK)

Table F.7.1.1.1-1: FRC parameters for FR1 PDSCH performance requirements, 1 transmission layers, QPSK

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | M-FR1-F.7.1.1.1-1 |
| Channel bandwidth | MHz | 10 |
| Subcarrier spacing | kHz | 15 |
| Allocated resource blocks | PRBs | 52 |
| Number of consecutive PDSCH symbols |  | 12 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.3 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  | 4096 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 1 |
| Binary Channel Bits Per Slot |  | 13728 |

##### F.7.1.1.2	Fixed Reference Channels for PDSCH performance requirements (16QAM)

Table F.7.1.1.2-1: FRC parameters for FR1 PDSCH performance requirements, 1 transmission layers, 16QAM

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | M-FR1-F.7.1.1.2-1 |
| Channel bandwidth | MHz | 10 |
| Subcarrier spacing | kHz | 15 |
| Allocated resource blocks | PRBs | 52 |
| Number of consecutive PDSCH symbols |  | 12 |
| MCS table |  | 64QAM |
| MCS index |  | 13 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.48 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  | 13064 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 2 |
| Binary Channel Bits Per Slot |  | 27456 |

#### F.7.1.2	TDD

The parameters for the reference measurement channels are specified in table F.7.1.2.1-1and table F.7.1.2.2-1 for FR1 TDD 30kHz PDSCH performance requirements.

##### F.7.1.2.1	Fixed Reference Channels for PDSCH performance requirements (QPSK)

Table F.7.1.2.1-1: FRC parameters for FR1 PDSCH performance requirements, 1 transmission layers, QPSK

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | M-FR1-F.7.1.2.1-1 |
| Channel bandwidth | MHz | 40 |
| Subcarrier spacing | kHz | 30 |
| Allocated resource blocks | PRBs | 106 |
| Number of consecutive PDSCH symbols |  | 12 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.3 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  | 8456 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 2 |
| Binary Channel Bits Per Slot |  | 27984 |

##### F.7.1.2.2	Fixed Reference Channels for PDSCH performance requirements (16QAM)

Table F.7.1.2.2-1: FRC parameters for FR1 PDSCH performance requirements, 1 transmission layers, 16QAM

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | M-FR1-F.7.1.2.2-1 |
| Channel bandwidth | MHz | 40 |
| Subcarrier spacing | kHz | 30 |
| Allocated resource blocks | PRBs | 106 |
| Number of consecutive PDSCH symbols |  | 12 |
| MCS table |  | 64QAM |
| MCS index |  | 13 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.48 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  | 26632 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 4 |
| Binary Channel Bits Per Slot |  | 55968 |

### F.7.2	Fixed Reference Channels for PDCCH performance requirements

#### F.7.2.1	FDD

The parameters for the reference measurement channels are specified in table F.7.2.1-1 for FR1 FDD 15kHz PDCCH performance requirements.

Table F.7.2.1-1 . FR1 PDCCH Reference Channels

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | M-FR1-F.7.2.1-1 | M-FR1-F.7.2.1-2 | M-FR1-F.7.2.1-3 |
| Subcarrier spacing | kHz | 15 | 15 | 15 |
| CORESET frequency domain allocation |  | 24 | 48 | 48 |
| CORESET time domain allocation |  | 2 | 2 | 1 |
| Aggregation level |  | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 39 | 52 | 52 |

#### F.7.2.2	TDD

The parameters for the reference measurement channels are specified in table F.7.2.2-1 for FR1 TDD 30kHz PDCCH performance requirements.

Table F.7.2.2-1 . FR1 PDCCH Reference Channels

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | M-FR1-F.7.2.2-1 | M-FR1-F.7.2.2-2 | M-FR1-F.7.2.2-3 |
| Subcarrier spacing | kHz | 30 | 30 | 30 |
| CORESET frequency domain allocation |  | 102 | 102 | 102 |
| CORESET time domain allocation |  | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 41 | 53 | 53 |

### F.7.3	Fixed Reference Channels for CSI reporting performance

This clause defines the DL signal applicable to the reporting of channel status information.

Tables in this clause specifies the mapping of CQI index to Information Bit payload, which complies with the CQI definition specified in clause 5.2.2.1 of TS 38.214 [23] and with MCS definition specified in clause 5.1.3 of TS 38.214 [23].

Table F.7.3-1.  Fixed Reference Channels for FR1 CSI reporting with CQI table 2

| Reference channel |  |  |  | M-FR1-A.7.3-1 | M-FR1-A.7.3-2 |
| --- | --- | --- | --- | --- | --- |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 106 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 1 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 |
| Available RE-s for PDSCH |  |  |  | 6240 | 12720 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |
| 0 | OOR | OOR | OOR |  | N/A |
| 1 | 0.1523 | 0 | QPSK | 1480 | 2976 |
| 2 | 0.3770 | 1 |  | 2408 | 4744 |
| 3 | 0.8770 | 3 |  | 5504 | 11016 |
| 4 | 1.4766 | 5 | 16QAM | 9224 | 18960 |
| 5 | 1.9141 | 7 |  | 12040 | 24576 |
| 6 | 2.4063 | 9 |  | 15112 | 30728 |
| 7 | 2.7305 | 11 | 64QAM | 16896 | 34816 |
| 8 | 3.3223 | 13 |  | 20496 | 42016 |
| 9 | 3.9023 | 15 |  | 24576 | 49176 |
| 10 | 4.5234 | 17 |  | 28168 | 57376 |
| 11 | 5.1152 | 19 |  | 31752 | 65576 |
| 12 | 5.5547 | 21 | 256QAM | 34816 | 69672 |
| 13 | 6.2266 | 23 |  | 38936 | 79896 |
| 14 | 6.9141 | 25 |  | 43032 | 88064 |
| 15 | 7.4063 | 27 |  | 46104 | 94248 |

<END OF THE CHANGE 7>

<START OF THE CHANGE 8>

Annex G (normative):
Propagation conditions

# G.1	Static propagation condition

The propagation for the static performance measurement is an Additive White Gaussian Noise (AWGN) environment. No fading or multi-paths exist for this propagation model.

## G1.1	NCR-MT receiver with 2RX

For 1 port transmission the channel matrix is defined in the frequency domain by:

.

For 2 port transmission the channel matrix is defined in the frequency domain by:

.

## G.1.2	NCR-MT Receiver with 4Rx

For 1 port transmission the channel matrix is defined in the frequency domain by

.

For 2 port transmission the channel matrix is defined in the frequency domain by

.

For 4 port transmission the channel matrix is defined in the frequency domain by

.

# G.2	Multi-path fading propagation conditions

## G.2.1	General

The multipath propagation conditions consist of several parts:

-	A delay profile in the form of a "tapped delay-line", characterized by a number of taps at fixed positions on a sampling grid. The profile can be further characterized by the r.m.s. delay spread and the maximum delay spanned by the taps.

-	A combination of channel model parameters that include the Delay profile and the Doppler spectrum that is characterized by a classical spectrum shape and a maximum Doppler frequency.

## G.2.2	Delay profiles

### G.2.2.1	General

The delay profiles are simplified from the TR 38.901 [24] TDL models. The simplification steps are shown below for information. These steps are only used when new delay profiles are created. Otherwise, the delay profiles specified in G.2.2.1 can be used as such.

-	Step 1: Use the original TDL model from TR 38.901 [24].

-	Step 2: Re-order the taps in ascending delays.

-	Step 3: Perform delay scaling according to the procedure described in clause 7.7.3 in TR 38.901 [24].

-	Step 4: Apply the quantization to the delay resolution 5 ns. This is done simply by rounding the tap delays to the nearest multiple of the delay resolution.

-	Step 5: If multiple taps are rounded to the same delay bin, merge them by calculating their linear power sum.

-	Step 6: If there are more than 12 taps in the quantized model, merge the taps as follows:

-	Find the weakest tap from all taps (both merged and unmerged taps are considered):

-	If there are two or more taps having the same value and are the weakest, select the tap with the smallest delay as the weakest tap.

-	When the weakest tap is the first delay tap, merge taps as follows:

-	Update the power of the first delay tap as the linear power sum of the weakest tap and the second delay tap.

-	Remove the second delay tap.

-	When the weakest tap is the last delay tap, merge taps as follows:

-	Update the power of the last delay tap as the linear power sum of the second-to-last tap and the last tap.

-	Remove the second-to-last tap.

-	Otherwise:

-	For each side of the weakest tap, identify the neighbour tap that has the smaller delay difference to the weakest tap.

-	When the delay difference between the weakest tap and the identified neighbour tap on one side equals the delay difference between the weakest tap and the identified neighbour tap on the other side.

-	Select the neighbour tap that is weaker in power for merging.

-	Otherwise, select the neighbour tap that has smaller delay difference for merging.-	To merge, the power of the merged tap is the linear sum of the power of the weakest tap and the selected tap:

-	When the selected tap is the first tap, the location of the merged tap is the location of the first tap. The weakest tap is removed.

-	When the selected tap is the last tap, the location of the merged tap is the location of the last tap. The weakest tap is removed.

-	Otherwise, the location of the merged tap is based on the average delay of the weakest tap and selected tap. If the average delay is on the sampling grid, the location of the merged tap is the average delay. Otherwise, the location of the merged tap is rounded towards the direction of the selected tap (e.g. 10 ns & 20 ns  15 ns, 10 ns & 25 ns  20 ns, if 25 ns had higher or equal power; 15 ns, if 10 ns had higher power). The weakest tap and the selected tap are removed.

-	Repeat step 6 until the final number of taps is 12.

-	Step 7: Round the amplitudes of taps to one decimal (e.g. -8.78 dB  -8.8 dB)

-	Step 8: If the delay spread has slightly changed due to the tap merge, adjust the final delay spread by increasing or decreasing the power of the last tap so that the delay spread is corrected.

-	Step 9: Re-normalize the highest tap to 0 dB.

- Note 1:	Some values of the delay profile created by the simplification steps may differ from the values in tables G.2.2.2-2, G.2.2.2-3, and G.2.1.1-4 for the corresponding model.

- Note 2:	For Step 5 and Step 6, the power values are expressed in the linear domain using 6 digits of precision. The operations are in the linear domain.

### G.2.2.2	Delay profiles for FR1

The delay profiles for FR1 are selected to be representative of low, medium and high delay spread environment. The resulting model parameters are specified in G.2.2.2-1 and the tapped delay line models are specified in tables G.2.2.2-2 ~ table G.2.2.2-4.

Table G.2.2.2-1: Delay profiles for NR channel models

| Model | Number of channel taps | Delay spread (r.m.s.) | Maximum excess tap delay (span) | Delay resolution |
| --- | --- | --- | --- | --- |
| TDLA30 | 12 | 30 ns | 290 ns | 5 ns |
| TDLB100 | 12 | 100 ns | 480 ns | 5 ns |
| TDLC300 | 12 | 300 ns | 2595 ns | 5 ns |

Table G.2.2.2-2: TDLA30 (DS = 30 ns)

| Tap # | Delay (ns) | Power (dB) | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 |  |
| 2 | 10 | 0 |  |
| 3 | 15 | -5.1 |  |
| 4 | 20 | -5.1 |  |
| 5 | 25 | -9.6 |  |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 |  |
| 8 | 75 | -11.5 |  |
| 9 | 105 | -11.0 |  |
| 10 | 135 | -16.2 |  |
| 11 | 150 | -16.6 |  |
| 12 | 290 | -26.2 |  |

Table G.2.2.2-3: TDLB100 (DS = 100ns)

| Tap # | Delay (ns) | Power (dB) | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 |  |
| 2 | 10 | -2.2 |  |
| 3 | 20 | -0.6 |  |
| 4 | 30 | -0.6 |  |
| 5 | 35 | -0.3 |  |
| 6 | 45 | -1.2 | Rayleigh |
| 7 | 55 | -5.9 |  |
| 8 | 120 | -2.2 |  |
| 9 | 170 | -0.8 |  |
| 10 | 245 | -6.3 |  |
| 11 | 330 | -7.5 |  |
| 12 | 480 | -7.1 |  |

Table G.2.2.2-4: TDLC300 (DS = 300 ns)

| Tap # | Delay (ns) | Power (dB) | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -6.9 |  |
| 2 | 65 | 0 |  |
| 3 | 70 | -7.7 |  |
| 4 | 190 | -2.5 |  |
| 5 | 195 | -2.4 |  |
| 6 | 200 | -9.9 | Rayleigh |
| 7 | 240 | -8.0 |  |
| 8 | 325 | -6.6 |  |
| 9 | 520 | -7.1 |  |
| 10 | 1045 | -13.0 |  |
| 11 | 1510 | -14.2 |  |
| 12 | 2595 | -16.0 |  |

## G.2.3	Combinations of channel model parameters

The propagation conditions used for the performance measurements in multi-path fading environment are indicated as a combination of a channel model name and a maximum Doppler frequency, i.e., TDLA<DS>-<Doppler>, TDLB<DS>-<Doppler> or TDLC<DS>-<Doppler> where '<DS>' indicates the desired delay spread and '<Doppler>' indicates the maximum Doppler frequency (Hz).

Table G.2.3-1 show the propagation conditions that are used for the performance measurements in multi-path fading environment for low, medium and high Doppler frequencies for FR1.

Table G.2.3-1: Channel model parameters for FR1

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| TDLA30-5 | TDLA30 | 5 Hz |
| TDLA30-10 | TDLA30 | 10 Hz |
| TDLB100-400 | TDLB100 | 400 Hz |
| TDLC300-100 | TDLC300 | 100 Hz |

## G.2.4	MIMO channel correlation matrices

### G.2.4.1	General

The MIMO channel correlation matrices defined in annex G.2.4 apply for the antenna configuration using uniform linear arrays at both gNB and NCR-MT and for the antenna configuration using cross polarized antennas.

### G.2.4.2	MIMO correlation matrices using Uniform Linear Array

#### G.2.4.2.1	General

The MIMO channel correlation matrices defined in annex G.2.4.2 apply for the antenna configuration using uniform linear array (ULA) at both gNB and NCR-MT.

#### G.2.4.2.2	Definition of MIMO correlation matrices

Table G.2.4.2.2-1 defines the correlation matrix for the gNB.

Table G.2.4.2.2-1: gNB correlation matrix

|  | gNB correlation |
| --- | --- |
| One antenna |  |
| Two antennas |  |
| Note: The matrix applies to the gNB for NCR-MT requirements. |  |

Table G.2.4.2.2-2 defines the correlation matrix for the NCR-MT:

Table G.2.4.2.2-2: NCR-MT correlation matrix

|  | One antenna | Two antennas | Four antennas |
| --- | --- | --- | --- |
| NCR-MT correlation |  |  |  |
| Note: The matrix RUE applies to the NCR-MT for NCR-MT requirements. |  |  |  |

Table G.2.4.2.2-3 defines the channel spatial correlation matrix. The parameters, α and β in table G.2.4.2.2-3 defines the spatial correlation between the antennas at the gNB and NCR-MT respectively.

Table G.2.4.2.2-3: correlation matrices

| 1x2 case |  |
| --- | --- |
| 1x4 case |  |
| 2x2 case |  |
| 2x4 case |  |
| NOTE 1: RgNB refers to the correlation matrix of gNB for NCR-MT requirements. NOTE 2: RUE refers to the correlation matrix of NCR-MT for NCR-MT requirements |  |

For cases with more antennas at either gNB or NCR-MT or both, the channel spatial correlation matrix can still be expressed as the Kronecker product of  and according to.

#### G.2.4.2.3	MIMO correlation matrices at high, medium and low level

The  and  for different correlation types are given in table G.2.4.2.3-1.

Table G.2.4.2.3-1: Correlation for high, medium and low level

| Low correlation |  | Medium correlation |  | High correlation |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0.9 | 0.3 | 0.9 | 0.9 |

The correlation matrices for high, medium and low correlation are defined in table G.2.4.2.3-2, G.2.4.2.3-3 and G.2.4.2.3-4 as below.

The values in table G.2.4.2.3-2 have been adjusted for the 2x4 high correlation case to ensure the correlation matrix is positive semi-definite after round-off to 4 digit precision. This is done using the equation:

Where the value "a" is a scaling factor such that the smallest value is used to obtain a positive semi-definite result. For the 2x4 high correlation case, a = 0.00010.

Table G.2.4.2.3-2: MIMO correlation matrices for high correlation

| 1x2 case |  |
| --- | --- |
| 2x2 case |  |
| 2x4 case |  |

Table G.2.4.2.3-3: MIMO correlation matrices for medium correlation

| 1x2 case | [N/A] |
| --- | --- |
| 2x2 case |  |
| 2x4 case |  |

Table G.2.4.2.3-4: MIMO correlation matrices for low correlation

| 1x2 case |  |
| --- | --- |
| 1x4 case |  |
| 2x2 case |  |
| 2x4 case |  |

In table G.2.4.2.3-4,  is a  identity matrix.

- NOTE:	For completeness, the correlation matrices were defined for high, medium and low correlation but performance requirements exist only for low correlation.

### G.2.4.3	Multi-antenna channel models using cross polarized antennas

#### G.2.4.3.1	General

The MIMO channel correlation matrices defined in annex G.2.4.3 apply to two cases as presented below:

-	One TX antenna and multiple RX antennas case, with cross polarized antennas used at gNB

-	Multiple TX antennas and multiple RX antennas case, with cross polarized antennas used at both NCR-MT and gNB

The cross-polarized antenna elements with +/-45 degrees polarization slant angles are deployed at gNB. For one TX antenna case, antenna element with +90 degree polarization slant angle is deployed at NCR-MT. For multiple TX antennas case, cross-polarized antenna elements with +90/0 degrees polarization slant angles are deployed at NCR-MT.

For the cross-polarized antennas, the N antennas are labelled such that antennas for one polarization are listed from 1 to N/2 and antennas for the other polarization are listed from N/2+1 to N, where N is the number of TX or RX antennas.

#### G.2.4.3.2	Definition of MIMO correlation matrices using cross polarized antennas

For the channel spatial correlation matrix, the following is used:

Where

-	 is the spatial correlation matrix at the NCR-MT (NCR-MT requirements) with same polarization,

-	 is the spatial correlation matrix at the gNB (NCR-MT requirements) with same polarization,

-	 is a polarization correlation matrix,

-	 is a permutation matrix, and

-	denotes transpose.

Table G.2.4.3.2-1 defines the polarization correlation matrix.

Table G.2.4.3.2-1: Polarization correlation matrix

|  | One TX antenna | Multiple TX antennas |
| --- | --- | --- |
| Polarization correlation matrix |  |  |

The matrixis defined as

where  and  is the number of TX and RX antennas respectively, and  is the ceiling operator.

The matrix  is used to map the spatial correlation coefficients in accordance with the antenna element labelling system described in G.2.4.3.

#### G.2.4.3.3	Spatial correlation matrices at NCR-MT and gNB sides

##### G.2.4.3.3.1	Spatial correlation matrices at NCR-MT side

In this subclause, RUE refers to an NCR-MT for NCR-MT requirements.

For 1-antenna transmitter, .

For 2-antenna transmitter using one pair of cross-polarized antenna elements, .

##### G.2.4.3.3.2	Spatial correlation matrices at gNB side

In this subclause, RgNB refers to a gNB for NCR-MT requirements.

For 2-antenna receiver using one pair of cross-polarized antenna elements, .

For 4-antenna receiver using two pairs of cross-polarized antenna elements,.

#### G.2.4.3.4	MIMO correlation matrices using cross polarized antennas

The values for parameters α, β and γ for low spatial correlation are given in table G.2.4.3.4-1.

Table G.2.4.3.4-1: Values for parameters α,  and γ

| Low spatial correlation |  |  |
| --- | --- | --- |
| α | β | γ |
| 0 | 0 | 0 |
| Note 1: Value of α applies when more than one pair of cross-polarized antenna elements at gNB side. Note 2: Value of β applies when more than one pair of cross-polarized antenna elements at NCR-MT side. |  |  |

The correlation matrices for low spatial correlation are defined in table G.2.4.3.4-2 as below.

Table G.2.4.3.4-2: MIMO correlation matrices for low spatial correlation

| 1x8 case |  |
| --- | --- |
| 2x8 case |  |

In table G.2.4.3.4-2,  is a  identity matrix.

#### G.2.4.3.5	Beam steering approach

For the 2D cross-polarized antenna array at gNB, given the channel spatial correlation matrix in G.2.4.3.2, G.2.4.3.3 and G.2.4.3.4, the corresponding random channel matrix H can be calculated. The signal model for the k-th slot is denoted as

And the steering matrix is further expressed as following:

where

-	H is the Nr×Nt channel matrix per subcarrier.

-	 is the steering matrix,

-	 is the steering matrix in first dimension with same polarization,

-	 is the steering matrix in second dimension with same polarization,

-	 is the number of antenna elements in first dimension with same polarization,

-	 is the number of antenna elements in second dimension with same polarization,

-	For antenna array with only one direction, number of antenna element in second direction equals 1.

For 1 antenna element with the same polarization in one direction,

.

For 2 antenna elements with the same polarization in one direction,

.

where the index  stands for first dimension and second dimension respectively.

-	 controls the phase variation in first dimension and second dimension respectively, and the phase for k-th subframe is denoted by, where is the random start value with the uniform distribution, i.e., ,  is the step of phase variation, which is defined in Table G.2.4.3.5-1, and k is the linear increment of 2-μ for every slot throughout the simulation, the index  stands for first dimension and second dimension respectively.

-	 is the precoding matrix for Nt transmission antennas,

-	y is the received signal, x is the transmitted signal, and n is AWGN.

-	 corresponds to subcarrier spacing configuration,

For the 1D cross-polarized antenna array at gNB, the corresponding random channel matrix H can be calculated by letting N2=1, i.e.,

Table G.2.4.3.5-1: The step of phase variation

| Variation Step | Value (rad/ms) |
| --- | --- |
|  | 1.2566×10-3 |

<END OF THE CHANGE 8>

<START OF THE CHANGE 9>

Annex H (informative):
Change history

<END OF THE CHANGE 9>