3GPP TSG-RAN WG4 Meeting #110 	R4-2402711

Athens, Greece, February 26 – March 01, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation Results on NCR PDSCH and PDCCH Demodulation Requirements

Agenda item:	8.20.6

Document for:	Discussion

Introduction

The agreements made on NCR (Network-Controlled Repeaters) demodulation performance during RAN4#109 is documented in [1]. They were mainly agreements on the remaining open issue in RAN4#109, which was about Fixed Reference Channels (FRCs).

In RAN4#109, interested companies have already contributed to the simulation results as collected in [2]. It can be seen in [2] that there were some test cases in PDSCH with 1% BLER, which have considerably high spans. It was then agreed offline among companies to try to align the ideal (without impairments) simulation results prior to providing the simulation results with impairments in RAN4#110.

Simulation Results

In order to align the simulation results for the ideal (without impairments) case, offline discussions among companies have been conducted via email prior to RAN4#110. All interested companies have revisited and rerun their simulations. Based on the discussions and after rerunning the simulations, we are updating our simulation results for BLER 1% cases.

In the following, we provide our updated simulation results for ideal case. Furthermore, as companies need to provide the simulation results with impairments, we also add here our simulation results with impairments. We note here that for 1% BLER we consider it from the 1st transmission without HARQ retransmission, while for 70% Throughput from the HARQ retransmission with number of retransmissions 4.

PDSCH Simulation Results without Impairments

The results of our simulation on NCR-MT PDSCH using the corresponding FRCs [1] are tabulated in Table 1 below:

Table 1. Simulation Results on PDSCH without Impairments

| Case number | Bandwidth (MHz) | SCS (kHz) | MCS | Propagation condition | Antenna configuration | Reference Values |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | BLER (%) | SNR (dB) |
| 1 | 10 | 15 | 4 | TDLA30-10 | 2x2 | 70 | -2,45 | 1 | 3,71 |
| 2 | 10 | 15 | 4 | TDLA30-10 | 2x4 | 70 | -5,21 | 1 | -0,82 |
| 3 | 10 | 15 | 13 | TDLA30-10 | 2x2 | 70 | 4,32 | 1 | 11,38 |
| 4 | 10 | 15 | 13 | TDLA30-10 | 2x4 | 70 | 1,44 | 1 | 6,31 |
| 5 | 40 | 30 | 4 | TDLA30-10 | 2x2 | 70 | -2,89 | 1 | 3,05 |
| 6 | 40 | 30 | 4 | TDLA30-10 | 2x4 | 70 | -5,3 | 1 | -1,47 |
| 7 | 40 | 30 | 13 | TDLA30-10 | 2x2 | 70 | 4,67 | 1 | 10,7 |
| 8 | 40 | 30 | 13 | TDLA30-10 | 2x4 | 70 | 1,48 | 1 | 5,68 |
| 9 | 100 | 120 | 4 | TDLA30-75 | 2x2 | 70 | -3,21 | 1 | 0,82` |

PDSCH Simulation Results with Impairments

The simulation results on PDSCH with impairments is tabulated in Table 2 below:

Table 2. Simulation Results on PDSCH with Impairments

| Case number | Bandwidth (MHz) | SCS (kHz) | MCS | Propagation condition | Antenna configuration | Reference Values |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | BLER (%) | SNR (dB) |
| 1 | 10 | 15 | 4 | TDLA30-10 | 2x2 | 70 | 0,05 | 1 | 6,21 |
| 2 | 10 | 15 | 4 | TDLA30-10 | 2x4 | 70 | -2,71 | 1 | 1,68 |
| 3 | 10 | 15 | 13 | TDLA30-10 | 2x2 | 70 | 6,82 | 1 | 13,88 |
| 4 | 10 | 15 | 13 | TDLA30-10 | 2x4 | 70 | 3,94 | 1 | 8,81 |
| 5 | 40 | 30 | 4 | TDLA30-10 | 2x2 | 70 | -0,39 | 1 | 5,55 |
| 6 | 40 | 30 | 4 | TDLA30-10 | 2x4 | 70 | -2,8 | 1 | 1,03 |
| 7 | 40 | 30 | 13 | TDLA30-10 | 2x2 | 70 | 7,17 | 1 | 13,2 |
| 8 | 40 | 30 | 13 | TDLA30-10 | 2x4 | 70 | 3,98 | 1 | 8,18 |
| 9 | 100 | 120 | 4 | TDLA30-75 | 2x2 | 70 | -0,71 | 1 | 3,32 |

PDCCH Simulation without Impairments

The simulation results on NCR-MT PDCCH without impairments is shown in Table 3 below:

Table 3. Simulation Results on PDCCH without impairments

| Bandwidth (MHz)/ SCS (kHz) | CORESET RB | CORESET duration | Aggregation level | Antenna configuration and correlation Matrix | Propagation condition | Reference Values |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 10/15 | 48 | 1 | 8 | 2x4 Low | TDLA30-10 | 1 | -4,3 |

PDCCH Simulation with Impairments

Table 4 below shows the simulation results on PDCCH with impairments:

Table 4. Simulation Results on PDCCH with Impairments

| Bandwidth (MHz)/ SCS (kHz) | CORESET RB | CORESET duration | Aggregation level | Antenna configuration and correlation Matrix | Propagation condition | Reference Values |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 10/15 | 48 | 1 | 8 | 2x4 Low | TDLA30-10 | 1 | -1,8 |

More of our observations and proposals can be found in [3].

Conclusion

Interested companies have discussed offline the simulation alignment of Rel-18 NCR-MT PDSCH and PDCCH based on the agreed FRCs. In this contribution, we provide our simulation results on PDSCH and PDCCH for Rel-18 NCR-MT demodulation performance requirements. The collected and aligned simulation results from interested companies would be used to define demodulation performance requirements for Rel-18 NCR-MT in RAN4#110.

- References

- R4-2321059, WF on NCR_MT demodulation requirements, ZTE Corporation, 3GPP RAN4#109, Chicago, USA, November 13 – 17, 2023

- R4-2319538, Simulation results collection for NCR-MT demodulation requirements, ZTE Corporation, 3GPP RAN4#109, Chicago, USA, November 13 – 17, 2023

- R4-2402710, on NCR PDSCH and PDCCH Demodulation Requirements, Nokia, Nokia Shanghai Bell, 3GPP RAN4#110, Athens, Greece, February 26 – March 01, 2024