3GPP TSG-RAN WG4 Meeting #110 	R4-2402718

Athens, Greece, February 26 – March 01, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation Results on HST FR2 Enhanced with Multi-Rx Chain DL Reception

Agenda item:	8.8.3.3

Document for:	Information

Introduction

The way forward (WF) from the discussions on HST-FR2-Enhanced Demodulation in RAN4#109 is documented in [1], in which the agreements related to simulation on PDSCH with multi-RX are summarized below:

Issue 3-1-0: Test requirement to be defined Agreement: - Introduce PDSCH requirement with only considering RTD larger than CP. The PDSCH requirement is only applied for UE supporting [simultaneousReceptionFR2HST-r18] capability Issue 3-2-1: UE processing assumption for the FFT window Issue 3-2-2: RTD value for PDSCH requirement for multi-Rx reception Issue 3-2-3: MCS pair for PDSCH requirement for multi-Rx chain reception Issue 3-2-4: Test metric of SNR derivation for PDSCH requirement for multi-Rx chain reception Agreement: - Do not specify baseline UE processing assumption for the FFT window and leave it to UE implementation for FR2 HST performance requirements definition. - RTD introduced - 1.5xCP - MCS pairs - Option1: MCS 13 - Option2: MCS 11 - Test metric - 70% Tput for each PDSCH - Companies are encouraged to provide both ideal and impairments results for MCS pairs (MCS 19, MCS 13) and (MCS 19, MCS 11) based on RTD=1.5xCP in the RAN4#110 meeting, and make the down selection for specifying requirement in the next meeting

In RAN4#108, the updated simulation assumptions and parameters for HST FR2 PDSCH with multi-RX were agreed and documented in [3]. For a direct reference, we provide them in the Appendix.

Simulation Results

The simulation results presented here are based on the agreed simulation assumptions and parameters in [3]. Furthermore, as agreed in [4], the simulations did not consider the power scaling factor. While there was an agreement for not specifying the FFT assumption (refer to Issue 3-2-1 above), we note here that the simulations were assuming independent FFT per panel.

Some of the agreed assumptions and parameters used in the simulations to be highlighted here are listed below:

- mDCI transmission scheme,

- 2 layers from each TRP, that is, 2+2 layer combination,

- no inter-TRP interference,

- independent processing per TRP,

- MCS pairs of (19,11) and (19,13), and

- RTD = 1.5 CP.

The simulation results are presented in Table 1 and Table 2 below for the case of with and without impairments, respectively:

Table 1. Simulation Results for RTD=1.5 CP and Power Difference of 6.5 dB – Ideal

| Bandwidth (MHz) | Subcarrier Spacing (KHz) | Carrier Frequency (GHz) | MCS Pair (MCS1, MCS2) | Rank per TRP | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Throughput (%) | SNR Pair (SNR1, SNR2) (dB) |
| 200 | 120 | 30 | (19,11) | 2 | 2x2 Low | 70 | (12.6, 7.0) |
| 200 | 120 | 30 | (19,13) | 2 | 2x2 Low | 70 | (12.6, 10.0) |

Table 2. Simulation Results for RTD=1.5 CP and Power Difference of 6.5 dB – Impairments

| Bandwidth (MHz) | Subcarrier Spacing (KHz) | Carrier Frequency (GHz) | MCS Pair (MCS1, MCS2) | Rank per TRP | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Throughput (%) | SNR Pair (SNR1, SNR2) (dB) |
| 200 | 120 | 30 | (19,11) | 2 | 2x2 Low | 70 | (15.1, 9.5) |
| 200 | 120 | 30 | (19,13) | 2 | 2x2 Low | 70 | (15.1, 12.5) |

Furthermore, as side information, we provide simulation results for RTD = 2.5 CP without impairments below, for comparison to the agreed RTD= 1.5 CP:

Table 3. Simulation Results for RTD=2.5 CP and Power Difference of 10.8 dB - Ideal

| Bandwidth (MHz) | Subcarrier Spacing (KHz) | Carrier Frequency (GHz) | MCS Pair (MCS1, MCS2) | Rank per TRP | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Throughput (%) | SNR Pair (SNR1, SNR2) (dB) |
| 200 | 120 | 30 | (19,11) | 2 | 2x2 Low | 70 | (12.6, 17.1) |

More of our observations and proposals related to HST FR2 PDSCH with multi-RX can be found in the discussion paper in [2].

Conclusion

In this paper, we provide simulation results with the agreed simulation assumptions and parameters in [3]. Our simulation is considering only the case of independent FFT. More on our observations and proposals related to HST FR2 with multi-RX can be found in [2].

- References

- R4-2321063, WF for FR2 HST demodulation requirements, Samsung, RAN4#109, Chicago, USA, November 13-17, 2023.

- R4-2402717, HST FR2 Enhanced: UE Demodulation PDSCH Requirements with Multi-Rx Chain DL Reception, Nokia, Nokia Shanghai Bell, RAN4 #110, Athens, Greece, February 26 – March 01, 2024.

- R4-2313920, Simulation assumption for FR2 HST demodulation, Ericsson, RAN4 #108, Toulouse, France, August 21 – 25, 2023.

- R4-2317007, WF for FR2 HST demodulation requirements, Samsung, RAN4#108bis, Xiamen, China, October 09 - 13, 2023.

- Appendix

Table A. Simulation assumption for PDSCH demodulation requirements with multi-Rx simultaneous reception [3]

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| UL-DL configuration |  |  |  | FR2.120-1 |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | 13 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 2 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | full-overlapping |  |
| Number of HARQ Processes |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | As defined in Annex A.1.3 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| - Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | TRxP #1 | TRxP #2-1 | TRxP #2-2 |  |
| Channel bandwidth | MHz | 200 | 200 | 200 |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |
| Allocated resource blocks | PRBs | 132 | 132 | 132 |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 | 9 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 | 13 |  |
| For Slot i=1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Allocated slots per 2 frames |  | 124 | 124 | 124 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 17 | 13 | 19 |  |
| Modulation |  | 64QAM | 16QAM | 64QAM |  |
| Target Coding Rate |  | 0.43 | 0.48 | 0.50 |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 36 | 36 | 36 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 36 | 36 | 36 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Overhead for TBS determination |  | 6 | 6 | 6 |  |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 51216 | 33816 | 53288 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 77896 | 57376 | 92200 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 | 24 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 7 | 5 | 7 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 10 | 7 | 11 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slots i = 5 and 85 (Note 3) | Bits | 153456 | 102304 | 153456 |  |
| For Slots i = 6 and 86 (Note 3) | Bits | 153456 | 102304 | 153456 |  |
| For Slot i, if mod(i, 5) = 3 for i from {7,…, 94,87, …, 159} | Bits | 125136 | 72864 | 109296 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 182160 | 121440 | 182160 |  |
| For Slot i = 1,2,3 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Max. Throughput averaged over 2 frames | Mbps | 441.601 | 319.213 | 511.326 |  |
| - Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms - Note 2: Slot i is slot index per 2 frames - Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. - Note 4: SS/PBCH block is transmitted in slot #1, slot #2 and slot #3 with periodicity 20ms |  |  |  |  |  |