3GPP TSG-RAN WG4 Meeting #110	R4-2402755

Athens, GR, 26 February – 01 March, 2024

Source: 	Qualcomm Incorporated

Title: 	Updated simulation Results for UE PDSCH Demodulation for ATG

Agenda item:	8.9.7.2

Document for:	Information

- Introduction

In this paper, we share SNR simulation results for alignment based on the Agreed Simulation assumptions.

# General

## SNR Results for PDSCH ATG UE Demod Requirements

Observation 1: Alignment SNR considers Tx EVM 3% for MCS Table 2, Tx EVM = 6% for MCS Table 1;

Table 2-1

| Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | Correlation matrix and antenna configuration | Modulation format and code rate | MCS Table | Fraction of maximum throughput (%) | Tx EVM Assumption [%] | Alignment SNR [dB] | Impairment SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 / 15 | na | 2x2 | 13 | 1 | 70 | 6 | 4 | 6 |
|  |  |  | 22 | 1 | 70 | 6 | 11.6 | 13.6 |
|  |  |  | 24 | 2 | 70 | 3 | 20.7 | 22.7 |
|  |  | 2x4 | 13 | 1 | 70 | 6 | 1.1 | 3.1 |
|  |  |  | 22 | 1 | 70 | 6 | 8.7 | 10.7 |
|  |  |  | 24 | 2 | 70 | 3 | 18 | 20 |
| 40 / 30 | 7D1S2U | 2x2 | 13 | 1 | 70 | 6 | 4 | 6 |
|  |  |  | 22 | 1 | 70 | 6 | 11.4 | 13.4 |
|  |  |  | 24 | 2 | 70 | 3 | 20.8 | 22.8 |
|  |  | 2x4 | 13 | 1 | 70 | 6 | 1 | 3 |
|  |  |  | 22 | 1 | 70 | 6 | 8.8 | 10.8 |
|  |  |  | 24 | 2 | 70 | 3 | 18.1 | 20.1 |
|  | 30D4S6U | 2x2 | 13 | 1 | 70 | 6 | 4 | 6 |
|  |  |  | 22 | 1 | 70 | 6 | 11.4 | 13.4 |
|  |  |  | 24 | 2 | 70 | 3 | 20.8 | 22.8 |
|  |  | 2x4 | 13 | 1 | 70 | 6 | 1 | 3 |
|  |  |  | 22 | 1 | 70 | 6 | 8.8 | 10.8 |
|  |  |  | 24 | 2 | 70 | 3 | 18 | 20 |

- Conclusions

Observation 1: Alignment SNR considers Tx EVM 3% for MCS Table 2, Tx EVM = 6% for MCS Table 1;

- References